3GPP TSG-SA5 Meeting #159	S5-250599

Sophia-Antipolis, France, 17 - 21 February 2025								(is revision of S5-250210)

# Source:	Nokia

# Title:	DP on TS 28.554 KPIs related to NF Energy Consumption

# Document for:	Discussion

Agenda Item:	6.2.2

# 1	Decision/action requested

The group is requested to discuss and endorse the proposal in clause 4.

# 2	References

[1]	3GPP TS 23.501: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[2]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

# 3	Rationale

The following observations are made on the term Network Function (NF) defined in TS 23.501 [1] and its usage in TS 28.554 [2].

Observation #1:

SA2 specifiction TS 23.501 [1] specifies the 3GPP term Network Function (NF) as an entity with defined behaviour and interfaces. The emphasis is on well known/defined behaviour (specified by 3GPP).

"

Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

NOTE 2:	A network function can be implemented either as a network element on a dedicated hardware, as a software instance running on a dedicated hardware, or as a virtualised function instantiated on an appropriate platform, e.g. on a cloud infrastructure.

"

Observation #2:

The way that the term Network Function (NF) is used in clause 6.7.3.1.1 of TS 28.554 [2] (see below), is inconsistency with the 3GPP definition in TS 23.501 [1]. The term NF below seems to relate to the ETSI NFV view of a deployable entity that is agnostic of its 3GPP behaviour.

"

##### 6.7.3.1.1	Definition

a) ECNF

b) This KPI describes the Energy Consumption (EC) of a 5G Network Function (NF). This KPI is obtained by summing up the energy consumption of PNF(s) and/or VNF(s) which compose the NF. The unit of this KPI is J.

c)

- How a 5GC NF is composed of VNFs and PNFs is implementation specific. In particular, whether a VNF instance (respectively PNF) is shared or not between more than one NF is implementation specific. Hence, the case where a VNF instance (resp. PNF) is shared between multiple NFs is out of scope of the present document;

- ECPNF represents the Energy Consumption (EC) of a PNF;

- ECVNF represents the Energy Consumption (EC) of a VNF. It is obtained by summing up the Energy Consumption (EC) of all its constituent VNFCs;

- In the present document:

# ECPNF is measured according to ETSI ES 202 336-12 [10],

# it is considered that ECVNF cannot be measured hence is estimated. Therefore the resulting ECNF KPI is defined as:

"

# 4	Detailed proposal

The group is requested to evaluate the controversy between observations #1 and #2 above, identify necessary corrective actions and apply corresponding changes. A detailed CR maybe necessary as a result of the said evaluation.

The specific proposal for endorsement is to record this as an Action Item for SA5.