3GPP TSG-SA5 Meeting #159	S5-250915

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.572 Add information model

# Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

# 7	Information model

Editor's note: Add isWritable, isReadable?, isInvariant to the tables in this clause.

Editor's note: Add statements about uniqueness of ids.

Editor's note: Names of information elements need to be revisited.

## 7.1	PlannedConfigurationDescriptor

### 7.1.1	Definition

### This definition represents a planned configuration descriptor.

### 7.1.2	Information Elements

The following table specifies the information elements of a planned configuration descriptor.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| pcd-id | M | The identifier of the planned configuration. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |
| name | O | The name of the planned configuration. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| version | O | The version of the planned configuration. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| description | O | The textual human-readable description of the planned configuration. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| is-fallback-plan |  | The indication if the planned configuration was created by the MnS producer based on a MnS consumer's request to enable fallback. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA |
| additional-properties | O | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. | Type: String multiplicity: * isOrdered: False isUnique: True |
| configuration-content-type | M | The format of the planned configuration. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |
| current-conf-schema | O | The pointer to the schema of the current configuration data node tree. Editor's note: Details are ffs. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |
| apply-mode | M | Specifies if the operations in the operation set are treated as atomic, best effort, or if processing shall stop on the occurrence of the first error. allowedValues: - ATOMIC - BEST_EFFORT - STOP_ON_ERROR | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA |
| last-modified | M | The date and time at which the planned configuration descriptor was modified the last time by a MnS consumer. | Type: DateTime multiplicity: 1 isOrdered: NA isUnique: NA isWritable: False |
| operation-set | M | The operation set specifying the planned configuration. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |
| validation-state | M | The outcome of the last validation, or "NOT_VALIDATED" if the planned configuration has not been validated yet. allowedValues: - NOT_VALIDATED - VALIDATED - VALIDATION_FAILED - PARTIALLY_VALIDATED | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA isWritable: False |
| last-validated | M | The date and time at which the planned configuration was validated (the last time). The information element is absent, when the planned configuration has not been validated yet. | Type: DateTime multiplicity: 0..1 isOrdered: NA isUnique: NA isWritable: False |

## 7.2	PlannedConfigurationGroupDescriptor

### 7.2.1	Definition

This definition represents a planned configuration group descriptor.

### 7.2.2	Information Elements

The following table specifies the information elements of a planned configuration group descriptor.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| pcgd-id | M | The identifier of the planned configuration group descriptor | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |
| name | O | The name of the planned configuration group. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| version | O | The version of the planned configuration group. Its format is implementation specific. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| description | O | The textual human-readable description of the planned configuration group. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| is-fallback-plan-group |  | The indication if the planned configuration group was created by the MnS producer based on a MnS consumer's request to enable fallback. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA |
| additional-properties | O | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. | Type: String multiplicity: * isOrdered: False isUnique: True |
| pcd-ids | M | The identifiers of the planned configurations that are members of the planned configuration group. | Type: String multiplicity: 1..* isOrdered: True isUnique: True |
| apply-mode | M | Specifies if the members of the planned configuration group are treated as atomic, best effort, or if processing shall stop on the occurrence of the first error. allowedValues: - ATOMIC - BEST_EFFORT - STOP_ON_ERROR | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA |
| pcg-is-ordered | M | Specifies if the members of the planned configuration group are ordered. When ordered, the planned configuration group members shall be validated/activated in the order specified. When not ordered the planned configuration group members can be validated/activated in any order. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA |
| fail-on-conflict | M | Specifies if the activation shall fail on detection of conflicts, or if the operations shall be processed as if there were no conflicts. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA |
| last-modified | M | The date and time at which the planned configuration group descriptor was modified the last time by a MnS consumer. | Type: DateTime multiplicity: 1 isOrdered: NA isUnique: NA isWritable: False |
| validation-state | M | The outcome of the last validation, or "NOT_VALIDATED" if the planned configuration group has not been validated yet. allowedValues: - NOT_VALIDATED - VALIDATED - VALIDATION_FAILED - PARTIALLY_VALIDATED | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA isWritable: False |
| last-validated | M | The date and time at which the planned configuration group was validated (the last time). The information element is absent, when the planned configuration has not been validated yet. | Type: DateTime multiplicity: 0..1 isOrdered: NA isUnique: NA isWritable: False |

## 7.3	FallbackConfigurationDescriptor

### 7.3.1	Definition

This definition represents a fallback configuration descriptor.

### 7.3.2	Information Elements

The following table specifies the information elements of a fallback configuration descriptor.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| fcd-id | M | The identifier of the fallback configuration descriptor | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |
| name | O | The name of the fallback configuration. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| version | O | The version of the fallback configuration. Its format is implementation specific. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| description | O | The textual human-readable description of the fallback configuration. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| related-activation-job | M | The identifier of the related activation job. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |

## 7.4	TriggerCondition

### 7.4.1	Definition

This definition represents a trigger condition.

### 7.4.2	Information Elements

The following table specifies the information elements of a trigger condition.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| condition-expression | M | The condition expression. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |
| trigger-only-once | M | The boolean indication, if the trigger is disarmed after the first firing. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA |
| activation-jobs | M | The address of one or more activation jobs that shall be triggered by the condition. | Type: String multiplicity: * isOrdered: False isUnique: True |

## 7.5	ValidationJob

### 7.5.1	Definition

This definition represents a validation job.

### 7.5.2	Information Elements

The following table specifies the information elements of a validation job.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the validation job. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |
| name | O | The name of the validation job. | Type: String multiplicity: 0...1 isOrdered: NA isUnique: NA |
| description | O | The textual human-readable description of the validation job. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |
| pcd-id | M | The identifier of the planned configuration descriptor, whose operation set is requested to be validated. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| pcgd-id | M | or, alternatively, the identifier of the planned configuration group descriptor, whose operation sets are requested to be validated. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| pcd | O | or, alternatively, the planned configuration descriptor to be activated, if no planned configuration descriptor was created earlier. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| pcgd | O | or, alternatively, the planned configuration group descriptor to be validated, if no planned configuration group descriptor was created earlier. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| job-state | M | The validation job state, either "NOT_STARTED", "RUNNING", "CANCELLED" or "COMPLETED". allowedValues: - NOT_STARTED - RUNNING - CANCELLING - CANCELLED - COMPLETED - FAILED Editor's note: It is ffs if more detailed state and progress info needs to be added | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA |
| start-time | M | The date and time at which the validation process started. | Type: DateTime multiplicity: 1 isOrdered: NA isUnique: NA |
| stop-time | M | The date and time at which the validation process stopped. | Type: DateTime multiplicity: 1 isOrdered: NA isUnique: NA |
| result | M | The validation result, either "VALIDATED" or "VALIDATION_FAILED". When the activated operation set is non-atomic or the activated planned configuration group is non-atomic, also "VALIDATION_PARTIALLY_FAILED". allowedValues: - VALIDATED - VALIDATION_FAILED - PARTIALLY_VALIDATED | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA |
| error-details >> type >> reason >> title >> bad-operation | M | In case validation failed or partially failed, error reasons are specified here. Editor's note: This parameter is ffs | Type: multiplicity: 0..1 isOrdered: NA isUnique: NA |

## 7.6	ActivationJob

### 7.6.1	Definition

This definition represents an activation job.

### 7.6.2	Information Elements

The following table specifies the information elements of an activation job.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| job-id | M | The identifier of the activation job. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |
| name | O | The name of the activation job. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| description. | O | The textual human-readable description of the activation job. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| pcd-id | M | The identifier of the planned configuration descriptor, whose operation set is requested to be activated. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| pcgd-id | M | or, alternatively, the identifier of the planned configuration group descriptor, whose operation sets are requested to be activated. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| fcd-id | M | or, alternatively, the identifier of the fallback configuration descriptor, whose operation sets are requested to be activated. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| pcd | O | or, alternatively, the planned configuration descriptor to be activated, if no planned configuration descriptor was created earlier. | Type: String multiplicity: 0..1 isOrdered: NA isUnique: NA |
| pcgd | O | or, alternatively, the planned configuration group descriptor to be activated, if no planned configuration group descriptor was created earlier. | Type: String multiplicity: 1 isOrdered: NA isUnique: NA |
| enable-fallback | O | This boolean attribute allows to specify if the MnS producer shall create a plan with the current configuration to allow for a later fallback. Default value is "False", e.g. no fallback is enabled. The plan-id of the fallback plan shall be returned to the MnS consumer. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: False |
| service-impact | O | The service impact mode. allowedValues: - LEAST_SERVICE_IMPACT - SHORTEST_TIME | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA |
| immediate-activation | O | This Boolean attribute specifies if the activation job shall start immediately or, alternatively, by conditional activation. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA |
| cancel-request | O | This boolean attribute allows to request to cancel the activation process. | Type: Boolean multiplicity: 1 isOrdered: NA isUnique: NA |
| job-state | M | The activation job state. allowedValues: - NOT_STARTED - RUNNING - CANCELLED - CANCELLING - COMPLETED - FAILED Editor's note: It is ffs if more detailed state and progress info needs to be added | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA |
| start-time | O | The date and time at which the activation process started. | Type: DateTime multiplicity: 1 isOrdered: NA isUnique: NA |
| stop-time | O | The date and time at which the activation process stopped. | Type: DateTime multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None |
| result | M | The activation result, either "ACTIVATED" or "ACTIVATION_FAILED". When the activated operation set is non-atomic or the activated planned configuration group is non-atomic, also "ACTIVATION_PARTIALLY_FAILED". allowedValues: - ACTIVATED - ACTIVATION_FAILED - ACTIVATION_FAILED_ROLLEDBACK - ACTIVATION_FAILED_ROLLBACK_FAILED - PARTIALLY_ACTIVATED | Type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA |
| error-details | M | In case validation failed or partially failed, error reasons are specified here. Editor's note: This parameter is ffs | Type: multiplicity: 0..1 isOrdered: NA isUnique: NA |

End of modifications