3GPP TSG-SA5 Meeting #159	S5-250946

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	TS28.567 Coordinating CCLs with other management functions

# Document for:	Approval

Agenda Item:	6.19.4.1

# 1	Decision/action requested

Group is requested to discuss and agree on the “Detailed Proposal”.

# 2	References

[1]	None

# 3	Rationale

# 4	Detailed proposal

Start of First change

# 5	Management Capabilities

## 5.D	 CCL Coordination – COORD

### 5.D.1	Description

This use case related to the coordination of CCL, e.g. with other management capabilities

### 5.D.2	Use Cases

## 5.D.2.1	Coordinating CCLs with other management functions - FNCOORD

A CCL can make and execute decisions in different network contexts and for different network functions and parameters. Yet within the network, there may be other management functions or features including MDA functions, SON functions, and AIML Functions, which also make decisions that affect the same network functions and parameters as the CCL. The operation of CCLs needs to be coordinated with the other management functions.

NOTE: This use-case only focuses on coordinating CCLs with other management functions for executing decisions.

For a given context, the CCL should indicate to a coordination functionality the set of network functions and corresponding parameters which it is interested in changing. Accordingly, the coordination functionality may subscribe to changes to the network functions and corresponding parameters. Then in case of changes to subscribed network functions and parameters, the network functions will notify the coordination functionality of these changes. The coordination functionality subsequently notifies the CCL of the information on the latest changes to a network function parameter and an identifier of a management entity/function (e.g. CCL, MDA, SON, AI/ML inference Function) responsible for the change to the parameter.

The CCL can determine the history of previous values of the parameter, e.g., from the coordination functionality. The history includes, for each previous value, the identifier of a respective management entity/function responsible for that change to the parameter. The CCL may define a favourable range of values of the parameter based on the received information on the latest change and the history of previous changes to the parameter. The CCL can calculate a new value of the parameter considering the favourable range as a constraint for the new value. The CCL needs then to update the value of the parameter of the network function to the new value.

### 5.D.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-COORD-FNCOORD-01 | The CCL MnS producer should have a capability to indicate to a coordination functionality the set of network functions including their parameters which it is interested in changing | COORD-FNCOORD |
| REQ-COORD-FNCOORD-02 | The CCL coordination MnS producer should have a capability to enabling an authorized MnS consumer to receive information on the latest changes to a network function parameter and an identifier of a management entity/function responsible for the change to the parameter. NOTE 1: an example MnS consumer may be a CCL instance. NOTE 2: an example of management entity/function responsible for the change to the parameter may include an instance of a MDA Function, a SON Function or an AI/ML inference Function. These functions may not necessarily execute or be responsible for executing the change. | COORD-FNCOORD |
| REQ-COORD-FNCOORD-03 | The CCL coordination MnS producer should have a capability to enabling an authorized MnS consumer to receive the history of previous values of the parameter, e.g., from the coordination functionality. NOTE 3: The history includes, for each previous value, the identifier of a respective management entity/function responsible for that change to the parameter. | COORD-FNCOORD |

End Change