3GPP TSG-SA5 Meeting #159	S5-250948

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	TS28.567 Concepts and overview

# Document for:	Approval

Agenda Item:	6.19.4.1

# 1	Decision/action requested

Group is requested to discuss and agree on the “Detailed Proposal”.

# 2	References

[1]	None

# 3	Rationale

# 4	Detailed proposal

Start of First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[y]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3".

next change

# 4	Concepts and overview

## 4.1	Closed Control Loops

A Closed Control Loop (CCL) is a type of control mechanism that monitors and regulates a set of managed entities with the objective of achieving a specific goal. A CCL can be logically decomposed into several stages or steps, each providing a specific functionality and where the steps work together to achieve the stated goal. Any two CCLs with the same functionality may have the functionality supported in different count of steps implementing the functionality and similarly, any two CCLs with the same functionality and same count of steps, the respective steps may not have the same functionality.

A control loop is a building block for management of networks and services. The basic principle of any control loop is to adjust the value of an observed variable (expressed as for example an attribute) to control/influence the value of a desired goal (expressed as for example an attribute) for a controlled entity, such as a managed entity or managed function. The producers of the measurements or observations services, analysis services and  control service, are all required to fully realize and use a control loop.

A Closed Control Loop (CCL) is a control loop which operates without any intervention from a human operator or any other management entity other than possibly the initial configuration of the measurement producer and configuration of the control loop. In a closed control loop the input to the control loop provided by human operator or other management entity may include the goal or policies. Besides the provisioning needed to realize the goal, the output of the closed control loop may also include closed control loop status to a human operator or other management entity.

Examples of well-known Closed Loop types are OODA loop, composed of 4 stages/steps (Observe, Orient, Decide, Act) and MAPE, also composed of 4 stages/steps (Monitor, Analyse, Plan, Execute).

- b)

Figure 4.1-1: Open control loop entities versus Closed control loop entities (see TS28.535[x])

### 4.2	Functional steps of a closed control loop

A closed control loop may manage any managed entity, e.g. a network resource or a communication service as described in [x] and [y]. Generally, the control loop consists of the steps Monitoring/data collection, Analysis, Decision and Execution. The adjustment of the resources of the managed entity used is completed by the continuous iteration of the steps in a management control loop.

Figure 4.1.2-1: Steps of a Control Loop
a) the four functional steps; and
b) 2 steps combined into a single management function

The "Monitoring/data collection" step is responsible for collecting and pre-processing data from managed entities or from external sources.

The "Analysis" step derives insights from the available data obtained in the monitoring/data collection step. The insights provide answers to the questions, for example "What is likely to happen, what has happened and why.

The "Decision" step is responsible for deriving workflows from insights provided by the analysis step. It decides which reactive, proactive or predictive actions should be taken in consideration of insights obtained in the analysis step.

The execution step manages the activation of commands on the controlled resources or entities. The decision step should decide which actions are required, but not necessarily how they should be taken in the managed entities. So, the translation from actions to commands is a responsibility of the execution step.

### 4.3	Characteristic information of a CCL

#### 4.3.1	Overview

A CCL is associated with a set of Characteristic information that describes its properties, behaviours and impact. This information includes CCL Goals, CCL Triggers, CCL actions and action plans as well as CCL scopes.

#### 4.3.1	CCL Goals and targets

The CCL is the set of outcomes that need to be achieved or realized by the CCL. Each expected outcome is called a CCL target. A CCL may have one more CCL goals each containing a set of CCL targets.

#### 4.3.2	CCL actions and actions plans

A CCL action is a configuration change that a CCL can perform over a managed entity such as configuring an attribute of  managed object. A CCL may decide to perform several actions which can be combined in a single CCL action plan.

#### 4.3.3	CCL scopes

The scope is the set of managed objects, their properties and network outcomes that are associated with the CCL for measurement, configuration and impact. The scopes for the different CCLs can be managed by the MnS consumer, i.e. they can be defined on to the CCL or revised by the MnS consumer. A CCL may have four scopes - the measurement scope, target (impact) scope, control scope and impact scope, defined as follows:

- measurement scope: the measurement scope is where related measurements are collected

- control scope: control scope is the scope to which the CCL's actions are desired to be applied, e.g.,  the set of network functions and attributes that are the planned candidates to be modified by the CCL. The control scope is also called the action-space as it describes the set of candidate actions that the CCL can (is configured to be able to) execute.

- target scope: which relates to purpose of the CCL

- desired impact scope: the scope to which the CCL's actions are desired to have influence, e.g., it is both the network functions and attributes as well network outcomes like coverage areas that are planned to be influenced by the configuration’s actions of the CCL.

- impact scope: impact scope is the scope to which the CCL's actions have influence, e.g., it is both the network functions and attributes as well network outcomes like coverage areas that are influenced by the configurations actions of the CCL. This is different from the measurement scope, i.e. the scope where the CCLs measure and control scope, i.e. the scope where they act.

The impact scope may be known and bounded or unbounded and thus unknown - see figure 4.3.5.4-1. The bounded scope indicates that the area known by the CCL is the scope where its actions will impact. The unbounded impact-scope is the full network scope where the CCL’s action will have impact, but the CCL does have information that its action will have that impact to that scope.

Figure 4.3.5.4-1: Exemplification of known/bounded vs. unknown/unbounded impact scope: CCL A takes action in cell A expecting impact in cells A, B, C and D. if the impact is strictly in cells A, B, C and D, then the impact scope is known and bounded. However, if the impact scope includes cells E and F, then for the CCL, the true impact scope is unknown and thus unbounded.

-

## 4.4	Closed control loops purposes and uses

CCLs automate the management of network resources thereby taking control away from operators. TS28.535[x], TS28.536[y] describe the use of CCLs for service assurance. Additionally, CCLs may be used for several other use cases including among others use of CCLs for problem analysis and for fault management.

In each case, the CCL can be viewed as an entity to be managed, management capabilities related to the CCL will be exposed by the MnS producer that is associated with the CCL to enable the MnS consumer to manage the CCL. E.g., the characteristics and behaviours of the CCLs need to be directed by operators as consumers of CCL-related management services.  Moreover, the MnS also exposes capabilities for coordinating the CCL's activities. Example capabilities include capability for providing information on conflict resolution and feedback on monitoring the impact of the CCL's actions on other closed control loops or management functions.

The 3GPP management system should provide capabilities that enable a consumer to:

-	Manage the execution of CCLs. E.g. to request for and be notified about the instantiation of CCLs. For instance, if the consumer wants to request for instantiation of an Energy saving CCL for 10,000 cells.

-	Compose or request for and be notified about the composition of a CCL from a set of specific components (such as analytics services or SON functions).

-	Manage a closed loop composed from multiple components.

End Change