3GPP TSG-SA5 Meeting #159 	S5-250047

Sophia-Antipolis, France, 17 – 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0649 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Corrections on Radio Resource Utlization measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Subcounter naming rules and filter naming rules are specified in TS32.404/TS28.552. However, there are many misalignments in TS28.552 on the Radio Resource Utlization measurement definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections on Radio Resource Utlization measurements definition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear definition and format may lead incorrect implementations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

#### 5.1.1.2	Radio resource utilization

##### 5.1.1.2.1	DL Total PRB Usage

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) on the downlink for any purpose. The measurement is optionally calculated per PLMN ID, or per supported S-NSSAI and per QoS resource type (as specified in subclause 5.7.3.1 of TS 23.501[4]).

b)	SI

c)	This measurement is obtained as: , where is the DL PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of full physical resource blocks and all PRBs used for DL traffic transmission per PLMN ID and per supported S-NSSAI and per QoS resource type shall be included; is total number of PRBs available for DL traffic transmission during time period ; and is the time period during which the measurement is performed.

d)	A single integer value from 0 to 100. If the optional measurements are performed, the number of measurements is equal to the number of supported PLMN and the number of supported S-NSSAIs and the number of supported S-NSSAIs multiplied by the number of QoS resource types.

e)	RRU.PrbDl, which indicates the DL PRB Usage for all traffic, or 
RRU.PrbDl_PLMN, where the PLMN identifies the PLMN ID, 
RRU.PrbDl_SNSSAI, where the SNSSAI identifies the S-NSSAI, or 
RRU.PrbDl_SNSSAI_ResType, where the ResType identifies the resource type of 5G QoS characteristics, with the filter value of integer represent the ENUM of the resource type.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

##### 5.1.1.2.2	UL Total PRB Usage

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) on the uplink for any purpose. The measurement is optionally calculated per PLMN ID or per supported S-NSSAI and per QoS resource type (as specified in subclause 5.7.3.1 of TS 23.501[4]).

b)	SI

c)	This measurement is obtained as: , where is the UL PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of full physical resource blocks and all PRBs used for UL traffic transmission per PLMN ID and per supported S-NSSAI and per QoS resource type shall be included; is total number of PRBs available for UL traffic transmission during time period ; and is the time period during which the measurement is performed

d)	A single integer value from 0 to 100. If the optional measurements are performed, the number of measurements is equal to the number of supported PLMN and the number of supported S-NSSAIs and the number of supported S-NSSAIs multiplied by the number of QoS resource types.

e)	RRU.PrbUl, which indicates the UL PRB Usage for all traffic, or 
RRU.PrbUl_PLMN, where the PLMN identifies the PLMN ID, 
RRU.PrbUl_SNSSAI, where the SNSSAI identifies the S-NSSAI, 
RRU.PrbUl_SNSSAI_ResType, where the ResType identifies the resource type of 5G QoS characteristics, with the filter value of integer represent the ENUM of the resource type.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

##### 5.1.1.2.3	Distribution of DL Total PRB Usage

a)	This measurement provides the distribution of samples with total usage (in percentage) of physical resource blocks (PRBs) on the downlink in different ranges. This measurement is a useful measure of whether a cell is under high loads or not in the scenario which a cell in the downlink may experience high load in certain short times (e.g. in a millisecond) and recover to normal very quickly.

b)	CC

c)	Each measurement sample is obtained as: Mn=M1nPn*100, where  Mn  is total PRB usage at sample n for DL, which is a percentage of PRBs used, averaged during time period tn (e.g. a millisecond) with value range: 0-100%; M1n is a count of full physical resource blocks and all PRBs used for DL traffic transmission shall be included; Pn is the total number of PRBs available for DL traffic transmission during time period tn and n is the sample with time period tn during which the measurement is performed.

Distribution of total PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The distribution of PRB usage provides the histogram result of the samples collected during time period tn.

Depending on the value of the sample, the proper bin of the counter is increased. The number of samples during one measurement period is defined by the vendor.

d)	A set of integers. Each representing the (integer) number of samples with a DL total PRB percentage usage in the range represented by that bin.

e)	RRU.PrbTotDlDist.Bin, which Bin indicates the distribution of DL PRB Usage for all traffic which is vendor specific.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

##### 5.1.1.2.4	Distribution of UL total PRB usage

a)	This measurement provides the distribution of samples with total usage (in percentage) of physical resource blocks (PRBs) on the uplink in different usage ranges. This measurement is a useful measure of whether a cell is under high loads or not in the scenario which a cell in the uplink may experience high load in certain short times (e.g. in a millisecond) and recover to normal very quickly.

b)	CC

c)	Each measurement sample is obtained as: Mn=M1nPn*100, where  Mn  is total PRB usage at sample n for UL, which is a percentage of PRBs used, averaged during time period tn (e.g. a millisecond) with value range: 0-100%; M1n is a count of full physical resource blocks and all PRBs used for UL traffic transmission shall be included; Pn is the total number of PRBs available for UL traffic transmission during time period tn and n is the sample with time period tn during which the measurement is performed.

Distribution of total PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The distribution of PRB usage provides the histogram result of the samples collected during time period tn.

Depending on the value of the sample, the proper bin of the counter is increased. The number of samples during one measurement period is defined by the vendor.

d)	A set of integers, each representing the (integer) number of samples with a UL PRB percentage usage in the range represented by that bin.

e)	RRU.PrbTotUlDist.Bin, which Bin indicates the distribution of UL PRB Usage for all traffic which is vendor specific.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

##### 5.1.1.2.5	Mean DL PRB used for data traffic

a)	This measurement provides the number of physical resource blocks (PRBs) in average used in downlink for data traffic. The measurement is optionally calculated per QoS level (mapped 5QI or QCI in NR option 3) or per supported S-NSSAI and subcounters per supported PLMN ID.

b)	SI.

c)	Each measurement is obtained as the averagenumber (arithmetic mean)  of all PRBs used for DL data traffic transmission per S-NSSAI and per PLMN ID during a time period T.

d)	Each measurement is a single integer value. If the optional measurements are performed, the number of measurements is equal to the number of QoS levels and the number of supported S-NSSAIs and the number of supported PLMN.

e)	RRU.PrbUsedDl, or 
RRU.PrbUsedDl_QOS, where the QOS identifies the target quality of service class,
RRU.PrbUsedDl_SNSSAI, where SNSSAI identifies the S-NSSAI, 
RRU.PrbUsedDl_PLMN, where PLMN identifies the PLMN ID.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the DL PRB load of the radio physical layer per S-NSSAI.

##### 5.1.1.2.6	DL total available PRB

a)	This measurement provides the total number of physical resource blocks (PRBs) in average available downlink.

b)	SI.

c)	The measurement is obtained as the average (arithmetic mean) of total availible  count of PRBs available for DL traffic transmission during time period T.

d)	One measurement, (average number of DL PRBs) is a single integer value.

e)	RRU.PrbAvailDl

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the total number of available PRBs in average for DL traffic.

##### 5.1.1.2.7	Mean UL PRB used for data traffic

a)	This measurement provides the number of physical resource blocks (PRBs) in average used in uplink for data traffic. The measurement is optionally calculated per QoS level (mapped 5QI or QCI in NR option 3) or per supported S-NSSAI or per supported PLMN ID.

b)	SI

c)	Each measurement is obtained as the average number (arithmetic mean) of all PRBs used for UL data traffic transmission per S-NSSAI and per PLMN ID during a time period T.

d)	Each measurement (number of PRBs) is a single integer value. If the optional measurements are performed, the number of measurements is equal to the number of QoS levels and the number of supported S-NSSAIs and the number of supported PLMN ID.

e)	RRU.PrbUsedUl, or 
RRU.PrbUsedUl_QOS, where the QOS identifies the target quality of service class, 
RRU.PrbUsedUl_SNSSAI, where SNSSAI identifies the S-NSSAI, 
RRU.PrbUsedUl_PLMN, where PLMN identifies the PLMN ID.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the UL PRB load of the radio physical layer per S-NSSAI.

##### 5.1.1.2.8	UL total available PRB

a)	This measurement provides the total number of physical resource blocks (PRBs) available uplink.

b)	SI.

c)	The measurement is obtained as the average number (arithmetic mean) of total available count of PRBs available for UL traffic transmission during time period T.

d)	One measurement, (average of total number of UL PRBs) that is a single integer value.

e)	RRU.PrbAvailUl, which indicates the UL PRB available.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the total number of available PRBs in average UL.

##### 5.1.1.2.9	Peak DL PRB used for data traffic

a)	This measurement provides the maximum number of PRBs used in downlink for data traffic. The measurement is optionally calculated per QoS level (mapped 5QI or QCI in NR option 3) or per supported S-NSSAI or per supported PLMN ID.

b)	SI.

c)	Each measurement is obtained by sampling at a pre-defined interval, the PRBs used for DL data traffic transmission per S-NSSAI and per PLMN ID during a time period T, and selecting the sample with the maximum value from the samples collected in a given period.

d)	Each measurement is a single integer value. If the optional measurements are performed, the number of measurements is equal to the number of QoS levels and the number of supported S-NSSAIs and the number of supported PLMNs.

e)	RRU.MaxPrbUsedDl, or 
RRU.MaxPrbUsedDl_QOS, where the QOS identifies the target quality of service class RRU.MaxPrbUsedDl_SNSSAI, where SNSSAI identifies the S-NSSAI, 
RRU.MaxPrbUsedDl_PLMN, where PLMN identifies the PLMN ID.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the DL PRB load of the radio physical layer per S-NSSAI to support RRM resources optimization (see TS 28.313 [30]).

##### 5.1.1.2.10	Peak UL PRB used for data traffic

a)	This measurement provides the number of PRBs used in uplink for data traffic. The measurement is optionally calculated per QoS level (mapped 5QI or QCI in NR option 3) or per supported S-NSSAI or per supported PLMN ID.

b)	SI

c)	Each measurement is obtained by sampling at a pre-defined interval, the PRBs used for UL data traffic transmission per S-NSSAI and per PLMN ID during a time period T, and selecting the sample with the maximum value from the samples collected in a given period.

d)	Each measurement (number of PRBs) is a single integer value. If the optional measurements are performed, the number of measurements is equal to the number of QoS levels and the number of supported S-NSSAIs and the number of supported PLMNs.

e)	RRU.MaxPrbUsedUl, or 
RRU.MaxPrbUsedUl_QOS, where the QOS identifies the target quality of service class RRU.MaxPrbUsedUl_SNSSAI, where SNSSAI identifies the S-NSSAI, 
RRU.MaxPrbUsedUl_PLMN, where PLMN identifies the PLMN ID.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the UL PRB load of the radio physical layer per S-NSSAI to support RRM resources optimization (see TS 28.313 [30]).

##### 5.1.1.2.11	PDSCH PRB Usage per cell for MIMO

a)	This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) per cell for MIMO with time domain averaged maximum scheduled layer number as spatial factor in the downlink.

b)	SI

c)	This measurement is obtained as:

Where

MET denotes total PDSCH PRB usage per cell which is percentage of PRBs used, averaged during time period 𝑇 with integer value range: 0-100;

RijT denotes the number of PDSCH PRBs multiplexed by i MIMO layers at sampling occasion j.

PjT denotes total number of PDSCH PRBs available for sampling occasion j on single MIMO layer per cell;

LM(T) denotes the time-domain averaged maximum scheduled layer number of PDSCH in time period T defined in clause 5.1.1.30.3 of the present document;

NOTE:  At every sampling occasion the maximum scheduled layer number of all PRBs included in PDSCH is collected as a sampling value and at the end of statistical duration the average of all non-zero sampling values is the measuremnt result as defined in clause 5.1.1.30.3 of the present document.

T denotes the time period during which measurement is performed;

i is an integer denoting a MIMO layer number that is scheduled in time period T;

j denotes sampling occasion (e.g. 1 slot) during time period T.

d)	A single integer value from 0 to 100.

e)	RRU.PrbTotDlMimo, which indicates the PDSCH PRB Usage per cell for MIMO

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer under MIMO scenario.

##### 5.1.1.2.12	PUSCH PRB Usage per cell for MIMO

a)	This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) per cell for MIMO with time domain averaged maximum scheduled layer number as spatial factor in the uplink.

b)	SI

c)	This measurement is obtained as:

,

Where

MET denotes total PUSCH PRB usage per cell which is percentage of PRBs used, averaged during time period 𝑇 with integer value range: 0-100;

RijT denotes the number of PUSCH PRBs multiplexed by i MIMO layers at sampling occasion j.

PjT denotes total number of PUSCH PRBs available for  sampling occasion j on single MIMO layer per cell;

LM(T) denotes the time-domain averaged maximum scheduled layer number of PUSCH in time period T defined in  clause 5.1.1.30.4 of the present document;

NOTE:  At every sampling occasion the maximum scheduled layer number of all PRBs included in PUSCH is collected as a sampling value and at the end of statistical duration the average of all non-zero sampling values is the measuremnt result as defined in  clause 5.1.1.30.4 of the present document.

T denotes the time period during which measurement is performed;

i is an integer denoting a MIMO layer number that is scheduled in time period T;

j denotes sampling occasion (e.g. 1 slot) during time period T.

d)	A single integer value from 0 to 100.

e)	RRU.PrbTotUlMimo, which indicates the PUSCH PRB Usage per cell for MIMO

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer under MIMO scenario.

##### 5.1.1.2.13	SDM PDSCH PRB Usage

##### 5.1.1.2.13	SDM PDSCH PRB Usage

a)	Due to MIMO technology (strong Space Division Multiplexing ability), the cell capacity has been improved obviously. This measurement provides the total usage (in percentage) of PDSCH physical resource blocks (PRBs), based on statistical MIMO layers. The objective is to measure the usage of cell DL capacity in MIMO scenario. A use-case is wireless network workload observation.

b)	SI

c)	This measurement is defined according to "PDSCH PRB Usage based on statistical MIMO layer in the DL per cell " in TS 38.314 [29] as:

MT1=∀i∀j{M1ij(T1)*Lij(T1)}∀j{Pj(T1)}*β*100

β=maxT2LaveDL(T)

Where

M(T1) denotes total PDSCH PRB usage per cell which is percentage of PRBs used, averaged during time period T1 with integer value.

M1ij(T1) denotes a count of PDSCH PRBs used for traffic transmission for UE i on single MIMO layer per cell at sampling occasion j. Counting unit for PRB is 1 Resource Block x 1 symbol. (1 Resource Block = 12 sub-carriers).

Lij(T1) denotes the number of MIMO layers scheduled for UE i at sampling occasion j.

i denotes a UE i that is scheduled during time period T1.

j denotes sampling occasion during time period T1 . A sampling occasion is 1 symbol.

Pj(T1) denotes total number of PDSCH PRBs available for sampling occasion j on single MIMO layer per cell.

T1 denotes the time period during which the measurement is performed to calculate M(T1), e.g. 15min, 1 hour, etc.

β is a variable factor for MIMO layers assigned with the maximum LaveDL during time period T2 with float value 1.00-100.00. For this measurement, the same β value is used for the entire duration of T1.

LaveDL(T) is the "Average value of scheduled MIMO layers per PRB on the DL", during time period T with float value 1.00-100.00, as defined in 5.1.1.30.

T denotes time period during which the measurement is performed to calculate LaveDL(T), as defined in 5.1.1.30.

T2 is the time period during which the measurement is performed to calculate β, e.g.1 week, etc.

d)	A single integer value from 0 to 100.

e)	RRU.PrbTotSdmDl, which indicates the DL SDM PRB Usage in a Cell supporting MIMO.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the DL Radio Resource Utilization in a cell supporting MIMO.

##### 5.1.1.2.14	SDM PUSCH PRB Usage

##### 5.1.1.2.14	SDM PUSCH PRB Usage

a)	Due to MIMO technology (strong Space Division Multiplexing ability), the cell capacity has been improved obviously. This measurement provides the total usage (in percentage) of PUSCH physical resource blocks (PRBs), based on statistical MIMO layers. The objective is to measure the usage of cell UL capacity in MIMO scenario.  A use-case is wireless network workload observation.

b)	SI

c)	This measurement is defined according to "PUSCH PRB Usage based on statistical MIMO layer in the UL per cell " in TS 38.314 [29] as:

MT1=∀i∀j{M1ij(T1)*Lij(T1)}∀j{Pj(T1)}*β*100

β=maxT2LaveUL(T)

Where

M(T1) denotes total PUSCH PRB usage per cell which is percentage of PRBs used, averaged during time period T1 with integer value.

M1ij(T1) denotes a count of PUSCH PRBs used for traffic transmission for UE i on single MIMO layer per cell at sampling occasion j. Counting unit for PRB is 1 Resource Block x 1 symbol. (1 Resource Block = 12 sub-carriers).

Lij(T1) denotes the number of MIMO layers scheduled for UE i at sampling occasion j.

i denotes a UE i that is scheduled during time period T1.

j denotes sampling occasion during time period T1 . A sampling occasion is 1 symbol.

Pj(T1) denotes total number of PUSCH PRB available for sampling occasion j on single MIMO layer per cell.

T1 denotes the time period during which the measurement is performed to calculate M(T1), e.g. 15min, 1 hour, etc.

β is a variable factor for MIMO layers assigned with the maximum LaveUL during time period T2 with float value 1.00-100.00. For this measurement, the same β value is used for the entire duration of T1.

LaveUL(T) is the "Average value of scheduled MIMO layers per PRB on the UL", during time period T with float value 1.00-100.00, as defined in 5.1.1.30.

T denotes time period during which the measurement is performed to calculate LaveUL(T), as defined in 5.1.1.30.

T2 is the time period during which the measurement is performed to calculate β, e.g.1 week, etc.

d)	A single integer value from 0 to 100.

e)	RRU.PrbTotSdmUl, which indicates the UL SDM PRB Usage in a Cell supporting MIMO.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the UL Radio Resource Utilization in a cell supporting MIMO.

##### 5.1.1.2.15	DL PRB Usage per SSB

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) per SSB beam of the NR cell on the downlink for any purpose. The measurement is optionally calculated per QoS resource type (as specified in subclause 5.7.3.1 of TS 23.501[4]).

b)	SI

c)	This measurement is obtained as: , where is the DL total PDSCH PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of all PDSCH PRBs used for DL traffic transmission for the SSB beam per QoS resource type; is the cell total PDSCH PRB number during time period ; and is the time period during which the measurement is performed.

d)	A single float value from 0 to 100. If the optional measurements are performed, the number of measurements is equal to the number of QoS resource types.

e)	RRU.PrbDlSSB, which indicates the DL PRB Usage for all traffic, or
RRU.PrbDlSSB_ResType, where the ResType identifies the resource type of 5G QoS characteristics, with the filter value of integer represent the ENUM of the resource type.

f)	Beam

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for radio resource status, which may be used as the input for AI/ML model training at OAM, such as load balacing.

##### 5.1.1.2.16	UL PRB Usage per SSB

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) per SSB beam of the NR cell on the uplink for any purpose. The measurement is optionally calculated per QoS resource type (as specified in subclause 5.7.3.1 of TS 23.501[4]).

b)	SI

c)	This measurement is obtained as: , where is the UL total PUSCH PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of all PUSCH PRBs used for UL traffic transmission for the SSB beam per QoS resource type; is the cell total PUSCH PRB number during time period ; and is the time period during which the measurement is performed.

d)	A single float value from 0 to 100. If the optional measurements are performed, the number of measurements is equal to the number of QoS resource types.

e)	RRU.PrbTotUlSSB, which indicates the UL PRB Usage for all traffic,or
RRU.PrbUlSSB_ResType, where the ResType identifies the resource type of 5G QoS characteristics, with the filter value of integer represent the ENUM of the resource type.

f)	Beam

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for radio resource status, which may be used as the input for AI/ML model training at OAM, such as load balacing.

##### 5.1.1.2.17	UL CI Time Domain Proportion

a)	This measurement provides the proportion of time domain resources that invoke the cancellation indication (CI) feature during the sampling period. The number of sampling occasions that invoke the CI feature (see subclause 11.2A in TS 32.213 [64]), when the number of cancelled PRBs is greater than 0.

b)	SI

c)	This measurement is obtained as: PCIT=CITNUTT*100,

where

PCI(T) denotes the proportion of time domain resources that invoke the CI feature during the time period T, with value range: 0-100%;

CI(T) is the number of sampling occasions that invoke the CI feature during the time period T,  CI(T)=iCIi(T);

CIiT is the invoking CI feature result of sampling occasion i,  when the number of physical resource blocks (PRBs) that invoke the CI feature at sampling occasion i is greater than 0, CIi(T) = 1, and when the number of PRBs that invoke the CI feature at sampling occasion i is equal to 0, CIi(T) = 0;

NUT(T) is the number of sampling occasions with UL data scheduled (eMBB, URLLC, etc.) during the time period T, NUT(T)=iNUT,i(T)；

NUT,i(T) is the UL data scheduled result of sampling occasion i, when there is UL data scheduled at sampling occasion i, NUT,i(T)=1, and when there is no UL data scheduled at sampling occasion i, NUT,i(T)=0;

NOTE:	UL data scheduled is scheduled data of user plane, such as eMBB data, URLLC data, etc.

T denotes the time period during which measurement is performed;

i denotes sampling occasion (e.g. 1 slot) during time period T.

d)	A single integer value from 0 to 100.

e)	RRU.CiUtUl, which indicates the proportion of time domain resources that invoke the CI feature

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for evaluating the resource load of URLLC services under eMBB and URLLC multiplexing scenarios.

##### 5.1.1.2.18	Distribution of PDCCH CCE PRB Usage

a)	This measurement provides the distribution of samples with PDCCH usage (in percentage) of physical resource blocks (PRBs) in different ranges. This measurement is a useful indicator of whether a cell is under high load in scenarios where a cell on the PDCCH may experience high load in certain short times (e.g. in a millisecond) and recovers to normal very quickly.

b)	CC

c)	Each measurement sample is obtained as: Mn=M1nPn*100, where Mn is PRB usage at sample n for PDCCH CCE, which is a percentage of PRBs used, averaged during time period tn (e.g. a millisecond) with value range: 0-100%; M1n is a count of full physical resource blocks and all PRBs used for PDCCH transmission shall be included; Pn is the total number of PRBs available for PDCCH transmission during time period tn and n is the sample with time period tn during which the measurement is performed.

Distribution of PDCCH CCE PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The distribution of PDCCH CCE PRB usage provides the histogram result of the samples collected during time period tn.

Depending on the value of the sample, the proper bin of the counter is increased. The number of samples during one measurement period is defined by the vendor.

d)	A set of integers. Each representing the (integer) number of samples with a PDCCH CCE PRB percentage usage in the range represented by that bin.

e)	RRU.PrbPdcchDist.Bin, which Bin indicates the distribution of PDCCH CCE PRB Usage for a cell which is vendor specific.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

##### 5.1.1.2.19	PDCCH CCE Usage

5.1.1.2.19.1	PDCCH CCE Usage per cell

a)	This measurement provides the total usage (in percentage) of PDCCH control-channel elements (CCEs) per cell. The objective of the measurement is to measure usage of time, frequency, and space resources.

b)	SI

c)	This measurement is defined:

MT=∀i∀j{M1ij(T)*Lij(T)}∀j{Pj(T)}*Alpha*100,

where M(T) denotes total PDCCH CCE usage per cell which is percentage of CCEs used for MIMO and non-MIMO, averaged during time period T with integer value range: 0-100.

M1ijT denotes a count of PDCCH CCEs used for control information transmission for UE i on each MIMO layer per cell at sampling occasion j for MIMO scenario, or a count of PDCCH CCEs used for control information transmission for UE i per cell at sampling occasion j for non-MIMO scenario.

Lij(T) denotes the number of MIMO layers scheduled for UE i at sampling occasion j for MIMO scenario or equals 1 for non-MIMO scenario.

i denotes a UE i that is scheduled during time period 𝑇.

j denotes sampling occasion during time period 𝑇. A sampling occasion is 1 symbol.

PjT denotes total number of PDCCH CCEs available for sampling occasion j on each MIMO layer per cell for MIMO scenario, or total number of PDCCH CCEs available for sampling occasion j per cell for non-MIMO scenario.

T denotes the time period during which the measurement is performed to calculate MT, e.g., 15min, 1 hour, etc.

Alpha is a constant value of available MIMO layers configured by OAM during time period 𝑇 with float value 1.00-100.00 for MIMO scenario, or a constant value configured by OAM during time period 𝑇 with float value 1.00 for non-MIMO scenario. With this parameter, M(T) should not be larger than 100.

d)	A single integer value from 0 to 100.

e)	RRU.CceTot, which indicates the PDCCH CCE Usage per cell.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the PDCCH Usage in a cell.

*** END OF CHANGE ***