3GPP TSG-SA5 Meeting #159	S5-250060

Sofia-Antipolis, US, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.567 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.567 CCL Goal- and Goal-targets- conflicts handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add use case, requirements and solution description for Conditional control of CCLs of CCLs |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses on the new use case to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 (new clause added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[X]	3GPP TS 28.872: "Study on Management of planned configurations".

next change

# 6	Closed Control Loops Coordination Capabilities

## 6.A	 Closed Control Loop Conflict management

### 6.A.1	Description

#### 6.A.1.1	CCL Goal- and Goal-targets- conflicts handling

##### 6.A.1.1.1	Introduction

CCL instances will be responsible for related, adjacent or in some cases overlapping scopes. In such cases it is good to ensure that the goals of two CCLs are not contradictory or conflicting or leading to contradictory or conflicting outcomes. The goal may be seen as a set of network measurements and KPIs (i.e. targets) to be concurrently achieved by the CCL. A coordination layer, say a CCL coordination entity, may have a goal management capability responsible for managing and optimizing the goals of the CCLs based on general objectives for the network scope, where the general objective describe the priorities among the different target values on the specific KPIs. For example, as illustrated by Figure 5.6.3.3.1-1, the input network scope objectives may simply require ensuring, for KPI K1, that with priorities, p1, p2, p3, the value of K1 should respectively be less than values V1, V2, V3. This may be provided for different KPIs on a network level (e.g. by the operator). Then, the goal management functionality compares these objectives to choose the appropriate KPI targets for each CCL. In the example, the CCL goals are set as K1 < 0.1 and K2 > 85 %.

Figure 7.1.1.2-1: Illustration of example prioritized goal targets on a set of KPIs that need to be coordinated among a group of CCL instances. This solution assumes that the scopes are prefixed but the outcomes of the solution may a recommendation to adjust the scopes

##### 6.A.1.1.1	Detection and avoidance of potential goal targets conflicts

To support detection and avoidance of potential goal targets conflicts:

-	In the case where the CCL has been pre-configured e.g., by the operator with the expected targets, the CCL may register its goal targets with the CCL coordination entity goal management functionality which triggers an evaluation of potential conflict, i.e. whether those targets are likely to conflict with the targets of another CCL.

-	In case of a potential conflict, the goal management functionality of CCL coordination entity sends the selected new or revised goal targets to each CCL ensuring to minimize contradictions or conflicts among the targets in the different goals of different CCLs, for example, that for a given scope a specific target is assigned to only one CCL.

-	Alternatively, using the define general objectives for the network scope, the CCL Coordination entity goal management functionality may select the appropriate goal targets for the CCL

##### 6.A.1.1.2	Detection and avoidance of actual goal targets conflicts

To support detection and resolution of actual goal targets conflicts:

-	The CCLs attempt to fulfil its set targets, and where they ae unable to, the CCL sends feedback to the goal management functionality in the coordination entity indicating which targets cannot be fulfilled. A CCL may for example indicate that there are ping-pong effects on a target, i.e. whenever the target is pushed in a given direction, it flips back to a previous state. The flipflop is an indication of a potential goal conflict which the CCL should notify to the goal management.

-	The CCL could detect potential constraints with the defined goals, e.g., that targets that cannot be reached due to conflicts among with other targets. The CCL should notify the coordination entity goal management functionality, e.g., the response could be that “Target x cannot be achieved because it causes problems on higher priority target y.”.

-	Based on the feedback, the goal management functionality acting as CCL coordination MnS producer can then confirm the existence of goal-target conflict and may revise the targets by setting new target or goal target values.

##### 6.A.1.1.3	CCL Goal-conflicts resolution

The targets in the goals of Closed Control Loop should not contradict one another within that goal or contradict with other targets in goals of related CCLs, otherwise a goal conflict is observed. For such a goal conflict, goal coordination interactions are needed to resolve the conflict, i.e. to align goals (and related targets) that should be achieved by the various deployed Closed Control Loops. Given the potentially high number and diversity of Closed Control Loops, the process of setting and coordination goals for the Closed Control Loops should be accomplished using another entity, the CCL coordination entity, that consumes the CCL-related monitoring and governance management services to coordinate the resolution of conflicts with the CCL.

The MnS producer for the CCL instance should inform the CCL coordination entity (acting as the MnS consumer) about a candidate goal conflict, e.g. about the values of the goal's targets that are in conflict with the targets of another goal. In response, the CCL coordination entity (acting as the MnS consumer) could revise the goals of that CCL instance, terminate the execution of the CCL instance, delete the CCL instance.

### 6.A.2	Requirements

REQ-CCL-CONFLICT-1: The 3GPP Management System should support a capability to detect potential or actual CCL goal- and goal-targets conflicts, CCL scope conflicts, direct-actions conflicts, CCL-trigger-time and CCL-action-execution-time conflicts as well as indirect target conflicts.

REQ-CCL-CONFLICT-2: The 3GPP Management System should support a capability to inform an authorized MnS consumer about a potential CCL goal- or goal-targets conflict, CCL scope conflict, CCL direct-actions conflict, CCL-trigger-time and CCL-action-execution-time conflict or CCL indirect target conflict that has been detected.

REQ-CCL-CONFLICT-3: The 3GPP Management System should support a capability to confirm a detected potential CCL goal- or goal-targets conflict, CCL scope conflict, CCL direct-actions conflict, CCL-trigger-time and CCL-action-execution-time conflict or CCL indirect target conflict.

REQ-CCL-CONFLICT-4: The 3GPP Management System should support a capability to resolve a CCL goal- or goal-targets conflict, CCL scope conflict, CCL direct-actions conflict, CCL-trigger-time and CCL-action-execution-time conflict or CCL indirect target conflict that has been detected.

REQ-CCL-CONFLICT-5: The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to avoid a CCL goal- or goal-targets conflict, CCL scope conflict, CCL direct-actions conflict, CCL-trigger-time and CCL-action-execution-time conflict or CCL indirect target conflict.

REQ-CCL-CONFLICT-6: The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to resolve a CCL goal- or goal-targets conflict, CCL scope conflict, CCL direct-actions conflict, CCL-trigger-time and CCL-action-execution-time conflict or CCL indirect target conflict.

REQ-CCL-CONFLICT-7: The 3GPP Management System should support a capability to coordinate the resolution of CCL goal- or goal-targets conflicts, CCL scope conflicts, CCL direct-actions conflicts, CCL-trigger-time and CCL-action-execution-time conflicts or CCL indirect target conflicts.

REQ-CCL-CONF_RES-3: The 3GPP Management System should support a capability enabling an MnS consumer to define and coordinate the hierarchies of the CCL.

REQ-CCL-COORD-1: The 3GPP management system should support a capability enabling the MnS consumer to configure the scopes of a CCL, including the measurement scope, the control scope , and the impact scope.

NOTE:	Measurement scope is where the related measurements are collected, the control scope is where the CCL acts and the impact scope is where the CLL actions causes effects.

REQ-CCL-COORD-2: The 3GPP management system should support a capability to detect, avoid and resolve conflicts among the scopes of multiple CCLs, including the measurement scope, the control scope and the impact scope.

### 5.2.3	Solution Description

End Change