3GPP TSG-SA5 Meeting #159	S5-250089

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.572 Add clause Fallback configurations and Managing fallback configurations

# Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

## 6.7	Fallback configurations

### 6.7.1	Definition

Fallback is the process of returning to a previously prevailing current configuration. A fallback configuration is a special kind of planned configuration that achieves exactly this fallback when applied to the current configuration. Fallback configurations are generated by the system. Fallback may be applied when the activation of a planned configuration does not yield the desired results. Use cases include:

- A wrong planned configuration was activated. The operator wants to return to the old configuration.

- The new configuration does not perform as expected. The operator or the automation system wants to return to the old configuration.

- The new configuration was deployed on existing Network Functions in anticipation of some new Network Functions being rolled out. The roll out does not happen. The existing Network Functions are rolled back to their old configuration.

### 6.7.2	Fallback preparation

A fallback configuration is created by the system upon the start of an activation process when enable fallback is requested. The format of the fallback configuration is implementation specific. For example, for a planned configuration that adds a new object to the current data node tree the fallback configuration simply removes this object.

Note that a fallback configuration just reverts the changes performed by the activation of the related planned configuration. It does not undo other changes that may have occurred between the activation of the related planned configuration and falling back.

### 6.7.3	Meta data for fallback configurations

Each fallback configuration is described by meta data. This includes the name, the version, and a human readable textual description.

Furthermore, an identifier of the activation process whose result is to be rolled back must be provided.

### A fallback is always atomic; no control parameter needs to be provided for configuring if the rollback should be performed in atomic or best effort mode.

### 6.7.4	Fallback configuration descriptor

The fallback configuration and its meta data are specified in a fallback configuration descriptor. Each descriptor has a unique identifier.

End of modifications