3GPP TSG-SA5 Meeting #159	S5-250092

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.572 Improve clause Activation

# Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

## 6.4	Activation of planned configurations

### 6.4.1	Definition

Activation is the process of applying a planned configuration to the managed system.

### 6.4.2	Potential problems

A planned configuration is validated before activation. A planned configuration that is activated should hence not have any of the problems that are discovered by validation. Only successfully validated planned configurations should be activated. Therefore, other types of problems are encountered during activation. These include

- A Network Function cannot be updated, because it is not reachable.

- A Network Function cannot be updated, because it is locked.

- A Network Function cannot be updated, because it does not exist.

### 6.4.3	Activation process

Activation is the process of applying the updates specified by the operations contained in a planned configuration to the managed system. When the managed system is updated the updates are reflected in the current configuration. Note that the system may not apply the operations to the managed system exactly as specified. For example, to change the configuration of a function, the function may have to be disabled. Disabling operations may not be included in the planned configuration and therefore the configuration update operation is augmented with disabling and enabling operations.

In atomic mode, the system may have to roll back to the current configuration that was effective when the activation process started. Therefore, the system needs to take and store a (conceptual) snapshot of the impacted part of the current configuration upon starting the activation process.

The activation process may run for quite some time, for example when large configurations are downloaded to the Network Functions as part of the activation process.

The planned configuration to be activated should be validated first.

Problems related to the activation of a planned configuration shall be reported.

Note that the new current configuration is not always equal to the old current configuration with the operations in the planned configuration applied. For example, the system may create additional data nodes beyond what is requested by the operations in the planned configuration.

### 6.4.4	Activation of planned configuration groups

The planned configuration group members are activated in the order specified. If no order is specified, they can be activated in any order. Each member is activated as specified in the previous clause. The activation of atomic members concludes with either activated or failed. The activation of best effort members concludes with either activated, partially activated or failed.

An atomic group is considered as activated, if atomic members can be activated, and best effort members can be activated or partially activated. If the activation of one (atomic or best effort) member fails, the activation of the complete group shall fail (and all updates must be undone).

A best effort group is considered as activated, if atomic members can be activated, and best effort members can be activated or partially activated. If the activation of at least one (atomic or best effort) member fails, the group is considered as partly activated. If the activation of all members fails, the activation of the complete group shall fail (and all updates must be undone).

### 6.4.5	Activation control parameters

Editor's note: Explain "enable-fallback", "service-impact and "immediate-activation". Also "cancel-request".

Next modification

End of modifications