3GPP TSG-SA5 Meeting #159	S5-250172

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.561 Generic NDT NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NDT |  |  |  |  |  | Date: |  |  | 2024-01-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Define a generic NDT NRM to support NDT Lifecycle management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define a generic NDT NRM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NDT MnS cannot work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Stage 3 will be provided later |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First Changes

6 Stage 2 definition for NDT MnS

6.1	Management operation for NDT MnS (MnS component type A)

The operations (e.g. createMOI operations) and notifications (e.g. notifyMOIcreation) of generic provisioning MnS defined in 3GPP TS 28.532 [x] can be used for NDT MnS. The NDTFunction and NDT can be treated as Managed Object instances.

Following is the IS to support capability obtaining:

Table 6.1-1: IS to support NDT capability obtaining

| NDT lifecycle management | IS operation |
| --- | --- |
| Query NDT capability | getMOIAttributes operation |

6.2	Information model definition for NDT MnS (MnS component typeB)

6.2.1	Generic Information model definition

6.2.1.0	Imported information entities and local labels

Table 6.2.1.2.1.2-1

| Label reference | Local label |
| --- | --- |
| TS 28.622 [x], IOC, Top | Top |
| TS 28.622 [x], IOC, SubNetwork | SubNetwork |

6.2.1.1	Class diagram

6.2.1.1.1	Relationship

Figure 6.2.1.1.1-1: Relationship UML diagram for NDT Management

Figure 6.2.1.1.1-2: NRM fragment for NDT LCM Management

Figure 6.2.1.1.1-3: Relationship UML diagram for NDT Request

6.2.1.1.2	Inheritance

Figure 6.2.1.1.2-1: Inheritance UML diagram for NDT management

6.2.1.2	Class definition

6.2.1.2.1	NDT <<InformationObjectClass>>

6.2.1.2.1.1	Definition

This IOC represents the properties of the NDT and it is name-contained by NDTRepository.

The NDT IOC contains one or multiple nDTProfile(s) which includes the detailed information of the specific NDT.

The NDT IOC includes the attribute nDTCapability that indicates the scenario specific capability of the NDT producer.

The NDT IOC includes the attribute nDTFormingType that indicates the forming type of the NDT including NestedNDT, ComposedNDT and Individual NDT. The difference between NestedNDT and ComposedNDT lies in whether their components are NDT or NDTComponent. NDT can provide NDT management services independently, while NDTComponent can only provide NDT MnS when combined with other NDTComponents to form a ComposedNDT.

6.2.1.2.1.2	Attributes

The NDT IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [x]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nFProfiles | M | T | T | F | F |
| nDTCapability | M | T | T | F | F |
| nDTFormingType | M | T | T | F | F |
| Attribute related roles |  |  |  |  |  |
| nDTRefList | CM | T | F | F | F |
| nDTComponentRefList | CM | T | F | F | F |

6.2.1.2.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| nDTRefList Support Qualifier | Condition: The forming type of the NDT is NestedNDT. |
| nDTComponentRefList Support Qualifier | Condition: The forming type of the NDT is ComposedNDT. |

6.2.1.2.1.4	Notifications

The common notifications defined in clause 6.2.1.5 are valid for this IOC.

6.2.1.2.2	NDTRepository <<InformationObjectClass>>

6.2.1.2.2.1	Definition

The IOC NDTRepository represents the centralized repository that contains the NDT and NDTComponent. It is name-contained by SubNetwork.

This NDTRepository instance can be created by the system (MnS producer) or pre-installed.

The NDTRepository MOI may contain one or more NDT(s)/NDTComponent(s), which decides the allowed value of nDTRepositoryType.

6.2.1.2.2.2	Attributes

The NDTRepository IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [x]) and the following attributes.

Table 6.2.1.2.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTRepositoryType | M | T | T | F | F |

6.2.1.2.2.3	Attribute constraints

None.

6.2.1.2.2.4	Notifications

The common notifications defined in clause 6.2.1.5 are valid for this IOC.

6.2.1.2.3	NDTComponent <<InformationObjectClass>>

6.2.1.2.3.1	Definition

The IOC NDTComponent represents the detailed information of the NDTComponent.

nDTComponentType indicates the entity the NDTComponent represent.

6.2.1.2.3.2	Attributes

The NDTComponent IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [x]) and the following attributes.

Table 6.2.1.2.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTComponentType | M | T | T | F | F |

6.2.1.2.3.3	Attribute constraints

None.

6.2.1.2.3.4	Notifications

The common notifications defined in clause 6.2.1.5 are valid for this IOC.

6.2.1.2.4	NDTFunction <<InformationObjectClass>>

6.2.1.2.4.1	Definition

The IOC NDTFunction represents the NDT function which supports one or more NDT capabilities.

6.2.1.2.4.2	Attributes

The NDTFunction IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [x]) and the following attributes.

Table 6.2.1.2.4.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedNDTFormingTypes | M | T | T | F | F |
| supportedNDTCapabilities | M | T | T | F | F |

6.2.1.2.4.3	Attribute constraints

None.

6.2.1.2.4.4	Notifications

The common notifications defined in clause 6.2.1.5 are valid for this IOC.

6.2.1.2.5	NDTRequest <<InformationObjectClass>>

6.2.1.2.5.1	Definition

The IOC NDTRequest represents the NDT output request created by an MnS consumer.

6.2.1.2.5.2	Attributes

The NDTRequest IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [x]) and the following attributes.

Table 6.2.1.2.5.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTCreationRequest | M | T | T | F | F |
| nDTConfigurationRequest | M | T | T | F | F |
| nDTRunningRequest | M | T | T | F | F |

6.2.1.2.5.3	Attribute constraints

None.

6.2.1.2.5.4	Notifications

The common notifications defined in clause 6.2.1.5 are valid for this IOC.

6.2.1.2.6	NDTReport <<InformationObjectClass>>

6.2.1.2.5.1	Definition

The IOC NDTReport represents the report containing the outputs for one or more NDTRequest delivered to the NDT MnS Consumer.

6.2.1.2.6.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [x]) and the following attributes.

Table 6.2.1.2.6.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTReportID | M | T | F | T | T |
| Attribute related to roles |  |  |  |  |  |
| nDTRequestRef | M | T | F | F | F |

6.2.1.2.6.3	Attribute constraints

None.

6.2.1.2.6.4	Notifications

The common notifications defined in clause 6.2.1.5 are valid for this IOC.

6.2.1.3	DataType definition

6.2.1.3.1	NDTCreationRequest <<dataType>>

6.2.1.3.1.1	Definition

NDTCreationRequest <<dataType>> represents the request from the NDT MnS Consumer in the NDT creation phase, which may include NDT instance creation, orchestration, updating, and deletion.

nDTCapability indicates the scenario specific capabilities that should be suppored by the NDT to be created from the consumer’s perspective.

nDTCreationOp indicates the method to generate a NDT instance. When the value of nDTCreationOp is “Creation”, it means consumer requests the NDT producer to create a new NDT from scratch. When the value of nDTCreationOp is “Orchestration”, it means consumer requests the NDT producer to create a NestedNDT based on exsting NDT. When the value of nDTCreationOp is “Updating”, it means consumer requests the NDT producer to update the exsting NDT by replacing, adding, or removing the NDT component.

nDTCreationOpInfo represents specific information for each operation in the NDT creation phase, e.g., the identifier of the managed object being operated on.

6.2.1.3.1.2	Attributes

The NDTCreationRequest <<dataType>> includes the following attributes.

Table 6.2.1.3.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTCapability | M | T | T | F | F |
| nDTCreationOp | M | T | F | T | T |
| nDTCreationOpInfo | M | T | F | T | T |

6.2.1.3.1.3	Attribute constraints

None.

6.2.1.3.2	NDTConfigurationRequest <<dataType>>

6.2.1.3.2.1	Definition

NDTConfigurationRequest <<dataType>> represents the request from the NDT MnS Consumer in the NDT configuration phase, which may include direct configuration for non-real time NDT and data collection & synchronization for real-time NDT.

6.2.1.3.2.2	Attributes

The NDTConfigurationRequest includes the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

6.2.1.3.2.3	Attribute constraints

None.

6.2.1.3.3	NDTRunningRequest <<dataType>>

6.2.1.3.3.1	Definition

NDTRunningRequest <<dataType>> represents the request from the NDT MnS Consumer in the NDT running phase.

6.2.1.3.37.2	Attributes

The NDTRunningRequest includes the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| startTime | M | T | F | T | T |
| stopTime | M | T | F | F | T |

6.2.1.3.3.3	Attribute constraints

None.

6.2.1.3.4	NDTProfile <<dataType>>

TBD.

6.2.1.4	Attribute definition

Table 6.2.1.4-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| nDTformingType | It indicates the forming type of the NDT including NestedNDT, ComposedNDT and Individual NDT. The difference between NestedNDT and ComposedNDT lies in whether their components are NDT or NDTComponent. NDT can provide NDT management services independently, while NDTComponent can only provide NDT MnS when combined with other NDTComponents to form a ComposedNDT. allowedValues: NestedNDT, ComposedNDT and Individual NDT. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTRepositoryType | This parameter describes the type of the components in the repository, which includes NDTOnly, NDTComponentOnly, and NDTandNDTComponent. allowedValues: NDTOnly, NDTComponentOnly, and NDTandNDTComponent. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTComponentType | nDTComponentType indicates the entity the NDTComponent represent.The detailed training format is vendor specific. allowedValues: N/A. | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedNDTFormingTypes | It indicates the NDT forming capabilities supported by the NDT function. AllowedValues: NestedNDT, ComposedNDT and Individual NDT. | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedNDTCapabilities | It indicates the specific NDT capabilities supported by the NDT function. AllowedValues: TBD. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTReportID | It indicates the identifier for the NDTReport. allowedValues: N/A. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTRequestRef | It indicates the DN of the NDTRequest MOI for which the results are generated by the NDT producer. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTCreationRequest | It represents the NDT MnS Consumer’s requirements in the NDT Creation Phase. allowedValues: N/A. | type: NDTCreationRequest multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTConfigurationRequest | It represents the NDT MnS Consumer’s requirements in the NDT Configuration Phase. AllowedValues: N/A | type: NDTConfigurationRequest multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTRunningRequest | It represents the NDT MnS Consumer’s requirements in the NDT Running Phase. AllowedValues: N/A. | type: NDTRunningRequest multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCreationOp | It indicates the requested opration from the consumer in the NDT Creation phase. AllowedValues: Creation, Orchestration, Updating and Deletion. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTCreationOpInfo | It represents specific information for each operation in the NDT creation phase, i.e., the identifier of the managed object being operated on. When the nDTCreationOp is “Orchestration”, it includes a list of DN of NDT that need to be used to generated the NestedNDT. When the nDTCreationOp is “Updating”, it includes a list of DNs of NDT component that need to be updated. AllowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| startTime | It indicates the start time of the NDT running requested by the MnS consumer. AllowedValues: N/A | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| stopTime | It indicates the stop time of the NDT running requested by the MnS consumer. AllowedValues: N/A | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

6.2.1.5	Common notifications

6.2.1.5.1	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [x], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

Table 6.2.1.5.1-1

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |

End of First Changes

Start of Second Changes

Annex A (informative):
PlantUML source code

A.1	Information model definition for NDT

A.1.1	Relationship UML diagram for NDT Management (figure 6.2.1.1.1-1)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

skinparam nodesep 60

class ManagedEntity <<ProxyClass>>

class NDTRepository <<InformationObjectClass>>

class NDT <<InformationObjectClass>>

class NDTComponent <<InformationObjectClass>>

ManagedEntity "1" *-- "*" NDTRepository : <<names>>

NDTRepository "1" *-- "*" NDTComponent: <<names>>

NDTRepository "1" *-- "*" NDT: <<names>>

NDT "*" --> "2..*" NDT

NDT "*" --> "2..*" NDTComponent

note left of ManagedEntity

This represents the following IOCs:

SubNetwork

end note

@enduml

A.1.2	Relationship UML diagram for NDT LCM Management (figure 6.2.1.1.1-2)

@startuml

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

skinparam  NoteBackgroundColor White

class "<<ProxyClass>> \n  ManagedEntity " as ManagedEntity{}

class  "<<InformationObjectClass>>\n NDTFunction" as NDTFunction{}

class "<<InformationObjectClass>>\n NDTRequest " as NDTRequest {}

class "<<InformationObjectClass>>\n NDTReport" as NDTReport{}

class "<<InformationObjectClass>>\n NDTRepository" as NDTRepository{}

class "<<InformationObjectClass>>\n NDT" as NDT{}

ManagedEntity "1" *-- "*" NDTFunction : <<names>>

NDTFunction "*" -right-> "1" NDTRepository

NDTFunction "1" *-- "*" NDTRequest : <<names>>

NDTFunction "1" *-- "*" NDTReport : <<names>>

NDTReport "*" -left-> "1" NDTRequest

NDTReport "1" --> "*" NDT

NDTRequest "1" --> "*" NDT

note left of ManagedEntity

Represents the folllowing IOCs:

SubNetwork

end note

@enduml

A.1.3	Relationship UML diagram for NDT Request (figure 6.2.1.1.1-3)

@startuml

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

skinparam  NoteBackgroundColor White

class "<<InformationObjectClass>>\n NDTRequest" as NDTRequest{}

class "<<dataType>>\n NDTCreationRequest" as NDTCreationRequest{}

class "<<dataType>>\n NDTConfigurationRequest" as NDTConfigurationRequest{}

class "<<dataType>>\n NDTRunningRequest" as NDTRunningRequest{}

NDTRequest "1" -- "*" NDTCreationRequest

NDTRequest "1" -- "*" NDTConfigurationRequest

NDTRequest "1" -- "*" NDTRunningRequest

@enduml

A.1.4	Relationship UML diagram for NDT Inheritance (figure 6.2.1.1.1-4)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class NDTRepository <<InformationObjectClass>>

class NDT <<InformationObjectClass>>

class NDTComponent <<InformationObjectClass>>

class NDTFunction <<InformationObjectClass>>

class NDTRequest <<InformationObjectClass>>

class NDTReport <<InformationObjectClass>>

Top <|-- NDTRepository

Top <|-- NDT

Top <|-- NDTComponent

Top <|-- NDTRequest

Top <|-- NDTReport

Top <|-- NDTFunction

@enduml

End of Second Changes