3GPP TSG-SA5 Meeting #159	S5-250186

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR |  | rev | - | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add UC for using NDT support for network automation in TS 28.561 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NDT |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR includes the updating of the agreed use case of using NDT in TR28.915 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description and requirements of the use case of using NDT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case of using NDT for signaling storm will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

5	Use cases

5.Y	NDT support for network automation

5.Y.1	NDT for signaling storm simulation and validation

5.Y.1.1	Description

Signaling storm refers to the situation where a large number of signaling messages suddenly surge in the mobile communication network, resulting in the network processing capacity overload, thus affecting the network performance and stability. Signaling storm may be caused because of big event happened that too many users request services at the same time, or by network failure, configuration error or malicious attacks. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signaling messages surge suddenly, causing signaling storm.

Therefore, the NDT can be used to model (either simulation or emulation) various network scenarios to help network against the signaling storm, which could include the following aspects:

- The NDT can be used to model network failures or surges in requests and assess their potential impact on the network. It helps determine whether the network can defend against a possible future signaling storm and validates strategies for addressing such scenarios when they occur.

- When a signaling storm occurs on the network (e.g., identified by MDA, see MDA type for 5GC control plane congestion analysis), the NDT can be used to evaluate whether the proposed solution can resolve the signaling storm issue, thereby preventing potential impacts. For example, operators need to effectively control the flow at each signaling control node in the network to prevent nodes from malfunctioning due to the signaling storm. Network Functions (NFs) such as AMF, SMF, and UDM in the 5G network might all be potential impact points during a signaling storm. The NDT can model network behaviors and help identify the optimal flow control parameters for each signaling impact point to prevent serious damage to the 5G network when affected by a signaling storm.

5.Y.1.2	Requirements

REQ-NDT-VER-SS-01: NDT should have the capability to model (either using emulation method or simulation method) the behaviour of signaling storm.

REQ-NDT-VER-SS-02: NDT should have a capability enabling the MnS consumer to define a network configuration to be modelled for checking the network response to a signaling storm.

REQ-NDT-VER-SS-03: NDT should have the capability to report the network behavior related to a signaling storm.

End of changes