3GPP TSG-SA5 Meeting #159	S5-250209

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	DP on TS 28.554 KPIs related to NetworkSlice and NetworkSliceSubnet KPI Object

# Document for:	Discussion

Agenda Item:	6.2.2

# 1	Decision/action requested

The group is requested to discuss and endorse the proposal in clause 4.

# 2	References

[1]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[2]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[3]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TS 23.501: " System Architecture for the 5G System; Stage 2".

# 3	Rationale

The following observations are made on the KPIs related to NetworkSlice and NetworkSliceSubnet KPI Object, defined in TS 28.554 [3]:

Observation #1:

Network slice is defined by SA2 as in clause 3.1 of TS 23.501 [5] as below:

"

Network Slice: A logical network that provides specific network capabilities and network characteristics.

"

The following terms are defined SA5 in clause 3.1 of TS 28.530 [4] as below. These describe the difference between “network slice” as defined in SA2, and “NetworkSlice instance” as defined in SA5.

"

network slice: a logical network that provides specific network capabilities and network characteristics, supporting various service properties for network slice customers.

NOTE 1: NetworkSlice Information Object Class (IOC)  (refer to TS 28.541 [6]) is used to model network slice.

NOTE 2: Represent network slice defined in TS 23.501 [3] with added service properties.

NetworkSlice instance: A Managed Object Instance (MOI) of NetworkSlice IOC.

NOTE 3: NetworkSlice instance represents service view of a network slice which exposes the root NetworkSliceSubnet instance .

network slice subnet: a representation of a set of network functions and the associated resources (e.g. compute, storage and networking resources) supporting network slice.

NOTE 4: NetworkSliceSubnet IOC (refer to TS 28.541 [6]) is used to model network slice subnet which may include core network functions and/or RAN network functions and/or other network slice subnets. The network slice instance defined in TS 23.501 [3] can be reflected via the NetworkSliceSubnet IOC and the allocated resources.

NetworkSliceSubnet instance: A Managed Object Instance (MOI) of NetworkSliceSubnet IOC.

"

Observation #1:

The NetworkSlice IOC is defined in clause 6.3.1 of TS 28.541 [1], with attribute serviceProfileList that holds a list of ServiceProfile (defined in clause 6.3.3 of TS 28.541 [1]) supported by the NetworkSlice MOI.

ServiceProfile is a data type with attribute pLMNInfoList (a list of data type PLMNInfo defined in clause 4.3.41 of TS 28.541 [1]) that defines which PLMN and S-NSSAI combinations that are assigned for the service to satisfy service requirements represented by the ServiceProfile in case of network slicing feature is supported.

A NetworkSlice MOI can be configured, in attribute serviceProfileList, with a list of ServiceProfile value, where the list can hold one or more than one element. Each element in the list attribute serviceProfileList can be configured with a list PLMN and S-NSSAI combinations in attribute pLMNInfoList.

Hence a NetworkSlice MOI can be configured with a list of PLMN and S-NSSAI combinations

Observation #2:

The KPIs in 28.554 [3] related to “network slice” (as defined in SA2) are reported on NetworkSlice KPI objects, while “network slice” in SA2 are identified using S-NSSAI.

The KPIs in 28.554 [3] that are reported on NetworkSlice KPI objects based on measurement (defined in TS 28.552 [2]) with SNSSAI filter. These KPIs are are not split based on per PLMN and per S-NSSAI.

Observation #3:

The NetworkSliceSubnet IOC is defined in clause 6.3.2 of TS 28.541 [1], with attribute sliceProfileList that holds a list of SliceProfile (defined in clause 6.3.4 of TS 28.541 [1]) supported by the NetworkSliceSubnet MOI.

SliceProfile is a data type with attribute pLMNInfoList (a list of data type PLMNInfo defined in clause 4.3.41 of TS 28.541 [1]) that defines which PLMN and S-NSSAI combinations that are assigned for the service to satisfy service requirements represented by the ServiceProfile in case of network slicing feature is supported.

A NetworkSliceSubnet MOI can be configured, in attribute sliceProfileList, with a list of SliceProfile value, where the list can hold one or more than one element. Each element in the list attribute sliceProfileList can be configured with a list PLMN and S-NSSAI combinations in attribute pLMNInfoList.

Further, attribute networkSliceSubnetRef of NetworkSliceSubnet IOC is a list of DN of constituent NetworkSliceSubnet. Each consitituent NetworkSliceSubnet can further be configured with a list of PLMN and S-NSSAI combinations.

Hence a NetworkSliceSubnet MOI can be configured with a list of PLMN and S-NSSAI combinations.

Observation #4:

The KPIs in 28.554 [3] that are reported on NetworkSliceSubnet KPI objects based on measurement (defined in TS 28.552 [2]) with SNSSAI filter. These KPIs are are not split based on per PLMN and per S-NSSAI.

Based on the above observations, the following issues are identified:

Issue #1:

The NetworkSlice IOC can be configured with multiple PLMN and S-NSSAI combinations. The KPIs in TS 28.554 [3] with KPI object as NetworkSlice, are defined based on measurements with SNSSAI filter. It is unclear which value of SNSSAI configured in NetworkSlice MOI is used for filtering.

Issue #2:

Since the NetworkSliceSubnet IOC can be configured with multiple PLMN and S-NSSAI combinations. The KPIs in TS 28.554 [3] with KPI object as NetworkSliceSubnet, are defined based on measurements with SNSSAI filter. It is unclear which value of SNSSAI configured in NetworkSliceSubnet MOI in is used for filtering.

Issue #3:

The issues #1 and #2 are present in TS 28.554 from Rel-16 and onwards.

# 4	Detailed proposal

The group is requested to endorse the following proposals:

1) The KPIs defined with KPI object as NetworkSlice and NetworkSliceSubnet in TS 28.554 [3], should be corrected to include the measurements of all PLMN and SNSSAI combinations that are configured for NetworkSlice and NetworkSliceSubnet MOIs.

2) The KPIs defined with KPI object as NetworkSlice and NetworkSliceSubnet in TS 28.554 [3] should be enhanced to be split per PLMN and per S-NSSAI, to provide the KPIs for network slice (as defined by SA2).

3) The corrections should be done for TS 28.554 [3] from Rel-16 onwards using TEI16.

4) Any new KPI proposed with KPI object as NetworkSlice and NetworkSliceSubnet in TS 28.554 [3], should not introduce the issues described in Issue #1 and Issue #2.