3GPP TSG-SA5 Meeting #159	S5-250328

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0299 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.312 Add generic utility function support (stage 1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Per recommendations in TR 28.914 IDMS will be enhanced to add support for utlity functions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add utlity function support as a new generic use case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Recommendation from TR would be missing from TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.x (new), 5.3.x.1 (new), 5.3.x.2 (new), 5.3.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR adds the introduction and REQs. Subsequent CRs will add the stage2 and stage3. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 5.3.x	Intent Utility Function

### 5.3.x.1	Introduction

In some cases, it may not be obvious to the MnS producer how to select from multiple available solutions for fulfilling an intent.  For example, an MnS consumer expressing multiple intents may want to provide further information to MnS producer to improve overall fulfilment, rather than rely on simple satisfied/unsatisfied.

Extra information provided in an intent can indicate preferences between expectation targets and assist the MnS producer in balancing between acceptable alternatives. This is especially relevant when no solution fully satisfies all intents or meets the expectations of a particular intent. In such situations, balancing trade-offs among different intents becomes necessary.

For example, a network performance function could be used to quantify network performance relative to latency. providing a means for MnS Consumer “to quantify an intent’s relative value as higher when the latency is lower”:

U1 = f(throughput, latency, packet loss)

U2 = f(latency) and U3=f(throughput)

An MnS consumer could express this in the form of an Intent Utility Function along with their expected target value.  This assists the MnS producer(s) in fulfilling an MnS consumer’s intents as it provides an indicator to the MnS Producer of the satisfaction level of the intent fulfilment.

### 5.3.x.2	Definition

Intent Utility Function defines a method by which consumers can express the relative value of an intent's expectations to assist the IDMS producer(s) in fulfilling their intents in the most acceptable manner.

Intent utility functions are mathematical expressions that quantify the satisfaction or utility derived from the various levels of fulfilment. The basic components of which include:

-	variables: to quantify specific aspects of the fulfilment, e.g. network performance.

-	weights: to define the relative importance of each variable, e.g. for network performance a variable representing low latency might be assigned higher weight than throughput.

-	function: the mathematical functions to be applied to the variables, e.g. linear, logarithmic, polynomials, scalars.

-	result: the output of the function. The value of which represents the utility level achieved, i.e. the satisfaction of the current fulfilment based on the acceptability of potential outcomes defined by the consumer.

MnS producer(s) can use Intent Utility Function information to assess the acceptability of potential outcomes, in addition to information such as resource availability and performance targets. Utility functions may be defined by the consumer and provided as part of the intent itself, i.e. the function is defined as part of the intent. A MnS consumer may also specify that an existing (i.e. predefined) utility function is used. Predefined utility functions may be vendor specified (e.g. system pre-installed) and/or specified by the MnS consumer.

A single intent may contain multiple intent expectations each having its own utility function. In such cases, aggregation of these utility functions can be used to represent the overall utility of the intent. Normalization may also be needed when dealig with Utility Functions expressed in different scales (e.g., latency in milliseconds, throughput in Mbps, energy consumption in Joules).

Figure [X] represents a case where MnS Consumer expresses an intent (A) with expectation which should be less than a while expectation of another intent (B) should be greater than b.  There may be no solution to satisfy both expectations. There may however be different solutions available for each intent with differing levels of fulfilment as indicated by the black and red circles. While both solutions would satisfy intent A neither would satisfy intent B and there is limited ability for the MnS producer to decide which would be better from MnS consumer perspective.

Figure X: Intents with undifferentiated fulfilment

Figure [Y] represents a case where MnS Consumer defines Utility Functions A and B.  Each provide the MnS producer with a means to calculate utility value based on the deviation between the achieved and target expectation target values for different solutions.  These utility results can be used by MnS producer to select between solutions to improve the overall utility across multiple intents.  For example, the solution with the red circle yields a better outcome since it can fully satisfy A and better satisfies B than the black solution.

Figure Y: Intents with quantitative fulfilment (via Utility Functions)

Figure [Z] represents a case where no solutions can satisfy any of the intents.  Although neither can be fulfilled the MnS producer may choose to prioritize the red circle solution as its utility increases exponentially for Intent B.  This would likely contribute to a better overall utility level across multiple intents.

Figure Z: Intents with quanititative fulfilment (via Utility Functions)

### 5.3.x.3	Requirements

REQ-Intent_Util-01: The intent driven MnS producer should have the capability to advertise its support for allowing MnS Consumers to express utility functions.

REQ-Intent_Util-02: The intent driven MnS producer should have the capability to advertise the methods by which an MnS Consumer can express utility functions.

REQ-Intent_Util-03: The intent driven MnS producer should have the capability to report potential outcomes, with the impacts to the related ExpectationObject, including consideration for utility functions when applicable.

REQ-Intent_Util-04: The intent driven MnS producer should allow an authorized MnS consumer to specify utility functions as part of an intent.

REQ-Intent_Util-05: The intent driven MnS producer should allow an authorized MnS consumer to reference utility functions defined external to the intent.