3GPP TSG-SA5 Meeting #159	S5-250333

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.558 | CR | 0032 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS28.558 update descriptions of some measurements and cleanup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | update descriptions of some measurements and cleanup |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update non-split gNB/split gNB to non-split gNB/split gNB deployment scenario Cleanup and correction |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | descriptions of some measurements are not clear; format issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2,3, 6.3.1.2, 6.3.1.5.1, 6.3.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 38.415: "NG-RAN; PDU session user plane protocol".

[6]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[7]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[8]	3GPP TS 38.314: "NR; Layer 2 Measurements".

[9]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT);Overall description; Stage 2".

[10]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[11]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[12]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[13]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace: Trace control and configuration management".

[14]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

[15]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)"

[16]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity;"

[17]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EN-DC	E-UTRA-NR Dual Connectivity

2nd change

#### 6.3.1.2	Packet Loss for all gNB deployment scenarios

##### 6.3.1.2.1	DL Packet Loss Rate on Uu

a)	This measurement provides the DL Packet (i.e., RLC SDU) Loss rate on Uu interface. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI. This measurement is also referred to as DL M7 in TS 37.320 [9].

b)	CC

c)	This measurement is obtained according to the definition in clause 4.2.1.5.1 of TS 38.314 [8], named "Packet Uu Loss Rate in the DL per DRB per UE". The measurement is performed per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value. The number of measurements is equal to the number of QoS levels multiplied by the number of supported S-NSSAIs.

e)	DRB.PacketLossRateUuDlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellDU

g)	5G-S-TMSI

h)	One usage of this measurement is to support ML training and performance evaluation.

##### 6.3.1.2.2	DL Packet Loss Rate with delay threshold on Uu

a)	This measurement provides the DL Packet (i.e., RLC SDU) Loss rate including any packets not successfully transmitted or packets successfully received but delayed more than a delay threshold that can be used when the resource type of corresponding QoS Flow is Delay-critical GBR (clause 5.7.3.4 in TS 23.501[2]）on Uu. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI. This measurement is also referred to as M7 in TS 37.320 [9].

b)	CC

c)	This measurement is obtained according to the definition in clause 4.2.1.5.2 of TS 38.314 [8], named "Packet Uu Loss Rate with delay threshold in the DL per DRB per UE". The measurement is performed per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value. The number of measurements is equal to the number of QoS levels multiplied by the number of supported S-NSSAIs.

e)	DRB.PacketLossRateDTUuDlUe.Filter, 
where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellDU

g)	5G-S-TMSI

h)	One usage of this measurement is to support ML training and performance evaluation.

3rd change

#### 6.3.1.5	UE Data Volume

##### 6.3.1.5.1	Measurements valid for non-split gNB deployment scenario

###### 6.3.1.5.1.1	DL PDCP SDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. The measurement can be filtered per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as DL M4 in TS 37.320 [9] clause 5.4.1.1. The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN PDCP layers. The measurement is performed at the PDCP SDU level. The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of QoS levels multiplied by the number of SNSSAIs.

[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeDlUe_Filter
, where Filter is a combination of QoS level and SNSSAI, whereQoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU

g)	N/A

h)	One usage of this measurement is to support ML training and performance evaluation.

###### 6.3.1.5.1.2	UL PDCP SDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to higher layers. The measurement can be filtered per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as UL M4 in TS 37.320 [9] clause 5.4.1.1. The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits delivered from PDCP layer to higher layers. The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of QoS levels multiplied by the number of S-NSSAIs.

[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduVolumeUlUe_Filter
, where Filter is a combination of QoS level and SNSSAI, where QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU

g)	N/A

h)	One usage of this measurement is to support ML training and performance evaluation.

##### 6.3.1.5.2	Measurements valid for split gNB deployment scenario

###### 6.3.1.5.2.1	DL PDCP SDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. The measurement can be filtered per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as DL M4 in TS 37.320 [9] clause 5.4.1.1. The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN PDCP layers. The measurement is performed at the PDCP SDU level. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of QoS levels multiplied by the number of S-NSSAIs.

[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpSduVolumeDlUe_Filter
, where Filter is a combination of QoS level and SNSSAI, where QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU, GNBCUUPFunction

g)	N/A

h)	One usage of this measurement is to support ML training and performance evaluation.

###### 6.3.1.5.2.2	UL PDCP SDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to higher layers. The measurement can be filtered per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as UL M4 in TS 37.320 [9] clause 5.4.1.1. The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits delivered from PDCP layer to higher layers. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of QoS levels multiplied by the number of S-NSSAIs.

[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpSduVolumeUlUe_Filter
, where Filter is a combination of QoS level and SNSSAI, where QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellCU, GNBCUUPFunction

g)	N/A

h)	One usage of this measurement is to support ML training and performance evaluation.

End of changes