3GPP TSG-SA5 Meeting #159	S5-250337

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0664 | rev | - | Current version: | 17.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS28.552 Correct unit and description of some measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correct unit and description of some measurements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct kbitk Moidfiy GTP SDU to GTP PDU |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong unit and GTP data unit cause confusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.6, 5.4.5.2.1, 5.4.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 5.4.1.6	Data volume of outgoing GTP data packets per QoS level on the N3 interface, from UPF to (R)AN

a)	This measurement provides the data volume of the outgoing GTP data packets per QoS level which have been generated by the GTP-U protocol entity on the N3 interface.  The measurement is calculated and split into subcounters per QoS level (5QI).

b)	CC.

c)	This measurement is obtained by counting the number of GTP PDU bits sent from UPF to GNB on the N3 interface. The measurement is performed per configured QoS level (5QI).

d)	Each measurement is an integer value representing the number of bits measured in kbit . The number of measurements is equal to the number of QoS levels.

e)	GTP.OutDataVolumeQoSLevelN3UPF

f)	EP_N3.

g)	Valid for packet switching.

h)	5GS.

2nd change

#### 5.4.5.2	UL GTP packets delay in UPF

##### 5.4.5.2.1	Average UL GTP packets delay in PSA UPF

a)	This measurement provides the average (arithmetic mean) UL GTP packets delay within the PSA UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained as: 1) sampling the UL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) sum of (time when sending the sampled UL data packet at the PSA UPF's egress IP termination for N6 interface, minus time of arrival of the corresponding GTP PDU from N3 or N9 interface at PSA UPF's ingress GTP termination) divided by total number of sampled UL data packets sent to N6 interface. The measurement is calculated per 5QI and per S-NSSAI.

d)	Each measurement is an integer representing the mean delay in microseconds.

e)	GTP.DelayUlInPsaUpfMean.5QI, where 5QI identifies the 5QI;
GTP.DelayUlInPsaUpfMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.5.2.2	Distribution of UL GTP packets delay in PSA UPF

a)	This measurement provides the distribution of UL GTP packets delay within the PSA UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained by 1) sampling the UL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) calculating the UL delay for a GTP PDU in I-UPF by:  time when sending the sampled UL data packet at the PSA UPF's egress IP termination for N6 interface, minus time of arrival of the corresponding GTP PDU from N3 or N9 interface at PSA UPF's ingress GTP termination; and 3) incrementing the corresponding bin with the delay range where the result of 2) falls into by 1 for the subcounters per 5QI and subcounters per S-NSSAI.

d)	Each measurement is an integer representing the number of sampled UL GTP PDUs measured with the delay within the range of the bin.

e)	GTP.DelayUlInPsaUpfDist.Bin.5QI, where Bin indicates a delay range which is vendor specific, and 5QI identifies the 5QI; 
GTP.DelayUlInPsaUpfDist.Bin.SNSSAI, where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.5.2.3	Average UL GTP packets delay in I-UPF

a)	This measurement provides the average (arithmetic mean) UL GTP packets delay within the I-UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained as: 1) sampling the UL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) sum of (time when sending the sampled UL GTP PDU to the PSA UPF at the I-UPF's egress GTP termination, minus time of arrival of the same packet from N3 interface at I-UPF's ingress GTP termination) divided by total number of sampled UL GTP PDUs sent to the PSA UPF. The measurement is calculated per 5QI and per S-NSSAI.

d)	Each measurement is an integer representing the mean delay in microseconds.

e)	GTP.DelayUlInIUpfMean.5QI, where 5QI identifies the 5QI;
GTP.DelayUlInIUpfMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.4.5.2.4	Distribution of UL GTP packets delay in I-UPF

a)	This measurement provides the distribution of UL GTP packets delay within the I-UPF. The measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	This measurement is obtained by 1) sampling the UL GTP PDUs (sampling rate is vendor specific) for this measurement, 2) calculating the UL delay for a GTP PDU in I-UPF by:  time when sending the sampled UL GTP PDU to the PSA UPF at the I-UPF's egress GTP termination, minus time of arrival of the same packet from N3 interface at I-UPF's ingress GTP termination; and 3) incrementing the corresponding bin with the delay range where the result of2) falls into by 1 for the subcounters per 5QI and subcounters per S-NSSAI.

d)	Each measurement is an integer representing the number of sampled UL GTP PDUs measured with the delay within the range of the bin.

e)	GTP.DelayUlInIUpfDist.Bin.5QI, where Bin indicates a delay range which is vendor specific, and 5QI identifies the 5QI; 
GTP.DelayUlInIUpfDist.Bin.SNSSAI, where Bin indicates a delay range which is vendor specific, and SNSSAI identifies the S-NSSAI.

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

End of changes