3GPP TSG-SA5 Meeting #159	S5-250373

Sophia-Antipolis, FRANCE 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0172 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.104 adding threshold related requirements for Fault management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is related to WT-6.1 (See new WID proposal in S5-246126) : Add new capability to perform analysis and provide report related to threshold crossing events. Specification of this new capability may need a new use case description, new requirements, new attribute for threshold information in the MDARequest and new output data (possible statistics information). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new use case description and requirements related to threshold crossing events. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new capability as studied and concluded in TR 28.866 is not fulfiled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.1.2 and 7.2.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is resubmission of endored contribution (S5-247163) in SA5#158. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

7.2.3	MDA assisted fault management

7.2.3.1	Failure prediction

7.2.3.1.1	Description

This MDA capability is for failure prediction.

7.2.3.1.2	Use case

There are multiple sources of faults which may cause the 5G system to fail to provide the expected service. These faults and the associated failures need extensive troubleshooting. In order to reduce network and service failure time and performance degradation, it is necessary to supervise the status of various network functions and resources, and predict the running trend of network and potential failures to intervene in advance. These predictions can be used by the management system to autonomously maintain the health of the network, e.g. speedy recovery actions on a network function related to the predicted potential failure.

Due to the fact that failure prediction could depend on the existing alarm incidents and relevant historical and real-time data (performance measurement information, configuration data, network topology information, etc.), there is a possibility for MDA to be used in conjunction with AI/ML technologies and model training to predict potential failures.

In order to avoid the occurrence of failures and abnormal network status, it is necessary for consumers of analytics to obtain the required details of potential failure and the corresponding degradation trend (abnormal KPI, performance measurement information, possible alarm type, fault root cause, etc.). Therefore, MDA, may in conjunction with AI/ML technology, be required to obtain basic health maintenance knowledge (e.g. the relationship between the failures or potential failures and the related maintenance actions) through predefined expertise or model training, so as to effectively predict potential failures. The basic health maintenance knowledge could be updated with feedback. The consumer of analytics could provide the performance crossing threshold information and resource usage threshold information to MDAS producer for calculating and collecting the statistics result, e.g. percentage of time when the performance KPI exceeds the threshold, and percentage of time when resource usage of the NF is beyond the threshold. Such statistics information helps the MDAS consumer to know whether the NF is in the overloaded state. Furthermore, the statistics information can be combined with historical data and ML model for MDAS producer to perform failure prediction of the NF.

Besides the MDA capability to obtain basic health maintenance relationships between the service failure and related potential failures at network levels, the MDA capability for failure prediction may take role of coordination in cross domain. When MDA capability takes role of coordination in cross domain, the MDA capability for failure prediction can collect analytics output of failure prediction from single domain management and provide recommendation actions accordingly.

If necessary, MDA could also provide corresponding recommended actions for failure prevention.

7.2.3.1.3	Requirements

Table 7.2.3.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-FAILURE_PRED_MDA-01 | Void | Void |
| REQ-FAILURE_PRED_MDA-02 | MDA capability for failure prediction shall be able to obtain basic health maintenance knowledges (including, the relationship between the failures or potential failures and the related maintenance actions) through predefined expertise or model training. | Failure prediction |
| REQ-FAILURE_PRED_MDA-03 | MDA capability for failure prediction shall be able to provide the analytics output including predictions of potential service failures, as well as the possible recommendation actions to prevent failures. | Failure Prediction |
| REQ-FAILURE_PRED_MDA-04 | MDA capability for failure prediction should include the ability to predict failures across or within domains and provide analytics outputs for predicted failures. | Failure Prediction |
| REQ-FAILURE_PRED_MDA-05 | MDA capability for failure prediction should be able to provide the analytics output including information used for failure prediction based on the threshold information provided by the consumer. | Failure Prediction |

End of changes