3GPP TSG-SA5 Meeting #159	S5-250390

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Samsung

# Title:	Network Issue Inducement UC and Requirements

# Document for:	Approval

Agenda Item:	6.19.5.1

# 1	Decision/action requested

Group is requested to discuss and agree on the “Detailed Proposal”.

# 2	References

[1]	None

# 3	Rationale

# 4	Detailed proposal

First Change

## 5.x	NDT support to network automation capability

### 5.x.1	Description

NDTs may be used to support many automations use cases (e.g. MDA, SON, etc.). An NDT may be integrated into a network automation function, or it may be external to the network automation function. In the case where the NDT is external to the network automation function, it could be possible for the network automation function to configure the NDT and the scenario that could be modelled and simulated by the NDT. Then the NDT needs to implement the defined scenario, simulate it, and subsequently provide an output representing the statues of different network metrics for the simulated scenario. NDTs may not make decisions for the configuration of live network but could support to make decision recommendations.

An NDT, depending upon the network or service management use case and scenario to be modelled and simulated, might need data originating from various sources (network data, environment data, analytics, UEs data) and suitable hardware/software resources to function properly. MnS consumers would prefer to specify needed NDT characteristics or configurations to the NDT tailored to fulfil consumer specific needs i.e. to define the consumer preference for the specific NDT. For example, consumer preferences may be related to environment data sources e.g. weather, synthetic data etc, data characteristics (e.g. robustness, data granularity, maximum tolerable latency), required NDT output latency, characteristics of the service to be twinned, resource constraints (HW/SW), etc. Furthermore, in the case that consumer's preference on NDT characteristics or configuration may change over time and MnS consumer may update the NDT with the needed changes.

The achieved performance by NDT may depend on NDT characteristics such as e.g load; the simulation results would change based on the network load, time; the simulation results would change based on the target time of the day, energy saving state; the simulation results would change considering whether the simulated node is running in energy saving mode.

### 5.x.2	Potential Requirements

REQ-NDT-1: The NDT should support a capability to model the behavior of the network and provide the outcomes of such modelling to consumers.

REQ-NDT-2: The NDT should support a capability enabling an Mns consumer to define the network scenario that should be modelled and simulated.

REQ-NDT-3: The NDT should support a capability to provide an output representing the statues of different network metrics for the simulated scenario.

REQ-NDT-4: The NDT should support a capability enabling an MnS consumer to specify NDT characteristics or configurations

NOTE:	Example characteristics include:

-	Environment data sources (e.g. weather, synthetic data etc), and the related data characteristics (e.g. robustness, data granularity, maximum tolerable latency),

-	Environment characteristics  (e.g., network load, time of the day and energy saving state).

REQ-NDT-5: The NDT should support a capability to inform the MnS consumer whether a specific set of NDT characteristics or configurations defined by the Mns consumer are feasible or if not, what changes could be made to the NDT configurations to make then feasible.

REQ-NDT-6: The NDT should support a capability to provide a report/output for NDT simulation/emulation achieving certain target performance threshold and QoS.

End of Change