3GPP TSG SA WG5 Meeting #159	S5-250401

Sophia-Antipolis, FRANCE 17 - 21 February 2025

# Source:	Huawei

# Title:	Add conclusion for Topic 2

# Document for:	Approval

Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to approve the following update in TR 28.851 [1].

2	References

[1]	3GPP TR 28.851: "Study on charging aspects of next generation real time communication services phase 2".

3	Rationale

This pCR provides the conclusion for Topic 2 on "charging for DC application download and usage " in TR 28.851 [1].

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.851 [1].

First Change

5.2.6	Conclusion

For key issue #1, solution #x is recommended into the normative work, to support the volume based charging per IMS bootstrap DC for application downloading. No extension to the charging information is required. The charging principle can be specified in TS 32.255 [10].

The solution #1 for event based charging for applicaton download requires to specify a new CTF, which may be re-evaluated in future depends on business requirement.

For key issue #2, solution #2 and solution #y are recommended into the normative work, to support the duration and volume based charging per IMS application DC. No extension to the charging information is required. The charging principle can be specified in TS 32.255 [10] and TS 32.260[11].

End of Change