3GPP TSG SA WG5 Meeting #159	S5-250404

Sophia-Antipolis, FRANCE 17 - 21 February 2025

# Source:	Huawei, CMCC

# Title:	New solutions for UAV indication

# Document for:	Approval

Agenda Item:	7.5.4

1 	Discussion

This is a pCR to provide new solutions for Topic 1 in UAS charging in TR 28.853.

2	References

[1]	3GPP TR 28.853: "Study on charging aspects of uncrewed aerial systems".

3	Rationale

To distinguish UAV from other types of 3GPP UE, one possible way is to introduce a generic UE type with enumerated values. This solution allows future extensibility in case more types of UE require charging differentiation.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.853 [1].

First change

5.1.4.x	Solution #1.x: Generic UE Type to identify UAV

5.1.4.x.1	General description

This solution #1.x addresses Key Issue#1a. To distinguish the UAV or any other future proofed 3GPP UE types for charging differentiation, a generic IE can be introduced to hold the specific type of a UE.

As per TS 23.256 [3] clause 5.2.2 and clause 5.2.3, AMF / SMF can receive the UAV information (e.g. CAA-Level UAV ID or the UAV) in the UE Registration / PDU Session Establishment request. In this case, both AMF and SMF can identify the specific type of the UE, e.g. whether it is a UAV or any other specific type of UE.

The User Information IE, which are part of the PDU session charging information in TS 32.255 [10] and Registration charging information in TS 23.256 [12], can be extended to include a new sub IE, e.g. UE Type, to indicate the specific type of UE defined in the 3GPP system. Table 5.1.4.x.1-1 demonstrate the example of the extended User Information.

Table 5.1.4.x.1-1: Extension to the User Information IE

| Information Element | Category | Description |
| --- | --- | --- |
| User Information | OM | Group of user information. |
| User Identifier | OC | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) It is used for identifying the user in case SUPI is not present during emergency service. The detail identification of the wireline access is specified in clause 4.7.7 of TS 23.316 [203]. |
| unauthenticated Flag | OC | This field indicates the served SUPI is not authenticated. |
| Roamer In Out | OC | This field holds an indication if the roamer is in-bound or out-bound. This field is present only if UE is identified as a roamer. |
| UE Type | OC | This field indicates the specific type of UE defined in the 3GPP system, e.g. UAV when charging for the UAV scenario. |

End of change