3GPP TSG SA WG5 Meeting #159	S5-250405

Sophia-Antipolis, FRANCE 17 - 21 February 2025

# Source:	Huawei

# Title:	Conclusion for A2X communication charging

# Document for:	Approval

Agenda Item:	7.5.4

1 	Discussion

This is a pCR to provide conclusion for Topic 4 in UAS charging in TR 28.853.

2	References

[1]	3GPP TR 28.853: "Study on charging aspects of uncrewed aerial systems".

3	Rationale

Disentangle the key issues in the Topic 4, by aligning with each of the requirement.

Recommend the solution for normative work in Topic 4.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.853 [1].

First change

5.4.3	Key issues

5.4.3.1	Key issue #4a: Charging information required for A2X service parameter provisioning

This key issue is for investigating how to support the A2X services for UAS charging considering REQ- 3GPPCH-A2X-01. This investigation covers the following:

-	identification of the charging architecture for A2X service parameter provisioning;

-	identification of triggers and charging information required to A2X service parameter provisioning.

5.4.3.2	Key issue #4b: Charging information required for non-roaming A2X communication

This key issue is for investigating how to support the A2X services for UAS charging considering REQ- 3GPPCH-A2X-02. This investigation covers the following:

-	identification of the charging architecture for non-roaming A2X communication;

-	identification of triggers and charging information required to non-roaming A2X communication.

5.4.3.3	Key issue #4c: Charging information required for roaming A2X communication

This key issue is for investigating how to support the A2X services for UAS charging considering REQ- 3GPPCH-A2X-03. This investigation covers the following:

-	identification of the charging architecture for roaming A2X communication;

-	identification of triggers and charging information required to roaming A2X communication.

Next Change

5.4.5	Evaluation

5.4.5.1	Solutions evaluation for Key issue #4a

The solution #4.1 addresses Key issue #4a for A2X service parameter provisioning using UAS NF/NEF to provide the charging information collecting and reporting based on the chargeable events about Provisioning for A2X communication API invocation from USS/TPAE. The NEF charging specified in the TS 32.254 [13] can be used for the UAS NF/NEF charging with the identifiers of AF (A2X application Server) and UAS UE (A2X application), see the clause 5.3.5.1.

5.4.5.2	Solutions evaluation for Key issue #4b

The solution #4.2 addresses Key issue #4b for non-roaming A2X communication using SMF/MB-SMF support the unicast, broadcast and multicast communication. The SMF charging specified in TS 32.255 [10] can be used to support the charging for non-roaming A2X communication with PDU session. The MB-SMF charging specified in TS 32.279 [11] can be reused to support the charging for non-roaming A2X communication with MBS session.

5.4.5.3	Solutions evaluation for Key issue #4c

The solution #4.3 addresses Key issue #4c for roaming A2X communication over Uu, using SMF support unicast service roaming communication. As defined in TS 23.247 [9], multicast/broadcast service for roaming is not supported in this release. The SMF charging specified in TS 32.255 [10] can be used to support the charging for roaming A2X communication with PDU session.

Next Change

5.4.6	Conclusion

For A2X service parameter provisioning, NEF charging specified in the TS 32.254 [13] can be reused, i.e. solution #4.1.

For non-roaming A2X communication, SMF charging specified in the TS 32.255 [10] and MB-SMF charging specified in TS 32.279 [11] can be reused, i.e. solution #4.2.

For roaming A2X communication, SMF charging specified in the TS 32.255 [10] can be reused, i.e. solution #4.3.

Next Change

6.	Conclusions and Recommendations

The present technical report studied 4 Topics for charging aspects of uncrewed aerial systems with the following conclusions:

-	Converged charging with UAV indication.

-	Charging support of C2 Communication.

-	Service Exposure to the USS: the solution #3.1 based on the UAS NF/NEF charging is selected in the clause 5.3.5.

-	Aircraft-to-Everything (A2X) services charging: the solution #4.1 based on NEF charging is recommended to support the charging for A2X service parameter provisioning. The solution #4.2 based on SMF charging and MB-SMF charging is recommended to support the charging for non-roaming A2X communication. The solution #4.3 based on SMF charging is recommended to support the charging for roaming A2X communication.

Editor's Note: The conclusion for UAV indication and C2 communication is FFS.

End of change