# 3GPP TSG SA WG5 Meeting #159	S5-250409

# Sophia-Antipolis, FRANCE 17 - 21 February 2025

# Source:	Huawei

# Title:	Discussion on inter-CHF failure handling

# Document for:	Discussion

Agenda Item:	7.3

1	Decision/action requested

Discuss the failure handling mechanism for inter-CHF communication.

2	References

[1]	S5-246608: "Discussion Paper on CHF as NF consumer detected failure".

[2]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[3]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2".

[4]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system, charging service; Stage 3".

3	Rationale

3.1			Background

Figure 1: A demonstration of inter-CHF failure handling

With inter-CHF interaction, when consumer CHF towards producer CHF request times out (Figure 1), the SA5 charging group has reached partial agreement on the inter-CHF failure handling mechanism during the discussion on S5-246608 [1] at SA5#158, i.e.:

-	Agreement#1: The application level failure handling (Terminate, Continue, Retry_and_terminate) only applies to how NF(CTF) handles the service in case of failure communication with CHF. It does not apply to the inter-CHF interaction.

-	Agreement#2: When failure happens between CHFs, it is up to the consumer CHF to decide how to handle the charging session with NF(CTF) based on operator agreement.

Based on the above agreements, the failure handling mechanism in TS 32.290 [2] clause 5.5.1.3 need to be modified.

However, there are leftover issues to be further discussed in order to complete the inter-CHF failure handling mechanism.

3.2			Observations and Proposals

Observation#1:

The value for failure handling is carried in the Invocation Result within the Charging Data Response, according to TS 32.290 [2] Table 7.2. In the inter-CHF interaction, as demonstrated in Figure 2, there are two charging data response:

-	Charging Data Response#1 is sent from producer CHF to consumer CHF, which may contain the value of failure handling reported by producer CHF. However, according to the Agreement#1, the value in this charging data response does not make sense to the failure handling between the inter-CHF charging session. Furthermore, as discussed with S5-246608 [1], the proposal to expand new values for inter-CHF failure handling was not preferred.

-	Charging Data Response#2 is sent from consumer CHF to NF (CTF), which may contain the failure handling values. It is not clear how does consumer CHF handle the value of failure handling received from producer CHF (i.e. Failure Handling #1), e.g. should it use or ignore the value when it sends the failure handling (i.e. Failure Handling #2) to NF (CTF)?

Figure 1-3: A demonstration of failure handling delivered for inter-CHF interaction

Proposal#1:

Introduce the following clarification to TS 32.290 [2] clause 5.5.1.3:

-	Consumer CHF may update the application level failure handling to the NF (CTF) based on operator agreement, i.e. ignores the value of failure handling when a Charging Data Response [Initial/Update/Termination] received from the producer CHF.

Observation#2:

Despite the failure handling between consumer CTF and NF(CTF) is based on operator agreement (i.e. Agreement #2), there are a few uncertainties to be clarified, i.e.

-	in case the consumer CHF (e.g. V-CHF) decides to continue the session with NF (CTF), the producer CHF (e.g. H-CHF) should be aware of the charging information for settlement.

-	in case the consumer CHF decides to terminate the session with NF(CTF), the NF (CTF) should be aware of the reason is due to inter-CHF communication failure, e.g. the 504 Gateway Timeout application error which was previously introduced in TS 32.291 [4].

Proposal#2:

Introduce the following clarification to TS 32.290 [2] clause 5.5.1.3:

In the case of the consumer CHF towards producer CHF request times out and without alternative producer CHF, consumer CHF decides how to handle the charging session with NF (CTF) based on operator agreement, i.e.:

-	If consumer CHF decides to continue the session with NF (CTF), consumer CHF stores the charging information received from NF (CTF) for settlement.

-	If consumer CHF decides to terminate the session with NF (CTF), consumer CHF sends the response code (i.e. 504 Gateway Timeout) for CTF to indicate the termination of charging session.

Clarify the description for the 504 Gateway Timeout error code in TS 32.291 [4].

4	Detailed proposal

A set of Rel-18 and Rel-19 CRs (S5-250410/S5-250411/S5-250412/S5-250413) are proposed in this meeting for approval.