3GPP TSG-SA5 Meeting #159	S5-250439

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0300 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Add use-case and requirements for intent driven management of network maintenance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Add introduction for use-case on intent driven management of network maintenance as it was studied in clause 5.19 of TR 28.914 and recommended for normative phase in clause 6.19 of TR 28.914 - Add requirements for use-case on intent driven management of network maintenance as it was studied in clause 5.19 of TR 28.914 and recommended for normative phase in clause 6.19 of TR 28.914 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add use case and requirements for intent driven management of network maintenance based on clause 5.19 and clause 6.19 of TR 28.914 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new use-case and corresponding requirements for intent driven management of network maintenance is missing in TS 28.312 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-250439 is the re-submission of S5-247181 (endorsed in SA5#158) |  |  |  |  |  |  |  |  |

Start of First change

# 5	Specification Level Requirements

## 5.1	Use cases

### 5.1.x	Intent containing an expectation for network maintenance

#### 5.1.x.1	Introduction

This use case describes a scenario in which an MnS consumer expresses its intent containing an expectation for delivering a network maintenance work, for example, a software upgrade. The expectation may be termed as network maintenance expectation. The network maintenance expectation may include information to describe the characteristics of the network maintenance expectation. Furthermore, the network maintenance expectation may include information for referring to any information specific to the network maintenance work, for example, maintenance target version, the start/end time of the intended network maintenance work and any additional guidelines for network maintenance.

The network maintenance expectation may also use expectation targets specified for other use-cases defined in the present document. For example, the network maintenance expectation may have RAN Energy consumption target, if the maintenance of a RAN network element aims at reducing the RAN Energy consumption.

The network maintenance expectation may also use existing expectation context, for example, to identify where the network maintenance takes place.

Based on network maintenance expectation received, MnS producer takes actions to fulfil the expectation and monitors the impact of the actions taken.  The actions may include, for example, preparing the NE/NF for maintenance to avoid any disruption in the network, and carrying out the intended maintenance tasks such as upgrading the related files of NE/NF, and any other actions to ensure fulfilment of expectation targets of network maintenance expectation.

The MnS producer notifies MnS consumer about the fulfilment information of the intent containing an expectation for network maintenance, including information regarding conflicts that might occur during fulfilment of network maintenance expectation.

#### 5.1.x.2	Requirements

REQ-Intent_NetworkMaintenance_CON_1: The intent driven MnS producer shall have the capability enabling MnS consumer to express intent containing a network maintenance expectation.

REQ-Intent_NetworkMaintenance_CON_2: The intent driven MnS producer shall have the capability enabling MnS consumer to obtain intent report information (including fulfilment information) for the intent containing a network maintenance expectation.

End of changes