3GPP TSG-SA5 Meeting #159	S5-250444

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.561 Add use case and requirements on Network issue inducement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NDT |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Use NDT for network issue inducement |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding use case and requirements on network issue inducement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not supported to use NDT for network issue inducement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.a (new clause 5.a added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X] 	ETSI GR ZSM 015: "Zero-touch network and Service Management (ZSM); Network Digital Twin".

2nd Change

5	Specification level use cases and requirements

5.a Automation

5.a.1	Network issue inducement

5.a.1.1	Description

This use case describes how a network issue can be induced using NDT. In order to develop a resilient network, the behavior and performance of the network should be monitored during certain network failure issue e.g. node/functionality failure, service degradation etc. In order to plan for the optimal network configuration in case of such network failure issue, the scenario itself need to be induced in the network. It is desirable, to use NDT for such an inducement process. A particular issue can be induced in the NDT instead of real network. After a particular issue is induced, the performance of the network can be monitored, other degradation/faults/failure can be identified and the mitigation actions can be decided and reported. The following are some of the examples of the issues that can be induced.

The network slice performance degradation in terms of low PDU session establishment success rate or in terms of high latency can be induced, in a NDT, to see how the related network functions will behave when the PDU session establishment success rate is degraded. The remedial actions can be decided to mitigate the problem.

The coverage hole can be induced, in a NDT, to see how the related services are getting effected. The remedial actions can be decided to mitigate the problems arising due to the induced coverage hole.

NDT can be used for fault injection experiments avoiding impact on the physical network while measuring and monitoring the impact of each injected fault in the NDT simulation. This could be leveraged, to build a training dataset for enhancing and enriching detection and diagnosing systems capabilities. In addition, NDT could be leveraged for improving root causes analysis.

Mobile network has become crucial infrastructure and the impact of network failures on society can be substantial. Therefore, it is important to consider measures against potential future network failures. To this end, it is useful to proactively analyse potential future network failures (e.g. analyse potential network failures caused by component failures like VMs or containers turned down). It is impossible to cause issues in actual commercial networks because it would affect users. Therefore, the use of Digital Twin can be investigated for this purpose.

NDT fault injection analysis is described in ETSI GR ZSM 015 [X], clause 5.13. In this use case, it is described as non-disruptive way of doing fault injection studies. Network digital twin simulates potential fault scenarios and provide results. This enables operator to learn network anomaly patterns of different faults.

For example, mobile networks are built on physical resources and can be built on virtualization technologies. Predicting the impacts on the network induced by the failures on physical resources (e.g. CPU, memory, storage failure of physical servers or port and transmission error of physical networks) or virtual resources (e.g. stopping of VMs or containers, host OS failures) can be challenging. Additionally, with the introduction of technologies like containers, the number of components in a mobile network increase, making it difficult to analyse the network-wide impacts of a single component failure. In the increasingly complex modern networks, it can also be difficult to ascertain the effectiveness of the countermeasures to those failures.

Furthermore, to allow verification of countermeasures, operator can repeat simulation of a failure on the network within NDT after applying the countermeasures, and observe the results, which can confirm the effectiveness of the countermeasures.

The consumer can request NDT to simulate failures and receive the simulation results regarding the impact on the network.

Moreover, network failures can sometimes cause issues originating in a part of the network to propagate throughout the entire network. In such cases, it is necessary to simulate not just individual components, but the entire network or a significant part as network digital twin. Additionally, as networks are structured in layers, an issue in one layer can impact other layers. Therefore, to accurately reproduce the entire network or its parts, it is crucial that 3GPP management system provide the network topology including how network elements are interconnected.

5.a.1.2	Requirements

REQ-NDTN_Induce-1: The NDT shall support a capability enabling a network issue (e.g. fault/failure) to be induced.

REQ-NDTN_Induce-2: NDT shall have a capability enabling the MnS consumer to measure and monitor the impact of the injected issue.

REQ-NDTN_Induce-3: The management system shall have a capability to provide topology data for simulating and/or emulating network failures in NDT.

End of Changes