3GPP TSG-SA5 Meeting #159	S5-250449

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0177 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 Update solution for control plane congestion analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The analytics output for 5GC control plane congestion analysis is updated because S5-247164 was endorsed to add the requirement that MDA capability for control plane congestion analysis should include providing root cause of the congestion issue and the predicted duration of congestion issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add rootCause and predictedCongestionEndTime as two new information elements for control plane congestion analysis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Information of root cause and predicted end time for control plane congestion issue is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.7.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

8.4.7.1.3	5GC Control plane congestion analysis

8.4.7.1.3.1	MDA type

The MDA type for 5GC control plane congestion analysis is: ResourceAnalytics.5GCControlPlaneCongestionAnalysis.

8.4.7.1.3.2	Enabling data

The enabling data for ResourceAnalytics.5GCControlPlaneCongestionAnalysis MDA type are provided in table 8.4.7.1.3.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.7.1.3.2-1: Enabling data for 5GC control plane congestion analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Registration procedure related measurements for AMF. | Number of registration requests (clause 5.2.2 of TS 28.552 [4]) Mean time of Registration procedure (clause 5.2.2.9 of TS 28.552 [4]) |
|  | Service Request procedure related measurements for AMF. | Number of service requests (clause 5.2.3.3 and clause 5.2.3.4 of TS 28.552 [4]) |
|  | Number of PDU sessions measurements for SMF | Number of PDU sessions (clause 5.3.1 of TS 28.552 [4]) |
|  | QoS flows measurements for SMF | QoS flows monitoring (clause 5.3.2 of TS 28.552 [4]) |
|  | VR (including Virtual CPU, Virtual Memory, and Virtual Disk) usage of NF | VR usage of NF (clause 5.7.1 of TS 28.552 [4]) |
| Alarm notifications | Alarm information, e.g. the alarm notification of network functions. | Alarm information and notifications as per TS 28.532 [11] |
| Configuration data | MOIs of 5GC NFs. | 5GC NRM as defined in TS 28.541 [15] |

8.4.7.1.3.3	Analytics output

The specific information elements of the analytics output for control plane congestion analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.7.1.3.3-1.

Table 8.4.7.1.3.3-1: Analytics output for 5GC control plane congestion analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| affectedObject | Indication of 5GC NFs where congestion issues occurred or potentially may occur. | M | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cPCongestionIssueID | This field holds the ID of the control plane congestion issue which is reported. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to orchestrate the resource allocation for 5GC NFs. The recommended action may be (but not limited to): - scale out a list of 5GC NFs; | O | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| rootCause | The root cause of control plane congestion issue, such as network failure, signalling overload. | O | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| predictedCongestionEndTime | It indicates the predicted end time of the congestion if the recommended actions are not performed. | O | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes