3GPP TSG-SA5 Meeting #159	S5-250476

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0181 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 Add stage 3 definition for MDA Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution propose to add Stage 3 definition for MDA Management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add stage 3 definition for MDA Management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation of MDA management will be unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X, X.1, X.1.1, X.1.2, X.2, X.2.1, X.2.2, X.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of next change

X	Stage 3 definition for MDA NRM

X.1	RESTful HTTP-based solution set

he RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in 3GPP TS 28.532 [11]. Corresponding className is MDARequest and MDAReport.

X.1.1	MDA request management

Following is the SS to support MDA request management based on Table 12.1.1.1.1-1 in TS 28.532 [11].

Table X.1.1-1: SS to support MDA request management

| MDA request management | IS operation | HTTP Method | Resource URI |
| --- | --- | --- | --- |
| Create an MDA request | createMOI operation | PUT | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{MDARequest}={id} |
| Delete an MDA request | deleteMOI operation | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{MDARequest}={id} |
| Modify an MDA request | modifyMOIAttributes operation | PUT PATCH | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{MDARequest}={id} |
| Query an MDA request | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{MDARequest}={id} |

X.1.2	MDA report management

Following is the SS to support MDA report management based on Table 12.1.1.1.1-1 in TS 28.532 [11].

Table X.1.1-1: SS to support MDA reporting (file-based) management

| MDA report management | IS operation | HTTP Method | Resource URI |
| --- | --- | --- | --- |
| Subscribe an MDA report | subscribe operation | POST | {MnSRoot}/FileDataReportingMnS/{MnSVersion}/subscriptions |
| Unsubscribe an MDA report | unsubscribe operation | DELETE | {MnSRoot}/FileDataReportingMnS/{MnSVersion}/subscriptions/{subscriptionId} |
| List available of MDA report lists | listAvailableFiles operation | GET | {MnSRoot}/FileDataReportingMnS/{MnSVersion}/files |

Table X.1.1-1: SS to support MDA report (streaming-based) management

| MDA report management | IS operation | HTTP Method | Resource URI |
| --- | --- | --- | --- |
| Establish an MDA report stream | establishStreamingConnection | HTTP POST HTTP GET | {MnSRoot}/StreamingDataReportMnS/{MnSVersion}/connections |
| Terminate an MDA report stream | terminateStreamingConnection | WebSocket Close frame sent, and WebSocket Close frame received | {MnSRoot}/StreamingDataReportMnS/{MnSVersion}/connections/{connectionId} |
| Report an MDA report stream | reportStreamData | WebSocket Data frame sent | {MnSRoot}/StreamingDataReportMnS/{MnSVersion}/connections/{connectionId} |
| Add an MDA report stream | addStream | HTTP POST | {MnSRoot}/StreamingDataReportMnS/{MnSVersion}/connections/{connectionId}/streams |
| Delete an MDA report stream | deleteStream | HTTP DELETE | {MnSRoot}/StreamingDataReportMnS/{MnSVersion}/connections/{connectionId}/streams |
| Query an MDA report stream | getConnectionInfo | HTTP GET | {MnSRoot}/StreamingDataReportMnS/{MnSVersion}/connections {MnSRoot}/StreamingDataReportMnS/{MnSVersion}/connections/{connectionId} |
| Query an MDA report stream | getStreamInfo | HTTP GET | {MnSRoot}/StreamingDataReportMnS/{MnSVersion}/connections/{connectionId}/streams {MnSRoot}/StreamingDataReportMnS/{MnSVersion}/connections/{connectionId}/streams/{streamId} |

Table X.1.1-1: SS to support MDA report (notification-based) management

| MDA report management | IS operation | HTTP Method | Resource URI |
| --- | --- | --- | --- |
| Query an MDA report | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{MDAReport}={id} |

X.2	OpenAPI specification

X.2.1	OpenAPI document for provisioning MnS

The OpenAPI/YAML definitions for provisioning MnS are specified in 3GPP Forge, refer to clause 4.3 (OpenAPI Definitions) of TS 28.623 [29] for the Forge location. An example of Forge location is: "https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel19_SA106/".

Directory: OpenAPI

File: TS28532_ProvMnS.yaml

X.2.2	OpenAPI document for MDA request NRM

The OpenAPI/YAML definitions for MDA request NRM are specified in 3GPP Forge , refer to clause 4.3 (OpenAPI Definitions) of TS 28.623 [29] for the Forge location. An example of Forge location is: "https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel19_SA106/".

Directory: OpenAPI

File: TS28104_MdaNrm.yaml

X.2.3	OpenAPI document for MDA report NRM

The OpenAPI/YAML definitions for MDA report NRM are specified in 3GPP Forge, refer to clause 4.3 (OpenAPI Definitions) of TS 28.623 [29] for the Forge location. An example of Forge location is: "https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel19_SA106/".

Directory: OpenAPI

File: TS28104_MdaReport.yaml

End of change