3GPP TSG-SA5 Meeting #159	S5-250478

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0671 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Update the handover related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 38.401, it defines the handover procedure between one gNB-DU to another gNB-DU within the same gNB-CU. In TS 28.552, the intra-hanover related measurements only defines handovers between cells within the same gNB, and does not consider handovers between gNB-DUs within the same gNB-CU in case of intra-gNB handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the intra-gNB measurements to support intra-gNB handover between gNB-DUs within the same gNB-CU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The intra-gNB-CU mobility measurements can not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.2.1, 5.1.1.6.2.2, 5.1.1.51.1, 5.1.1.51.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of next change

5.1.1.6.2	Intra-gNB handovers

5.1.1.6.2.1	Number of requested legacy handover executions

a)	This measurement provides the number of outgoing intra gNB legacy handover executions requested by the source NRCellCU.

b)	CC.

c)	On transmission of RRC Reconfiguration message to the UE triggering the legacy handover from the source NRCellCU to the target NRCellCU (see TS 38.331 [20]), or handover from the source NRCellDU to the target NRCellDU (see TS 38.401 [66]), indicating the attempt of an outgoing intra-gNB legacy handover, the counter is stepped by 1.

d)	A single integer value.

e)	MM.HoExeIntraReq.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

5.1.1.6.2.2	Number of successful legacy handover executions

a)	This measurement provides the number of successful intra gNB legacy handover executions received by the source NRCellCU.

b)	CC.

c)	On reception of RRC ReconfigurationComplete message from the UE to the target NRCellCU (see TS 38.331 [20]) or to the target NRCellDU (see TS 38.401 [66]) indicating a successful intra gNB legacy handover, the counter is stepped by 1.

d)	A single integer value.

e)	MM.HoExeIntraSucc.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

Next change

5.1.1.51	Measurements related to MLB

5.1.1.51.1	Inter-gNB handovers for MLB

5.1.1.51.1.1	Number of requested inter-gNB handover preparations for MLB

a)	This measurement provides the number of handover preparations requested by the source gNB CU for MLB.

b)	CC.

c)	On transmission of HANDOVER REQUIRED message (see TS 38.413 [11]) by the gNB CU to the AMF, or transmission of HANDOVER REQUEST message (see TS 38.423 [13]) by the source gNB CU to target the target gNB CU, where the message contains the cause IE set to “Reduce Load in Serving Cell” or “Resource Optimisation Handover”. Each message increments the relevant subcounter per cause by 1.

d)	Each measurement is an integer value.

e)	MM.HoPrepInterReqMLB.ResOpt;
MM.HoPrepInterReqMLB.RedLoad;

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

5.1.1.51.1.2	Number of successful inter-gNB handover preparations for MLB

a)	This measurement provides the number of successful handover preparations for MLB received by the source gNB CU.

b)	CC.

c)	On receipt of HANDOVER COMMAND message by the gNB CU from the AMF (see TS 38.413 [11]), or receipt of HANDOVER REQUEST ACKNOWLEDGE message (see TS 38.423 [13]) from the target gNB CU, indicating that the resources for the handover have been prepared at the target NR cell CU, where the message corresponds to a previously requested inter-gNB handover preparations for MLB (see clause 5.1.1.51.1.1).

d)	Each measurement is an integer value.

e)	MM.HoPrepInterSuccMLB.ResOpt;
MM.HoPrepInterSuccMLB.RedLoad;

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

5.1.1.51.1.3	Number of failed inter-gNB handover preparations for MLB

a)	This measurement provides the number of failed handover preparations received by the source gNB CU for MLB. This measurement is split into subcounters per failure cause.

b)	CC.

c)	On receipt of HANDOVER PREPARATION FAILURE message (see TS 38.413 [11]) by the gNB CU from the AMF, or receipt of HANDOVER PREPARATION FAILURE message (see TS 38.423 [13]) from the target gNB CU, indicating that the preparation of resources at the target NR cell CU has failed, where the message corresponds to a previously requested inter-gNB handover preparations for MLB (see clause 5.1.1.51.1.1). Each message increments the relevant subcounter per failure cause by 1.

d)	Each subcounter is an integer value.

e)	MM.HoPrepInterFailMLB.cause;
Where cause identifies the failure cause of the handover preparations for MLB.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

5.1.1.51.1.4	Number of requested inter-gNB handover executions for MLB

a)	This measurement provides the number of outgoing inter-gMB handover executions requested by the source gNB for MLB.

b)	CC.

c)	On transmission of RRCReconfiguration message to the UE requesting the inter-gNB handover from the source gNB CU to the target gNB CU (see TS 38.331 [20]), corresponding to the previously requested inter-gNB handover preparations for MLB (see clause 5.1.1.51.1.1).

d)	Each measurement is an integer value.

e)	MM.HoExeInterReqMLB.ResOpt;
MM.HoExeInterReqMLB.RedLoad.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

5.1.1.51.1.5	Number of successful inter-gNB handover executions for MLB

a)	This measurement provides the number of successful inter-gNB handover executions for MLB received by the source gNB.

b)	CC.

c)	On receipt at the source gNB of UE CONTEXT RELEASE (see TS 38.423 [13]) over Xn from the target gNB following a successful inter-gNB handover for MLB; or, if handover is performed via NG, on receipt of UE CONTEXT RELEASE COMMAND (see TS 38.413 [11]) from AMF following a successful inter-gNB handover for MLB, where the message denotes a handover corresponding to the a previously requested inter-gNB handover executions for MLB (see clause 5.1.1.51.1.4).

d)	Each measurement is an integer value.

e)	MM.HoExeInterSuccMLB.ResOpt;
MM.HoExeInterSuccMLB.RedLoad.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

5.1.1.51.1.6	Number of failed inter-gNB handover executions for MLB

a)	This measurement provides the number of failed inter-gNB handover executions at a source gNB for MLB.

b)	CC.

c)	This counter is incremented when failure occurs for the handover execution triggered by MLB. It is assumed that the UE context is available in the source gNB. The following events corresponds to a previously requested inter-gNB handover executions for MLB (see clause 5.1.1.51.1.4):

1)	On reception of NGAP UE CONTEXT RELEASE COMMAND (see TS 38.413 [11]) from AMF indicating an unsuccessful inter gNB handover;

2)	On reception of RrcReestablishmentRequest (see TS 38.331 [20]) where the reestablishmentCause is handoverFailure, from the UE in the source gNB, where the reestablishment occurred in the source gNB;

3)	On expiry of a Handover Execution supervision timer in the source gNB;

4)	On reception of XnAP RETRIEVE UE CONTEXT REQUEST (see TS 38.423 [13]) in the source gNB, when the reestablishment occurred in another gNB.

The failure causes for UE CONTEXT RELEASE COMMAND are listed in TS 38.413 [11] clause 9.3.1.2, this message increments the relevant subcounter per failure cause by 1.

As one handover failure might cause more than one of the above events, duplicates need to be filtered out.

d)	Each subcounter is an integer value.

e)	MM.HoExeInterFailMLB.UeCtxtRelCmd.cause;
MM.HoExeInterFailMLB.RrcReestabReq;
MM.HoExeInterFailMLB.HoExeSupTimer;
MM.HoExeInterFailMLB.RetrUeCtxtReq.

Where cause identifies the failure cause of the UE CONTEXT RELEASE COMMAND message.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

5.1.1.51.2	Intra-gNB handovers for MLB

5.1.1.51.2.1	Number of requested intra-gNB handover executions for MLB

a)	This measurement provides the number of outgoing intra-gNB handover executions triggered by MLB.

b)	CC.

c)	On transmission of RRC Reconfiguration message triggred by MLB to the UE requesting the handover from the source NRCellCU to the target NRCellCU (see TS 38.331 [20]), or handover from the source NRCellDU to the target NRcellDU (see TS 38.401 [x]).

d)	A single integer value.

e)	MM.HoExeIntraReqMLB.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

5.1.1.51.2.2	Number of successful intra-gNB handover executions for MLB

a)	This measurement provides the number of successful intra-gNB handover executions for MLB received by the source NRCellCU.

b)	CC.

c)	On reception of RRC ReconfigurationComplete message from the UE to the target NRCellCU or to the target NRCellDU indicating a successful intra-gNB handover (see TS 38.331 [20]) corresponding to a handover execution request triggered by MLB (see clause 5.1.1.51.2.1).

d)	A single integer value.

e)	MM.HoExeIntraSuccMLB.

f)	NRCellCU;
NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

End of change