3GPP TSG-SA5 Meeting #159	S5-250486

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Huawei

# Title:	Rel-19 pCR TS 28.561 NDT lifecycle management

# Document for:	Approval

Agenda Item:	6.19.5.1

1	Decision/action requested

For approval

2	References

[1] 	3GPP TS 28.561: “Management aspects of Network Digital Twins”.

3	Rationale

This contribution proposes to explain the status of NDT lifecycle.

4	Detailed proposal

It is proposed to make the following changes to TS 28.561 [1].

1st Change

X	Life-cycle management of NDT

Figure X-1 represents different steps of NDT for lifecycle management.

Figure X-1: NDT lifecycle management

Creation: MnS producer creates an NDT function which represents the replica of mobile network and can provide NDT services to MnS consumer.

Activation: NDT function receives the request of a simulation task (i.e., an NDT service) and prepares the execution of this simulation task. NDT function can be reactivated when receives the new or modified request of a simulation task or continues a suspended simulation task.

Operation: The simulation task runs in an NDT function.

Deactivation: The simulation task is finished or suspended.

Deletion: The NDT function is deleted by MnS producer.

End of changes