3GPP TSG-SA5 Meeting #159	S5-250488

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Huawei

# Title:	Rel-19 pCR TS 28.561 Use case of RAN energy saving policy verification

# Document for:	Approval

Agenda Item:	6.19.5.1

1	Decision/action requested

For approval

2	References

[1] 	3GPP TS 28.561: “Management aspects of Network Digital Twins”.

3	Rationale

This contribution proposes to add the use case of RAN energy saving policy verification.

4	Detailed proposal

It is proposed to make the following changes to TS 28.561 [1].

1st Change

# x	Scenario specific use cases and requirements

## x.1	Use cases

### x.1.1	RAN ES policy verification using NDT

#### x.1.1.1	Introduction

When configuring the energy saving for RAN, normally the policy is applied in execution with monitoring and optimization loop to minimize the influence on network service quality. That's to say, there could be multiple ES policies executed iteratively in actual mobile network until the network performance, e.g. energy efficiency of NG-RAN, UE throughput in gNB, etc., meets certain requirements from operators. The digital twin technology may be used to evaluate the impact of RAN ES policy while satisfying simulation performance requirements.

In this scenario, MnS consumer (the entity who consumes the network simulation service) sends the request to verify the RAN ES policy to MnS producer. The request includes RAN ES Policy and performance requirements, which is MnS consumer’s requirements for network simulation performance, e.g. precision, maximum run time, etc. Then, the NDT performs network simulation, i.e., simulating the execution of RAN ES policy of the mobile network according to the performance requirements and collects its impact on the simulated network. The RAN ES policy can be seen as a specific set of network management operations achieved by provisioning MnS defined in TS 28.532 [11]. After that, MnS producer sents the simulation report to MnS consumer to indicates the network simulation results (e.g. Energy efficiency, RAN UE throughput, etc).

#### 5.1.1.2	Requirements

REQ-NDT-FUN-01: The NDT MnS for RAN should have the capability to estimate the impact of RAN ES policies.

REQ-NDT-FUN-02: The NDT MnS for RAN should have the capability allowing the consumer to configure the RAN ES policies.

REQ-NDT-FUN-03: The NDT MnS for RAN should have the capability to report the simulated impact of RAN ES policies.

End of changes