3GPP TSG-SA5 Meeting #159	S5-250497

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0302 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Add description of intent negotiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR28.914 has studied intent negotiation during intent fulfilment. This CR adds that use case, requirements and solution. This CR also emphasizes intent negotiation report, possible impacts and implicit intents discussed in TR28.914. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New use case, requirements and solutions for intent negotiation is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intent negotiation will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.X, 6.2.1.1, 6.2.1.2, 6.2.1.3, 6.2.1.4, A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.3.X	Negotiation on fulfilment of intents

5.3.X.1	Introduction

For a given intent, an MnS Consumer may express a feasible intent, but the producer may have multiple ways to fulfil it. One of these solutions might be better in one aspect and another solution might be better in another aspect. Although this intent is feasible, the producer may not have the knowledge and capability to determine which solution is appropriate.

Example, the MnS Consumer may express intent for energy saving but only specify the target for energy consumption reduction without more constraints, e.g. reduce 20 % energy consumption. There will be numerous solutions to fulfil the energy consumption target with various impacts, e.g. one solution may reduce the coverage and one may reduce the number of users.

The MnS consumer may have concerns on the possible impact of the selected solution by the Producer, which means that negotiation with the MnS Consumer is necessary. In such cases, the MnS consumer and Mns producer may negotiate on the best way to fulfil the intent.

Intent negotiation is a set of procedures which are applicable throughout the life cycle of an intent.  Several negotiations are possible for a feasible intent, many employing interactions that are similar.

NOTE 1:	Some of these negotiations can also be applicable during the feasibility check process.

5.3.X.2	Checking for fulfillable outcomes

The MnS consumer wants to know possible fulfillable outcomes for a given intent. The MnS consumer creates an intent that should be evaluated by the MnS producer to see options which the MnS producer can deliver.

Figure 5.3.X.2-1: MnS consumer requests and receives a list of fulfillable outcomes on an intent.

Many dynamic factors impact possible outcomes.  As a result, it is unrealistic to expect all potential outcomes to be reported.   The list of possible outcomes an MnS Producer may provide is left to implementation.

Subsequently, the MnS producer provides a report indicating what is fulfillable for each intent aspect (intentExpectation and expectationTarget) within that intent. Since different fulfillable outcomes may have different impacts, the report should include the relative impacts of each outcome.

NOTE 2:  the mechanism by which MnS Consumer can indicate a preference for a specific possible outcome is out of scope of this TR.

5.3.X.3	Checking for best possible outcome on an intent, intent expectation, or expectation target

The MnS consumer wants to know the best possible outcome for a given intent or intent expectation or expectation target. This could be prior to or during fulfilment.

Figure 5.3.X.3-1: Checking for best possible outcome on intent or
intent expectation or expectation target

The MnS consumer creates an intent that should be evaluated by the MnS producer with a request to provide the best possible outcome. Subsequently, the MnS producer provides a report indicating that best possible outcome. The best possible outcome is defined as follows:

-	The request is to evaluate specific expectation target(s) in an intent with multiple expectation targets (e.g. multiple expectations or one expectation with multiple expectation targets): The best possible outcome is the best value on specific expectation target(s) that maintains the other expectation targets to within the ranges specified in the intent.

-	The request is to evaluate all expectation targets in an intent with multiple expectation targets (e.g. multiple expectations or one expectation with multiple expectation targets): The best possible outcome is the best value on each expectation target that maintains the other expectation targets to within the ranges specified in the intent.

The MnS producer should support an fulfillable outcomes report that lists the fulfillable outcomes for any of the three scenarios, the report including the related impact on other targets in the intent or on other metrics and contexts.

5.3.X.4	MnS producer to provide information about possible fulfillment of the intent

The MnS consumer wants and requests to know for a specific property what the MnS producer recommends what could be applied for particular intent characteristics. The MnS producer has attempted to fulfil the intent and indicated that it cannot be fulfilled, so the MnS consumer asks the MnS producer to recommend what changes could be made to their intent or another of their intents to make the intent fulfillable. The MnS consumer can receive recommendations from MnS producer and use the response to modify their existing intents in an attempt to improve the ability to fulfil the new intents.

Figure 5.3.X.4-1: Enabling the MnS consumer to request and receive a recommendation on
the possible fulfillable intent properties prior to or in case of failure of fulfilment.

Subsequently, the MnS producer provides a  report indicating the information about possible fulfilment of the intent. The MnS producer should support a recommended-changes report that lists different information about possible fulfilment of intent, e.g., candidate changes to unfulfillable targets within an unfulfillable expectation.

5.3.X.5	MnS consumer advises on preferred alternatives

The MnS consumer wants an intent fulfilled. The intent is feasible, but the MnS producer has multiple alternatives related to fulfilling the intent. The MnS producer wants the MnS consumer to advise on their (the MnS consumer's) preference among these alternatives.

NOTE 1:	An alternative is the combination a set of expectation target values that the MnS producer can achieve together with their (expected) impacts on the network (objects). E.g. for an expectation target on energy consumption, the impact may include coverage area as a result of the energy consumption.

Figure 5.3.X.5-1: MnS consumer advises the MnS producer
on the preferences among alternatives at the MnS producer

After the MnS consumer creates an intent to be fulfilled, the MnS producer determines that there are multiple alternatives, so the MnS producer and provides a report to the MnS consumer so that the MnS consumer may help chose the best alternative.

The report to the MnS consumer may include:

-	The list of available/fulfillable expectation target values that the MnS producer is able to apply/achieve.

-	The expected relative impacts of the different alternatives - on aspects of the submitted intent or their  intents and intent expectations.

-	A request to the MnS consumer to select one among the alternatives.

Given the alternatives, the MnS consumer takes any combination of these actions:

-	Chooses and indicates the preferred alternative.

-	Defines the relative importance of their expectation Targets (in the form of a utility function) so that the MnS producer may consider these in deciding upon the solution/ solution approach/ closed loops/ action/ outcome to be applied/deployed/achieved.

-	Provides the relative satisfaction level that each of the alternatives achieves. The satisfaction level is the computation of the utility achieved by each alternative according to the MnS consumer’s utility function. It is an integer in the range  [0,100] . The highest possible value indicates that the solution provided by the MnS producer achieves the best possible outcomes that the MnS consumer expected, e.g., that it achieves the highest range of a target whose desired values were defining as falling in a range. Correspondingly, the values lower than 100 indicate how far from the maximum satisfaction the outcome is.

5.3.X.6	Requirements

REQ-Intent_Negotiation _01: The intent-driven MnS producer should have the capability to provide possible outcomes to the MnS Consumer.

REQ-Intent_Negotiation_02: The intent-driven MnS producer should have the capability enabling MnS Consumer to  decide appropriate outcome for fulfilment based on possible outcomes.

NOTE 1:	The definition for outcome is For Further Study.

REQ_INT_NEGOT-1: The intent driver MnS producer should support a capability enabling an MnS consumer to provide an intent with a request for the MnS producer to provide information on the alternative fulfillable outcomes for an intent.

NOTE 2:	An alternative is the combination of a set of expectation target values that the MnS producer achieves together with their (expected) impacts.

NOTE 3:	The impacts refer to information about the changes and outcomes on the expectation objects of the intent and on other related intents from the same intent MnS consumer. The impact refers to outcomes of the contexts and targets that MnS consumers have not explicitly pointed out. The exact characterization of what is reported is For Further Study. E.g. for an expectation target on energy consumption, the impact may include coverage area as a result of the energy consumption.

NOTE 4:	Example changes could include: omitting certain intentExpectations and/or expectationTarget(s) or changing the properties of intentExpectations and/or expectationTarget(s).

REQ_INT_NEGOT-2: The intent driven MnS producer should support a capability to provide to an MnS consumer an intent report indicating the alternatives that the MnS producer supports for the provided intent, intent expectations, or expectation Targets and the expected relative impacts of the different alternatives.

REQ_INT_NEGOT-3: The intent driven MnS producer should support a capability to request an MnS consumer to indicate its preference among a set of alternatives that the MnS producer supports for the provided intent, intent expectations, or expectation Targets.

REQ_INT_NEGOT-4: The intent driven MnS producer should support a capability enabling an MnS consumer to provide to the MnS producer information indicating the MnS consumer's preference among alternatives that the MnS producer supports for the provided intent, intent expectations, or expectation Targets.

REQ_INT_NEGOT-5: The intent driven MnS producer should support a capability enabling an authorized MnS consumer to provide information on a policy (e.g. in form of a utility function) that should be used by the be used by the MnS producer to select among the alternatives available at the MnS producer.

REQ_INT_NEGOT-6: The intent driven MnS producer should support a capability to request the MS consumer to provide an evaluation of the MnS producer's alternatives based on the expected relative impacts of the different alternatives.

REQ_INT_NEGOT-7: The intent driven MnS producer should support a capability enabling an MnS consumer to provide an evaluation of the MnS producer's alternatives to then be used to select among the alternatives.

REQ_INT_NEGOT-8: The intent driven MnS producer should support a capability to inform an authorized MnS consumer that an alternative among multiple alternatives has been selected and (will be/has been) applied for the submitted intent.

REQ_INT_NEGOT-9: The intent driven MnS producer should support a capability to inform an authorized MnS consumer that since no more improvement to intent fulfillment shall be possible the MnS consumer should evaluate the extent to which the applied alternative satisfies the MnS consumer's objectives or provide extra information which helps improve satisfaction.

REQ_INT_NEGOT-10: The intent driven MnS producer should support a capability enabling an authorized consumer to inform the producer that the alternative selected by the MnS producer was not satisfactory and another alternative should be applied.

REQ_INT_NEGOT-11: The intent driven MnS producer should support a capability enabling an authorized MnS consumer to provide information on the level of fulfillment which the MnS producer uses to differently attempt the fulfillment.

REQ_INT_NEGOT-12 (already supported): The intent driven MnS producer should support a capability enabling an authorized MnS consumer to revise the properties of an intent as the means to improve the chances of the intent being fulfillable.

INT_NEGOT_REQ 13: The MnS producer should support a capability to provide an intent report including information on what is achievable for each intent aspect (intentExpectation and expectationTarget) within that intent and the relative cost/impact of achieving that outcome.

INT_NEGOT_REQ 14: The MnS producer should support a capability enabling an MnS consumer to provide an intent with a request for the MnS producer to provide the best possible outcome on an intent or intent expectation or expectation target.

INT_NEGOT_REQ 15: The MnS producer should support a capability to provide an intent report including information on the best possible outcome on intent or intent expectation or expectation target.

NOTE 6: This requirement could be combined with REQ-13.

INT_NEGOT_REQ-16: The MnS producer should support a capability enabling an MnS consumer to provide an intent with a request for the MnS producer to provide information on what changes could be made to the intent properties or to properties of other of their intents to make the intent fulfillable.

NOTE 7: Further discussion is needed for this requirement.

INT_NEGOT_REQ-17: The MnS producer may support a capability to provide a report indicating the changes, which if applied to the intent, would make the intent fulfillable

Note: The definition of outcomes need further study.

2nd Change

6.2.1.1	Class diagram

6.2.1.1.1	Relationship

NOTE:	Void

Figure 6.2.1.1.1-1: Relationship UML diagram for intent driven management

Figure 6.2.1.1.1-2: Relationship UML diagram for intent

Figure 6.2.1.1.1-3: Relationship UML diagram for intent report

6.2.1.1.2	Inheritance

Figure 6.2.1.1.2-1: Inheritance UML diagram for intent driven management

3rd Change

6.2.1.2	Class definition

6.2.1.2.1	Intent <<InformationObjectClass>>

6.2.1.2.1.1	Definition

This IOC represents the properties of an Intent driven management information between MnS consumer and MnS producer.

The Intent IOC contains one or multiple IntentExpectation(s) which includes MnS consumer's requirements, goals and contexts given to a 3GPP system.

The Intent IOC also contains intentAdminState to support intent suspension mechanism. In case MnS consumer wants to suspend an intent, MnS consumer can request MnS producer to configure attribute intentAdminState with the value "DEACTIVATED". A suspended intent means this intent is not considered for fulfillment. In case MnS consumer wants to resume an intent on the MnS producer side when the intent is suspended, MnS consumer can request MnS producer to configure attribute intentAdminState with the value "ACTIVATED".

The attribute "observationPeriod" indicates the time period for which the fulfilment process is observed and at the end of which the fulfilmentInfo for corresponding ExpectationTargets, IntentExpectations and Intent is updated. The observation period can be set by the MnS consumer or by the MnS producer if the MnS consumer does not provide a value.

The Intent IOC includes the attribute objectClass and objectInstance from the TOP IOC. The value of attribute objectClass is "Intent" and the value of attribute objectInstance is the DN of the instance of Intent IOC.

The Intent IOC includes contextSelectivity respectively used to define how to select among the stated intentContexts

6.2.1.2.1.2	Attributes

The Intent IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [6]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentExpectations | M | T | T | F | F |
| userLabel | M | T | T | F | F |
| contextSelectivity | O | T | T | F | F |
| intentContexts | O | T | T | F | F |
| observationPeriod | O | T | T | F | F |
| intentPriority | O | T | T | F | T |
| intentAdminState | CM | T | T | F | F |
| intentPreemptionCapability | CM | T | T | F | F |
| consumerImplicitInfoIndex | O | T | T | F | F |
| Attribute related roles |  |  |  |  |  |
| intentReportReference | M | T | F | F | F |

6.2.1.2.2	IntentReport <<InformationObjectClass>

6.2.1.2.2.1	Definition

This IOC represents intent report information from MnS producer to MnS consumer. The IntentReport instance is created by MnS producer automatically when creating an Intent instance. When the MnS producer delete an intent instance based on a request from MnS consumer, the corresponding intent report instance is also deleted by MnS producer automatically. MnS consumers cannot request MnS producer to create or delete IntentReport instance.

The IntentReport IOC includes

- intentFulfilmentReport, which represents the properties of fulfillment information for expectation target, intent expectation, and the whole intent. The fulfilmentReport will be observed from the start of each observation period (specified in Intent IOC), then at the end of each observation period, the corresponding values will be derived and configured.

- intentConflictReport, which represents detected conflict information, including conflict type (i.e., intent conflict, expectation conflict and target conflict) and possible solution recommendations to address the conflicts.

- intentFeasibilityCheckReport, which indicates that the intent is feasible or infeasible. Intent feasibility check information is provided after MnS producer automatically performs feasibility check when receiving the intent creation and modification request from MnS consumer.

- intentNegotiationReport, which represents the properties of intent negotiation information. Intent negotiation information is provided when MnS Consumer requests or MnS producer automatically performs intent negotiation.

Each instance of IntentReport IOC can contain one or any combination of intentFulfilmentReport, intentConflictReport, intentFeasibilityCheckReport and intentNegotiationReport.

Different MnS consumers can use the "getMOIAttributes" operation to query different attributes of the IntentReport <<IOC>> to obtain corresponding intent report information (including intentFulfilmentReport, intentConflictReport, intentFeasibilityCheckReport and intentNegotiationReport).

Different MnS consumers can subscribe attribute value change notifications for IntentReport <<IOC>> to obtain the notification for different intent report information.

6.2.1.2.2.2	Attributes

The IntentReport <<IOC>> includes attributes inherited from Top IOC (defined in TS 28.622 [6]) and the following attributes

Table 6.2.1.2.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentFulfilmentReport | CM | T | F | F | T |
| intentConflictReports | CM | T | F | F | T |
| intentFeasibilityCheckReport | CM | T | F | F | T |
| intentNegotiationReport | CM | T | F | F | T |
| lastUpdatedTime | M | T | F | F | T |
| Attribute related to roles |  |  |  |  |  |
| intentReference | M | T | F | F | F |

6.2.1.2.2.3	Attribute constraints

Table 6.2.1.2.2.3-1

| Name | Definition |
| --- | --- |
| intentFulfilmentReport | Condition: intent fulfilment information is supported by IntentReport |
| intentConflictReports | Condition: intent conflict information is supported by IntentReport |
| intentFeasibilityCheckReport | Condition: intent feasibility check information is supported by IntentReport |
| intentNegotiationReport | Condition: intent negotiation is supported by the Intent-driven Mns producer |

6.2.1.2.2.4	Notifications

The common notifications defined in clause 6.2.1.5 are valid for this IOC, without exceptions or additions

6.2.1.3.X	IntentNegotiationReport <<dataType>>

6.2.1.3.X.1	Definition

This <<dataType>> represents the MnS producer's information to the MnS consumer regarding intent negotiations. It may contain information on:

1) possibleOutcomeList, a list of which (among the desired outcomes in an intent) the MnS producer is able to fulfil. It is an ordered list where the top entry indicates the best outcomes according to information available at the MnS producer. For each possibleOutcome, the fulfillableOutcomesReport includes information on the impact on the related ExpectationObjects.If used to indicate the best possible outcome, it will contain only 1 entry.

2) fulfillableTargets, indicating the list of intent expectations and expectation targets that the MnS producer is able to fulfil according to the constraints at  the MnS producer. It includes information on the impact on the related ExpectationObjects.

3) supportedAlternativesReport, which is the list of alternatives available at the MnS producer with which the intent may be fulfilled. Providing such a list inherently asks the MnS consumer to provide information on the MnS consumer’s preference among the alternatives. For each alternative, the SupportedAlternativesReport includes information on the impact on the related ExpectationObjects.

4) intentNegotiationConsumerFeedback, which is written by MnS consumer when providing feedback on a specific intent negotiation report.

5) possibleImpact, which represent the possible impact of the outcome or supported alternatives. The impact refers to outcomes of the context and targets that MnS consumers have not explicitly pointed out.

6.2.1.3.X.2	Attributes

Table 6.2.1.3.X.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| possibleOutcomeList | O | T | T | F | T |
| fulfillableTargets | O | T | T | F | T |
| supportedAlternativesReport | O | T | T | F | T |
| intentNegotiationConsumerFeedback | O | T | T | T | T |
| possibleImpact | O | T | T | T | T |

6.2.1.3.X.3	Attribute constrains

None

4th Change

6.2.1.3.y	PossibleImpact <<dataType>>

6.2.1.3.y.1	Definition

PossibleImpact <<dataType>> indicates the possible impact of the possible outcome. The impact refers to outcomes of the context and targets that MnS consumers have not explicitly pointed out. For example, the expectation of reducing energy consumption may have an impact on the coverage area of the cell. If the coverage area is not explicitly mentioned in the intent but is part of the concerns of MnS consumers, it is necessary to inform MnS consumers about this potential impact. The impact can be regarded as part of the implicit intent, which includes implicit intentExpectations and implicit intentContexts.

6.2.1.3.y.2	Attributes

The PossibleImpact includes the following attributes.

Table 6.2.1.3.6.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| implicitIntentExpectations | O | T | F | F | T |
| implicitintentContexts | O | T | F | F | T |

6.2.1.3.y.3	Attribute constraints

None.

5th Change

6.2.1.4	Attribute definition

Table 6.2.1.4-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentExpectations | It describes the expectations including requirements, goals and contexts (including constraints and filter information) given to a 3GPP system. It states the list of specific outcomes desired to be realized for expectation object(s). The intentExpectations are arranged in an ordered list such that the most important intentExpectations are on the top of the list. allowedValues: Not Applicable | type: IntentExpectation multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| intentFulfilmentInfo | It describes status of fulfilment of an intent and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationFulfilmentInfo | It describes status of fulfilment of an intentExpectation and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetFulfilmentInfo | It describes status of fulfilment of an expectationTarget and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fulfilmentStatus | It describes the current status of the fulfilment result for intent, intentExpectation or expectationTarget, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", "NOT_FULFILLED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "NOT_FULFILLED" isNullable: False |
| notFulfilledState | It describes the current state for not achieving fulfilment for the intent, intentExpectation or expectationTarget. It is configured/written by MnS producer and can be read by MnS consumer. allowedValues: "ACKNOWLEDGED", "COMPLIANT", "DEGRADED", "SUSPENDED", "TERMINATED" "FULFILMENTFAILED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ACKNOWLEDGED" isNullable: False |
| notFulfilledReasons | It describes the reasons/observations related to the specific notFulfilledState allowedValues: Not Applicable | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentContexts | It describes the list of IntentContext(s) which represents the constraints and conditions that should apply for the entire intent even if there may be specific contexts defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationId | A unique identifier of the intentExpectation within the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationVerb | It describes the characteristic of the intentExpectation and is the property that describes the types of intentExpectations. Examples of verbs and their related types of expectation are Deliver: DeliveryIntentExpectation, e.g. Deliver a RAN network, Service, Slice, function Ensure: AssuranceintentExpectation, e.g. Ensure the target performance value. allowedValues: DELIVER, ENSURE Vendor extensions are allowed | type: String multiplicity: 1 isOrdered:N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationObject | It describes the expectation objects to which the IntentExpectation should apply. allowedValues: Not Applicable | type: ExpectationObject multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectType | It describes the type of expectation object of the IntentExpectation that is required to be applied to. It can be class name of the managed object. allowedValues: see scenario specific IntentExpectation | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstance | It describes a specific object instance (e.g. instance of managed object) to which the intentExpectation should apply. allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectContexts | It describes the list of ObjectContext(s) which represents the constraints and conditions to be used as filter information to identify the object(s) to which a given intentExpectation should apply. Note there may be other constraints and conditions defined either for the entire intent, for the specific intentExpectation or for the expectationTarget of the considered intentExpectation. The concrete ObjectContext depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ObjectContexts follow the common structure of ObjectContext. | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationTargets | It describes the list of ExpectationTarget(s) which represent specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. The concrete ExpectationTarget depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ExpectationTargets follow the common structure of ExpectationTarget. The expectionTargets are arranged in an ordered list such that the most important expectionTargets are on the top of the list. | type: ExpectationTarget multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| expectationContexts | It describes the list of context(s) which represents the constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Expectation Object in the IntentExpectation | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetName | It describes the name of the expectation target which represents specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. allowedValues: depends on ExpectationObject in the IntentExpectation | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| targetCondition | It expresses the limits within which the targetName is allowed/supposed to be. allowedValues: "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE", "IS_ONE_OF", " IS_EQUAL_TO_OR_LESS_THAN”, "IS_EQUAL_TO_OR_GREATER_THAN", "IS_NOT_ONE_OF", "IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "IS_EQUAL_TO" isNullable: False |
| targetValueRange | It describes the range of values that applicable to the targetName and the targetCondition. allowedValues: depends on the targetCondition. The value will be a single value when the targetCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS EQUAL TO OR LESS THAN", "IS EQUAL TO OR GREATER THAN” The value will be a pair of values when the targetCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the targetCondition is "IS_ONE_OF", "IS_NOT_ONE_OF","IS_ALL_OF". See NOTE 1. | type: ValueRangeType multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| targetContexts | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: Not Applicable | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| contextAttribute | It describes a specific attribute of or related to the object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric, etc) to which the expectation should apply or an attribute related to the operating conditions of the object (such as weather conditions, load conditions, etc). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE, "IS_ONE_OF", "IS_EQUAL_TO_OR LESS_THAN", "IS_EQUAL_TO_OR_GREATER_THAN", "IS_NOT_ONE_OF", "IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "IS_EQUAL_TO" isNullable: False |
| contextValueRange | It describes the range of values that applicable to the ContextAttribute and the ContextCondition. AllowedValue: depends on the contextCondition The value will be a single value when the contextCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_EQUAL_TO_OR_LESS_THAN", "IS_EQUAL_TO_OR_GREATER_THAN". The value will be a pair of values when the contextCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the contextCondition is "IS_ONE_OF", "IS_NOT_ONE_OF","IS_ALL_OF". See NOTE 1. | type: ValueRangeType multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| intentPriority | It expresses the priority of the stated intent within an MnS consumer. AllowedValue: values in the range [1-100] where 1 indicates the highest priority and 100 indicates the lowest priority. NOTE: The handing of the priorities across MnS consumers is left to implementation | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 1 isNullable: False |
| geoArea | It describes a geographical area defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: GeoArea multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| pLMNId | It describes the information of a PLMN identification defined in 3GPP 28.658[10] AllowedValue: As defined by the data type | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dateTime | It describes the information of a date time defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| timeWindow | It describes the information of a time window (including startTime, endTime) defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| geoCoordinate | It describes the information of a geoCoordinate defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: GeoCoordinate multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| frequency | It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number) and/or the frequency operating band used for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD. AllowedValue: As defined by the data type | type: Frequency multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| arfcn | It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number). Allowed Value: For NR, see TS 38.104 [8] clause 5.4.2.1. For EUTRAN, see TS 36.104 [14] clause 5.7.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| freqband | It desribes the the frequency operating band. Allowed Value: For NR, see TS 38.104 [8] clause 5.4.2.3. For EUTRAN, see TS 36.104 [14] clause 5.7.3. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uEGroup | It describes the information of a UE Group (represented by specific 5QI, specific S-NSSAI, or a specific combination of S-NSSAI and 5QI). AllowedValue: As defined by the data type | type: UEGroup multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| fiveQI | It describes the information of a 5QI defined in 3GPP TS 28.541[5]. AllowedValue: 0 - 255 | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sNSSAI | It describes the information of a S-NSSAI defined in 3GPP TS 28.541[5]. AllowedValue: As defined by the data type | type: S-NSSAI multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intentAdminState | It describes the intent administrative state, which enables the MnS consumer to suspend an intent or cancel the suspension for a suspended intent. A suspended intent means this intent is not considered for fulfilment allowedValues: "ACTIVATED", "DEACTIVATED" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ACTIVATED" isNullable: False |
| intentReference | It indicates the associated intent instance allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentReportReference | It indicates the associated intent report instance(s) allowedValues: Not Applicable | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| observationPeriod | It represents the observation period of the fulfilmentInfo for corresponding ExpectationTargets, IntentExpectations and Intent. At the end of the observation period, the corresponding fulfilment info is updated in the intent report. The observation period can be assigned by MnS consumer through requesting the MnS producer to set attribute "observationPeriod". MnS producer also can assign the observation period if MnS consumer didn’t assign it. The observation time is expressed in seconds. allowedValues: Not Applicable | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentFulfilmentReport | It describes the fulfillment information which is reported for the associated intent instance. allowedValues: Not Applicable | type: IntentFulfilmentReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentConflictReports | It describes the conflict information which is reported for associated intent instance if needed. allowedValues: Not Applicable | type: IntentConflictReport multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| conflictId | It is used to identify the detected conflict within an IntentReport instance. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictType | It describes the type of intent conflict. allowedValues: INTENT_CONFLICT, EXPECTATION_CONFLICT, TARGET_CONFLICT | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingIntent | It describes the DN of the conflicting intent allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingExpectation | It describes the expectationId of the conflicting IntentExpectation within an Intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingTarget | It describes the targetName of the conflicting ExpectationTarget within an IntentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedSolutions | It describes the action recommended by the MnS producer to be undertaken by the MnS consumer to resolve intent conflict. The recommended solution applies only for the specific intent whose intent report contains this attribute. allowedValues: "MODIFY", "DELETE" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationFulfilmentResults | It includes the expectationFulfilmentInfo and targetFulfilmentResults for each IntentExpectation. The expectationFulfilmentInfo describes status of fulfilment of an intentExpectation and the related reasons for infeasible status. allowedValues: Not Applicable | type: ExpectationFulfilmentResult multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetFulfilmentResults | It includes targetFulfilmentInfo and targetAchievedValue for each ExpectationTarget. The targetFulfilmentInfo describes status of fulfilment of an expectationTarget and the related reasons for infeasible status. The targetAchieveValue describes current performance value for the ExpectationTarget. allowedValues: Not Applicable | type: TargetFulfilmentResult multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetAchievedValue | It describes the value that has been achieved for the expectation target at the time at which the report is generated. allowedValues: Not Applicable | type: Number multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentFeasibilityCheckReport | It describes the intent feasibility check information which is reported if needed. allowedValues: Not Applicable | type: IntentFeasibilityCheckReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| feasibilityCheckResult | It describes the result of intent fulfilment feasibility check allowedValues: FEASIBLE, INFEASIBLE | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| infeasibilityReasons | It describes the reason (e.g. invalid intent expression, the intent conflict) of the result of intent fulfilment feasibility check is INFEASIBLE | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentHandlingCapabilityList | It describes the list of expectation object information and expectation target information which can be supported by intent handling function. allowedValues: Not Applicable | type: IntentHandlingCapability multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentHandlingCapabilityId | A unique identifier of property of intent handling capability should be supported by the intent handling function of MnS producer. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedExpectationObjectType | It describes the expectation object type which can be supported by a specific intent handling function of MnS producer. allowedValues: objectType defined in clause 6.2.1.3.2. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedExpectationTargetNames | It describes the supported expectation targets for the supported expectation object type. allowedValues: targetName defined in clause 6.2.1.3.3 | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| lastUpdatedTime | It describes the time for the latest update of the IntentReport Instance. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ContextSelectivity | It expresses the may in which all or a subset of the expectationTargets may be applied. AllowedValue: "ALL_OF", "ONE_OF", "ANY_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ALL_OF" isNullable: False |
| intentPreemptionCapability | It describes the pre-emption capability. The attribute is used by MnS producer to decide the target of intent deletion or intent modification allowedValue: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |
| consumerImplicitInfoIndex | It indicates that the MnS consumer needs impact information for the outcomes from the MnS producer. The impact refers to outcomes of the context and targets that MnS consumers have not explicitly pointed out. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |
| implicitIntentExpectations | It describes the implicit intent expectation of the Mns consumer pointed out by the Mns producer. | type: IntentExpectation multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| implicitintentContexts | It describes the list of implicit IntentContext(s) which represents the constraints and conditions that should apply for the entire implicit intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE: For "IS_ALL_OF", the value shall be a match of the entire list. |  |  |

6th Change

A.2.3	Relationship UML diagram for intentReport IOC (figure 6.2.1.1.1-3)

@startuml

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

skinparam  NoteBackgroundColor White

class "<<InformationObjectClass>>\n IntentReport" as IntentReport{}

class "<<dataType>>\n IntentFulfilmentReport" as IntentFulfilmentReport{}

class "<<dataType>>\n IntentConflictReport" as IntentConflictReport{}

class "<<dataType>>\n IntentFeasibilityCheckReport" as IntentFeasibilityCheckReport{}

class "<<dataType>>\n IntentNegotiationReport" as IntentNegotiationReport{}

class "<<dataType>>\n ExpectationFulfilmentResult" as ExpectationFulfilmentResult{}

class "<<dataType>>\n TargetFulfilmentResult" as TargetFulfilmentResult{}

IntentReport "1" -- "1" IntentFulfilmentReport

IntentReport "1" -- "*" IntentConflictReport

IntentReport "1" -- "1" IntentFeasibilityCheckReport

IntentReport "1" -- "*" IntentNegotiationReport

IntentFulfilmentReport "1" -- "1..*" ExpectationFulfilmentResult

ExpectationFulfilmentResult "1" -- "1..*" TargetFulfilmentResult

@enduml

End of changes