3GPP TSG-SA5 Meeting #159	S5-250498

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	China Mobile

# Title:	Rel-19 pCR 28.851 Adding solution on Support of Avatar communication

# Document for:	Approval

Agenda Item:	7.5.3

1	Decision/action requested

This is a pCR to add solution on support of Avatar communication in TR 28.851.

2	References

[1]	3GPP TR 28.851: "Study on charging aspects of next generation real time communication services phase 2".

3	Rationale

This pCR proposes to add solution on support of Avatar communication in TR 28.851 [1].

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.851.

First change

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

API	Application Programming Interface

AS	Application Server

A2P	Application to Person

DC	Data Channel

DCSF	Data Channel Signalling Function

NEF	Network Exposure Function

P2A	Person to Application

P2P	Person to Person

Second change

5.5.4	Possible solutions

5.5.4.1	Solution #1: IMS AS based charging solution for Avatar communication

5.5.4.1.1	General

Solution#1 addresses Key Issue #1a and #1b for REQ-NG_RTC_CH_NCIAC-01. The solution is based on the IMS AS, which collects and reports specific charging information for Avatar communication.

5.5.4.1.2	Description

- In TS 23.228 [5] Figure AC.11.3.3-1, UE1 requests the network to perform avatar animation. If the avatar animation negotiation between the DC AS and UE1 is successful, UE1 initiates new P2A2P application data channels for avatar data transmission. During the P2A2P channel establishment, the IMS AS notifies the DCSF of the media change request. The DCSF checks if the request is for Avatar communication, retrieves the DC AS URL, fetches the Avatar Media Specification data from the DC AS using this URL, and instructs the MF on establishing the data channel and the corresponding Avatar Media Specification via the IMS AS. When UE1 initiates avatar animation negotiation in Step 4, it sends the Avatar ID to the DC AS through the ADC channel. From Step 8 to 9, the DC AS returns the Avatar Media Specification (including the Avatar ID) to the DCSF. In Step 10, the DCSF uses Nimsas_MediaControl_MediaInstruction to pass the Avatar Media Specification to the IMS AS, which can then obtain the Avatar ID and determine the Avatar Media Type.

Based on the above procedures defined in TS 23.228 [5] Annex AC.11, IMS AS can fetch the Avatar ID and Avatar communication media type from DCSF via Nimsas_MediaControl_MediaInstruction. In this case, the support of the IMS AS for the collection and reporting of charging information related to the Avatar ID and Avatar communication media type in TS 32.260 [11] can be extended.

5.5.5	Evaluation

Solution #1 addresses Key Issue #1a and #1b proposing a converged charging method for network centric IMS Avatar communication. This requires IMS AS to collect and report specific charging information for Avatar communication to CHF.

5.5.6	Conclusion

Based on the evaluation in clause 5.5.1, Solution #1 is the only solution for network centric IMS Avatar communication. The Solution #1 is selected to the normative work.

End of changes