3GPP TSG SA WG5 Meeting #159 	S5-250502

Sophia-Antipolis, FRANCE 17 - 21 February 2025

# Source:	CATT

# Title:	Add TDF based solution for S&F operation charging with User Plane CIoT

# Document for:	Approval

# Agenda Item:	7.5.1

1	Decision

The group is asked to discuss and agree on the proposal.

2	References

[1]		3GPP TR 28.846: " Study on charging aspects of satellite access Phase 3 ".

3	Rationale

According to the conclusion of the SA2 SAT_Ph3_ARCH, the feature of S&F operation only supports the EPC in the Rel-19. Only delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS) have been supported in the Rel-19.

This pCR proposes to add S&F operation charging solutions for User Plane CIoT.

As specified in the TS 23.401, only the whole EPC on-board architecture support using the User Plane CIoT EPS optimisation to transfer data. Moreover, the UE using satellite access with S&F operation indication in the initial attach or the service request procedure can only be known by MME. The P-GW can not get the information about the S&F. In addition, the EPS may expose whether a UE is in S&F Mode and provide to the SCS/AS related timing information to guide the SCS/AS decision when to try to contact the UE.

Accroding to the TS23.401, TS 23.203 and TS32.251, the S&F information may be transferred from the AS to TDF via PCRF, therefore, this solution proposes to use TDF to trigger to charge the S&F operation of the User Plane CIoT service.

4	Detailed proposal

The following changes are proposed to be incorporated into the new TR.

1st Modified Section

6.1.4.x	Solution #1.x: TDF Charging Trigger Function (CTF) based solution for S&F operation charging with User Plane CIoT

This solution which relying on EPC offline Charging System for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As specified in the TS 23.401[5], only the whole EPC on-board architecture support using the User Plane CIoT EPS optimisation to transfer data. The EPS may expose whether a UE is in S&F Mode and provide to the SCS/AS related timing information to guide the SCS/AS decision when to try to contact the UE. The S&F information may be transferred from the AS to TDF via PCRF.

For the whole EPC onboard architecture, the CDF/CGF may be deployed on the satellite. The high level of the whole EPC on-board architecture is shown below:

Figure 6.1.4.x-1: Whole EPC on-board charging architecture

The TDF onboard reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following chargeable events:

-	PDN connection creation/release via the AS onboard for the User Plane data transfer.

Table 6.1.4.x-1 shows the enhancements needed for TDF-CDR compared with the Table 6.1.14.1 in TS 32.251 [7].

Table 6.1.4.X-1 Extension to TDF-CDR for S&F (3GPP TS 32.251 [7] – Table 6.1.14.1)

| Field | Category | Description |
| --- | --- | --- |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the UE, extended with RAT types for satellite. |
| Satellite information | OC | This field holds all the satellite information that support to finish the store and forward satellite operation. |
| Satellite Access Indicator | OC | This field indicates the use of satellite access. |
| S&F Indicator | OC | This field indicates the use of store and forward satellite operation. |
| S&F Monitoring List | OC | This field holds the satellite IDs that used for the store and forward satellite operation. |
| S&F Duration | OC | This field holds the storage duration of data on the satellite. |
| S&F Data volume | OC | This field holds the data volume stored on the satellite. |

End of Modified Sections