3GPP TSG-SA5 Meeting #159	S5-250529

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Ericsson

# Title:	Use of energy efficiency and consumption information

# Document for:	Discussion

Agenda Item:	7.4.3

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TS 23.501 	System architecture for the 5G System (5GS)

[2]	3GPP TS 28.201 	Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2

[3]	3GPP TS 28.541 Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

[4]	3GPP TS 28.552 Management and orchestration; 5G performance measurements

[5]	3GPP TS 28.554 Management and orchestration; 5G end to end Key Performance Indicators (KPI)

[6]	3GPP TS 32.255 Charging management; 5G data connectivity domain charging; Stage 2

[7]	3GPP TS 32.291 Charging management; 5G system, charging service; Stage 3

[8]	3GPP TR 28.880 Study on energy efficiency and energy saving aspects of 5G networks and services

[9]		ETSI ES 202 336-12 V1.2.1 Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model

[10]	ETSI GS NFV-IFA 027 V4.0.2 (2020-11) Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification

# 3	Rationale

## 3.1	Background

### 3.1.1	Network slice and PDU sessions

The current TS 28.201 [2] covers performance and analytics data for network slices and uses information from TS 28.541 [3] and TS 28.554 [5]. In TS 28.554 [5] network slice KPIs for number of PDU sessions and number of registered subscribers are specified, and TS 23.501 [1] clause 5.15.3 states:

“The Subscription Information shall contain one or more S-NSSAIs i.e. Subscribed S-NSSAIs. The subscription information shall include at least one default S-NSSAI. The UDM sends at the most 16 Subscribed S-NSSAIs to AMF, i.e. the number that can fit in a Configured NSSAI. The subscription information the UDM sends to the AMF shall include at least one default S-NSSAI.

If an S-NSSAI is marked as default, then the network is expected to serve the UE with a related applicable Network Slice instance when the UE does not send any permitted S-NSSAI to the network in a Registration Request message as part of the Requested NSSAI.

The Subscription Information for each S-NSSAI may contain:

-	a Subscribed DNN list and one default DNN; and

-	the indication whether the S-NSSAI is marked as default Subscribed S-NSSAI; and

-	the indication whether the S-NSSAI is subject to Network Slice-Specific Authentication and Authorization; and

-	Network Slice Simultaneous Usage Group (NSSRG) information (see clause 5.15.12).

The network verifies the Requested NSSAI the UE provides in the Registration Request against the Subscription Information. For the S-NSSAIs subject to Network Slice-Specific Authentication and Authorization the clause 5.15.10 applies.”

TS 23.501 [1] clause 5.15.3 states:

“In roaming case, the UDM shall provide to the VPLMN only the S-NSSAIs from the Subscribed S-NSSAIs the HPLMN allows for the UE in the VPLMN. If the UE is subject to restrictions of simultaneous registration of network slices (i.e. if the Subscription Information for the S-NSSAIs contains NSSRG information), the UDM provides to the VPLMN a subscribed S-NSSAIs and, if applicable, NSSRG information, as described in clause 5.15.12.”

TS 23.501 [1] clause 5.16.4.1 states:

“To provide Emergency Services, the AMF is configured with Emergency Configuration Data that are applied to Emergency Services that are established by an AMF based on request from the UE. The AMF Emergency Configuration Data contains the S-NSSAI and Emergency DNN which is used to derive an SMF. In addition, the AMF Emergency Configuration Data contains UE-AMBR and may also contain the statically configured SMF for the Emergency DNN. The SMF may also store Emergency Configuration Data that contains statically configured UPF information for the Emergency DNN.”

and TS 23.501 [1] clause 5.30.2.10.2 states:

“The AMF supporting UE onboarding is configured with AMF Onboarding Configuration Data that includes e.g.:

-	S-NSSAI and DNN to be used for onboarding or a configured SMF for the S-NSSAI and DNN used for onboarding.

-	Information to use a local AUSF(s) within the ON-SNPN for onboarding of UEs with a SUCI for a DCS with AAA Server or for onboarding of UEs in the case where the DCS is not involved during primary authentication.

NOTE 2:	The S-NSSAI used for onboarding is assumed to be configured in both the AMF (i.e. in the AMF Onboarding Configuration Data) and the NG-RAN nodes for the corresponding Tracking Areas where onboarding is enabled.”

### 3.1.2	Energy consumption and efficiency

For a specific NF the energy consumption is described in clause 6.7.3.1.1 “). This KPI is obtained by summing up the energy consumption of PNF(s) and/or VNF(s) which compose the NF.”, and if the NF is a VNF then it’s described in clause 6.7.3.1.2 “This KPI is obtained by summing up the estimated energy consumption of its constituent Virtualized Network Function Components (VNFC).”, where the VNFC is calculated in clause 6.7.3.1.3 as:

“the energy consumption of the virtual compute resource instance is estimated based on either:

- its mean vCPU usage, as per clause 6.7.3.1.4. The method for calculating ECVNFC,estimated is described in TS 28.310 [9] clause 6.3.2.2.1, or

- its mean vMemory usage, as per clause 6.7.3.1.5, or

- its mean vDisk usage, as per clause 6.7.3.1.6, or

- its I/O traffic volume, as per clause 6.7.3.1.7.”

The calculation of a specific network slice energy consumption is described in TS 28.554 [5] clause 6.7.3.3:

“The definition of ECns based on the following principles:

- 	For all gNBs in the network slice, clause 5.1.1.19.3 (PNF Energy consumption) of TS 28.552 [6] applies. This measurement is obtained according to the method defined in ETSI ES 202 336-12 [10] – clauses 4.4.3.1, 4.4.3.4, Annex A;

- 	In case a 5GC NF is composed of Virtualized Network Functions (VNF) and/or Physical Network Functions (PNF), clause 6.7.3.1 of this document defines the NF Energy Consumption (EC);

-	 In case a NF is dedicated to a network slice, the energy consumption of the NF is entirely attributable to the network slice;

- 	In case a NF is shared between multiple network slices, the participation of the NF to the energy consumption of the network slice has to be estimated, as it can't be measured:

- 	In case of a gNB shared between multiple network slices, the energy consumption attributable to each network slice is estimated as a proportion of the total gNB energy consumption, where the proportion is calculated as the data volume of the network slice relatively to the total data volume carried by the gNB,

- 	In case of a AMF shared between multiple network slices, the energy consumption attributable to each network slice is estimated as a proportion of the total estimated AMF energy consumption, where the proportion is calculated as the mean number of registered subscribers of the network slice relatively to the overall mean number of registered subscribers of the AMF during the same time period (see TS 28.552 [6] clause 5.2.1.1 for the definition of the mean number of registered subscribers),

-	In case of a SMF shared between multiple network slices, the energy consumption attributable to each network slice is estimated as a proportion of the total estimated SMF energy consumption, where the proportion is calculated as the mean number of PDU sessions of the network slice relatively to the overall mean number of PDU sessions of the SMF during the same time period (see TS 28.552 [6] clause 5.3.1.1 for the definition of the mean number of PDU sessions),

-	In case of a UPF shared between multiple slices, the energy consumption attributable to each network slice is estimated as a proportion of the total estimated UPF energy consumption, where the proportion is calculated as the data volume of the network slice relatively to the overall data volume of the UPF during the same time period.

-	In case of a UPF with N3 interface(s), the data volume of the UPF is obtained by summing up, for all N3 interface(s), the number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF (see TS 28.552 [6] clause 5.4.1.3) and the number of octets of outgoing GTP data packets on the N3 interface, from UPF to (R)AN (see TS 28.552 [6] clause 5.4.1.4)

-	In case of a PSA UPF with no N3 interface(s), the data volume of the UPF is obtained by summing up, for all N9 interface(s), the number of octets of incoming GTP data packets on the N9 interface for PSA UPF (see TS 28.552 [6] clause 5.4.4.2.3) and the number of octets of outgoing GTP data packets on the N9 interface for PSA UPF (see TS 28.552 [6] clause 5.4.4.2.4)”

The current energy consumption and efficiency KPIs that are supported for TS 28.554 [5] are described in table 3.1.2-1 and 3.1-2.

Table 3.1.2-1: Energy consumption KPIs in TS 28.554

| Name | Title |
| --- | --- |
| ECRANonlyns | Energy consumption of the RAN-only network slice (J) |
| ECNF | Energy consumption of a physical or virtual NF, it’s the sum of all virtual and physical NF which compose the NF. For virtual it is based on either mean vCPU usage, mean vMemory usage, mean vDisk usage, or I/O traffic volume. (J) |
| EC5GC | Energy consumption of the 5G core network (J) |
| ECns | Energy consumption of a network slice. For network slices that share NF it is based on data volume for gNB and UPF, mean number of PDU sessions for SMF, and mean number of registered subscribers for AMF (J) |
| ECNG-RAN | Energy consumption of the NG-RAN (J) |
| ECgNB | Energy consumption of the gNB (J) |

Table 3.1.2-2: Energy efficiency KPIs in TS 28.554

| Name | Title |
| --- | --- |
| EEMN,DV | Energy efficiency for in operational NG-RAN, based on data volume (bit/J) |
| EEns | Energy efficiency for any type of network slice, based on any type of performance measurement (type of performance /J) |
| EEeMBB,DV | Energy efficiency for eMBB network slice, based on data transmission (bit/J) |
| EERANonlyeMBB,DV | Energy efficiency for RAN part of eMBB network slice, based on data transmission (bit/J) |
| EEURLLC,Latency | Energy efficiency for URLLC network slice, based on latency (0.1ms-1*J-1)) |
| EEURLLC,DV,Latency | Energy efficiency for URLLC network slice, based on both latency and data volume (bit/(0.1ms*J)) |
| EEMIoT,RegSubs | Energy efficiency for MIoT network slice, based on the maximum number of registered UEs (UEs/J) |
| EEMIoT,ActiveUEs | Energy efficiency for MIoT network slice, based on the mean number of active UEs (UEs/J) |
| EE5GC,UO,UP,DV | Energy Efficiency of 5GC based on the useful output of 5GC user plane (bit/J) |

In TR 28.880 [8] the KPIs are proposed to be enhanced with definitions of KPIs regarding the energy consumption of gNB and 5GC NFs attributable to each network slice served by the gNB and 5GC NFs.

### 3.1.3	Charging information

In TS 32.291 [7] it is possible to get information about:

-	when a PDU Sessions started and ended with additional information on the UE identity (SUPI), NF type (SMF), SMF instance, S-NSSAI, and location.

-	data volume per PDU Session with additional information on the UE identity (SUPI), NF type (UPF), UPF instance, and S-NSSAI.

-	when a registration message is sent with additional information about the UE identity (SUPI), registration message type, NF type (AMF), AMF instance, and requested/allowed/rejected NSSAI.

-	when an SMS is sent/received with additional information about the UE identity (SUPI), SM message type, NF type (SMSF), and SMSF instance.

## 3.2	Evaluation

The statement the TS 23.501 [1] means that a PDU session must belong to an S-NSSAI which then means that any measurements of energy consumption or efficiency reported on network slice level will be possible to use for calculating the energy consumption of an individual PDU session.

There are some risks with the current definitions of energy consumption and efficiency:

-	the ECns is based data volume for the UPF while the UPF on a VNF may be based on or I/O traffic volume, mean vCPU, mean vMemory, or mean vDisk, usage, it may even be a combination of them. This will be causing more uncertainty in the result since if the ECupf is based on vCPU while the ECns uses volume it would mean that you have a quota of the energy consumption based on first vCPU and then volume.

-	the EEns uses the ECns in the calculation where the ECns is based, for the UPF part, on data volume to calculate the share one network slice have i.e., the network slice data volume divided by the total data volume gives the percentage of the ECNF for the network slice. The network slice data volume is then again used to calculate the EEns, by dividing the network slice data volume with the ECns. This means that the EEns is independent of the network slice for a UPF. In the case where the UPF is a VNF and uses the I/O traffic volume to calculate the energy consumption it would be the energy efficiency of the VNF independent of NF and network slice.

Energy consumption for a VNF based on volume

ECVNF=I/OVolumeVNFVNFI/OVolumeVNF×ECVNFINode

Energy consumption for an NF based on volume

ECNF=VNFECVNF

Energy consumption for an NS based on volume

ECns=NFECNF

Energy consumption for the UPF part of an NS based on volume

ECns=I/OVolumensnsI/OVolumens×ECUPF

Energy efficiency

EEns=I/OVolumensECns

Since the total volume of all NS through a UPF is equal to the UPF part of the traffic through a VNFI node it means that:

EEns=I/OVolumensI/OVolumensnsI/OVolumens×ECUPF=nsI/OVolumensECUPF=nsI/OVolumensI/OVolumeVNFVNFI/OVolumeVNF×ECVNFINode=VNFI/OVolumeVNFECVNFINode

This means that the energy efficiency of the UPF part of a network slice is independent of the network slice and only dependent on the VNFs.

-	The EEns uses the ECns in the calculation where the ECns is based on different things depending on the NF type e.g., for UPF it’s data volume, AMF it’s mean number of registered subscribers, while the EEns may be based on something else (e.g. mean number of PDU sessions) independent on NF type. This means that if EEns,DV is used the AMF energy will then be based on the data volume with may lead to uncertainty in the result.

Since the EE only introduces more uncertainties in the measurements it would be preferred if the CCS (Converged Charging System) could get the EC instead, and in this case on an NF level, not necessarily split on a per network slice.

The CCS can calculate the number of PDU Sessions, number of registrations, data volume either per NF type, NF instance, S-NSSAI, and per UE. This means that in the CCS it is possible to calculate the number of started PDU session or mean number of concurrent PDU session in an SMF instance by gathering all records that contain the SMF instance. With all the records for a specific SMF instance within specified duration it could calculate the percentage a specific S-NSSAI could be seen as contributing to the EC, it could even calculate the percentage for a specific SUPI. Alternatively, the CCS could use counters that would count per SMF instance and S-NSSAI (and/or SUPI) the number of PDU sessions and by this have a up to date percentage that could then change over time. If the CCS then has the ECNF with its measurement duration this could then be used to estimate the portion of the EC that could be considered used by that specific S-SNSSAI and/or SUPI.

Similarly, could be done for UPF but here the volume would be used instead of the PDU sessions, to calculate the percentage, and for SMSF the number of sent/received SMS would be used.

For the AMF only the NSSAI is known and not the actual S-NSSAI this means that the AMF portion would in this case have to be measured for the SMF which also include the AMF so by using both AMF and SMF information it can be estimated how much a specific S-NSSAI can be considered to have been used in the AMF. Using the S-NSSAI of the selected S-NSSAI means that the other S-NSSAIs in the NSSAI will be seen as not impacting the EC of the AMF, however if there would have been only one S-NSSAI that can be used this would require less processing. This means that the S-NSSAI selected will have to take the EC of being selected.

For the gNB it is possible to get information about the or location of the UE from the SMF to be able to calculate also the S-NSSAI EC in the gNB but it will increase the load in the system and thereby also increase the EC.

Combination of all the information and calculations mean that the CCS can do estimates for energy consumption for a UE, group of UEs, and S-NSSAI.

## 3.3	Observations

Observation 1: To be able to estimate energy consumption it would be preferred for the CCS to get the EC KPIs instead of the EE KPIs

Observation 2: The EC KPIs should be an a per NF bases to provide the most flexibility for calculations in CCS.

Observation 3: It could be good to have an informative annex describing on how the EC KPIs can be used together with the charging information.

# 4	Detailed proposal

Proposal 1: For observation 3 we could extent the current work item to also include the informative annex on energy consumption estimation calculations.