3GPP TSG-SA5 Meeting #159	S5-250532

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Ericsson

# Title:	Correction and conclusion update for topic 1

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TR 28.846 Study on charging aspects of satellite access Phase 3

# 3	Rationale

The key issues for topic 1 have the wrong number, addition of requirements evaluation and update of conclusion.

# 4	Detailed proposal

First change

#### 6.1.3.1	Key issue #1.1: Charging events and charging information required

This key issue is for investigating how to support the S&F operation service charging considering REQ- CH_ SAT_PH3-01. This investigation covers the following:

-	identification of the triggers for charging events for S&F operation;

-	identification and classification of the charging information for S&F operation;

Second change

#### 6.1.3.2	Key issue #3.2: NF/Service suitable to provide charging information

This key issue is for investigating how to support the S&F operation charging considering REQ- CH_ SAT_PH3-01. This investigation covers the following:

-	determination of which NF in the 4G system are suitable to provide the charging information to support the S&F operation.

Third change

#### 6.1.4.4	Solution #1.4: MME CDF/CGF based solution for S&F operation of SMS service

This solution which relying on EPC CDF/CGF for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As described in the TR 23.700-29 [3], S&F Satellite operation is especially suited for the delivery of delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS) that have been supported in the Rel-19. As described in the TS 23.401 [5], the UE registers in S&F mode to access S&F-based services from E-UTRAN satellite access running in S&F mode. The attach procedure for S&F-based services may use one or more satellites, depending on the deployment and implementation options. On-board MME determines a list of satellites (i.e. S&F Monitoring list) from the same (UE selected) PLMN with which UE can attempt to use to finish the attach procedure.

For the MME split architecture, the CDF/CGF may be deployed on the ground. The MME ground together with the MME onboard behaves jointly as a single MME entity and the UE context is synchronized between them. Each MME-onboard is associated with a different Satellite ID identifier. The high level of the MME split architecture is shown below:

Figure 6.1.4.4-1: MME split charging architecture

The MME-ground reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following trigger events:

-	Short Message sent by a UE via the MME onboard (MO direction) to the SMSC.

-	Short Message received by a UE via the MME onboard (MT direction) from the SMSC.

For the whole EPC onboard architecture, the CDF/CGF may be deployed on the satellite. The high level of the whole EPC on-board architecture is shown below:

Figure 6.1.4.4-2: Whole EPC on-board charging architecture

The MME onboard reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following trigger events:

-	Short Message sent by a UE via the MME onboard (MO direction) to the SMSC.

-	Short Message received by a UE via the MME onboard (MT direction) from the SMSC.

Table 6.1.4.4-1 shows the enhancements needed for S-SMO-CDR and S-SMT-CDR compared with the Tables 6.1.5.1/6.1.6.1 in TS 32.251 [7].

Table 6.1.4.4-1 Extension to S-SMO-CDR/ S-SMT-CDR for S&F
 (3GPP TS 32.251 [7] – Tables 6.1.5.1/6.1.6.1)

| Field | Category | Description |
| --- | --- | --- |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the UE, extended with RAT types for satellite. |
| Satellite information | OC | This field holds all the satellite information that support to finish the store and forward satellite operation. |
| Satellite Access Indicator | OC | This field indicates the use of satellite access. |
| S&F Indicator | OC | This field indicates the use of store and forward satellite operation. |
| S&F Monitoring List | OC | This field holds the satellite IDs that used for the store and forward satellite operation. |
| S&F Duration | OC | This field holds the storage duration of data on the satellite. |
| S&F Data volume | OC | This field holds the data volume stored on the satellite. |

Fourth change

#### 6.1.5.1	Solutions evaluation for Key issue #1.1

Solutions #1.1, addresses Key issue #1.1 and potential requirement REQ-CH_ SAT_PH3-01, for the SMS service using the satellite access running in the S&F mode, with the enhancement of the new trigger events and charging information. The new trigger events are Short Message sent by a UE via the MME on-board (MO direction) to the SMSC and Short Message received by a UE via the MME on-board (MT direction) from the SMSC. The enhancement of the charging information including RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring list, Store duration and Store data volume.

Solutions #1.2, addresses Key issue #1.1 and potential requirement REQ-CH_ SAT_PH3-02, for the Control Plane CIoT using the satellite access running in the S&F mode, with the new trigger events and charging information. The new trigger events are Attach Complete via the MME onboard for the CIoT CP Optimizations and Service Request via the MME onboard for the CIoT CP Optimizations. The charging information including RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring list, Store duration and Store data volume.

Solutions #1.3 addresses Key issue #1.1 and potential requirement REQ-CH_ SAT_PH3-03, for the User Plane CIoT using the satellite access running in the S&F mode, with the new trigger events and charging information. The new trigger events are Attach Complete via the MME onboard for the CIoT UP Optimizations and Service Request via the MME onboard for the CIoT UP Optimizations. The charging information including RAT type, Satellite Access Indicator, Store and Forward indicator and S&F Monitoring lists.

Solutions #1.4 addresses Key issue #1.1 and potential requirement REQ-CH_ SAT_PH3-01, for the SMS service using the satellite access running in the S&F mode, with the enhancement of the new trigger events and charging information. The CDF/CGF may be deployed on the satellite. The new trigger events are Short Message sent by a UE via the MME on-board (MO direction) to the SMSC and Short Message received by a UE via the MME on-board (MT direction) from the SMSC. The enhancement of the charging information including RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring List, S&F Duration and S&F Data volume.

Solutions #1.5 addresses Key issue #1.1 and potential requirement REQ-CH_ SAT_PH3-02, for the Control Plane CIoT using the satellite access running in the S&F mode, with the new charging information. The CDF/CGF may be deployed on the satellite. The extension to CPDT-SNN-CDR for S&F charging include RAT type, Satellite Access Indicator, Store and Forward indicator, S&F Monitoring list, Store duration and Store data volume.

Solutions #1.6 addresses Key issue #1.1 and potential requirement REQ-CH_ SAT_PH3-02, for the Control Plane CIoT using the satellite access running in the S&F mode, with the new charging information. The CDF/CGF may be deployed on the satellite. There is no enhancement required for the SCEF CDRs needed.

Sixth change

#### 6.1.5.2	Solutions evaluation for Key issue #1.2

All address Key issue #1.2 and all solutions propose to have CDR generation in MME and except solution #1.6 which proposes SCEF. Solution #1.1 enhances the MME over SMS charging that MME provides the charging information. Solution #1.2, Solution#1.3, Solution#1.4 and Solution#1.5 directly uses the MME provides the charging information and provides the message flow. Solution #1.6 uses SCEF to report charging information to CGF or Billing Domain about satellite access running in S&F mode.

Seventh change

### 6.1.6	Conclusion

It is concluded that the solution #1.4 is the feasible solution for the SMS service using the satellite access running in the S&F mode. New trigger events and new parameters need to be added for the S&F operation charging.

It is concluded that the solution #1.x is the feasible solution for the UP CIoT service using the satellite access running in the S&F mode. New trigger events and new parameters need to be added for the S&F operation charging.

It is concluded that the solution #1.6 is the feasible solution for the CP CIoT service using the satellite access running in the S&F mode. No enhancement for the CDRs is needed.

En of changes