3GPP TSG-SA5 Meeting #159 	S5-240544

Sophia Antipolis, France, 17 - 11 February 2025

# Source:	Ericsson

# Title:	PM for Inter-CU LTM and conditional LTM

# Document for:	Discussion, Endorsement

Agenda Item:	6.4.27

# 1	Decision/action requested

SA5 to discuss and endorse this proposal.

# 2	References

[1]	S5-221400, Revised WID on Enhancement of Handover Optimisation

[2]	3GPP TS 28.554, Management and orchestration; 5G end to end Key Performance Indicators (KPI)

[3]	SP-231747, 5G performance measurements and KPIs phase 4

# 3	Rationale

The traditional layer 3 handover has in recent releases been supplemented with new handover mechanisms. In Rel-16, Conditional Handover (CHO) was introduced. During Rel-17, SA5 added management support for CHO. At that time, SA5 added requirements, use cases, and related NRM additions as well as PM measurements and an updated a KPI to its specifications, under the Work Item Revised WID on Enhancement of Handover Optimisation [1].

In Rel-18, the new handover mechanism L1/L2 Triggered Mobility (LTM) was added to NR. During 2024, for Rel-19, SA5 added management support for Intra-CU LTM. SA5 added requirements, use cases, related NRM additions as well as PM measurements and an updated KPI to its specifications, under the Work Item 5G performance measurements and KPIs phase 4 [3]. This work concluded during SA5#157.

In Rel-19, the Rel-18 specified Intra-CU LTM is being augmented with Inter-CU LTM and conditional LTM. With Inter-CU LTM and conditional LTM, we see a need to add management support. Similar to Intra-CU LTM, we propose to add requirements, use cases, possibly NRM additions, related PM measurements and an updated KPI for Intra-CU LTM and conditional LTM.

When developing NRM support for Intra-CU LTM, the configuration SA5 concluded on was an on/off switch for LTM on cell relations. This switch can be used for Inter-CU LTM as well.

However, we believe that such a switch is needed for conditional LTM.

Proposal 1: SA5 to add an on/off switch for conditional LTM, on Neighbour Cell Relation level.

When introducing PM for CHO, extra effort was taken to separate measurements for legacy handover, and for CHO. One idea behind this effort was to provide a solution that could scale when new envisioned handover mechanisms were introduced. Intra-CU LTM was such a new envisioned handover mechanism. Inter-CU LTM made SA5 add PM for the Intra-CU case. We propose that PM for Inter-CU LTM, and for conditional LTM, shall be added in a way that follows the pattern SA5 created for adding PM for CHO.

Proposal 2: SA5 to add PM for Inter-CU LTM and conditional LTM in a way that follows the pattern SA5 created for adding PM for CHO.

Furthermore, we suggest that no new KPIs shall be defined, and existing KPIs shall be extended to encompass Inter-CU LTM and conditional LTM. For example, TS 28.554 [2] clause 6.6.1 defines NG-RAN handover success rate. We suggest that this KPI should cater for Inter-CU LTM and conditional LTM, in addition to existing legacy HO,CHO and Intra-CU LTM.

Proposal 3: Existing KPIs shall be extended to cater for Inter-CU LTM and conditional LTM.

# 4	Detailed proposal

Proposal 1: SA5 to add an on/off switch for conditional LTM, on Neighbour Cell Relation level.

Proposal 2: SA5 to add PM for Inter-CU LTM and conditional LTM in a way that follows the pattern SA5 created for adding PM for CHO.

Proposal 3: Existing KPIs shall be extended to cater for Inter-CU LTM and conditional LTM.