3GPP TSG-SA5 Meeting #159	S5-250547

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | 0150 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.533 Add details for MnS versioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBMA |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The eSBMA arhitecture defines mechanisms to version (i.e. MnSVersion) and publish (i.e. MnsRegisty) MnS Producers. At present the SA5 provided stage2 artefacts contain inconsistent versioning (i.e. ‘XXX’) and there are no no giidelines on version handling. TR 28.871 contains a proposal for MnS versioning handling based on similar in TS 29.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add versioning handling details to MnS definition. Add annex with examples. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies in MnS version handling can lead to confusion and cause interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x (new), 4.x.1 (new), 4.x.2 (new), 4.x.3 (new), 4.x.4 (new), A.x (new), A.x.1 (new), A.x.1.1 (new), A.x.1.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void

[3]	3GPP TS 28.530: "Management and orchestration of networks and network slicing; Concepts, use cases and requirements".

[4]	3GPP TS 28.541: "Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 2 and stage 3".

[5]	3GPP TS 28.552: "Management and orchestration of 5G networks; Performance measurements and assurance data".

[6]	3GPP TS 28.554: "Management and orchestration of 5G networks; 5G End to end Key Performance Indicators (KPI)".

[7]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[8]	3GPP TS 28.531: "Management and orchestration of 5G networks; Provisioning; Stage 1".

[9]	3GPP TS 28.532: "Management and orchestration; Management services".

[10]	3GPP TS 28.500: "Telecommunication management; Management concept, architecture and requirements for mobile networks that include virtualized network functions"

[11]	3GPP TS 28.510; "Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Requirements".

[12]	3GPP TS 28.511; "Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Procedures".

[13]	3GPP TS 28.512; "Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Stage 2".

[14]	3GPP TS 28.513: "Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Stage 3".

[15]	3GPP TS 28.515; "Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Requirements".

[16]	3GPP TS 28.516: "Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Procedures".

[17]	3GPP TS 28.517: "Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Stage 2".

[18]	3GPP TS 28.518: "Telecommunication management; Fault Management (FM) for mobile networks that include virtualized network functions; Stage 3".

[19]	3GPP TS 28.520: "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Requirements".

[20]	3GPP TS 28.521: "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Procedures".

[21]	3GPP TS 28.522: "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 2".

[22]	3GPP TS 28.523: "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized network functions; Stage 3".

[23]	3GPP TS 28.525: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements".

[24]	3GPP TS 28.526: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[25]	3GPP TS 28.527: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Stage 2".

[26]	3GPP TS 28.528: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Stage 3".

[27]	ETSI GS NFV 003: "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV V1.3.1 (2018-01)".

[28]	 Void

[29]	ETSI GS ZSM 002: "Zero-touch Network and Service Management (ZSM); Reference Architecture V.1.1 (2019-08)".

[30]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[31]	3GPP TS 23.501: "System Architecture for the 5G system".

[32]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[33]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[34]	IETF RFC 4253: "The Secure Shell (SSH) Transport Layer Protocol".

[35]	3GPP TS 28.100: "Management and orchestration; Levels of autonomous network".

[36]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[37]	3GPP TS 28.535: "Management services for communication service assurance; Requirements ".

[38]	3GPP TS 28.536: "Management services for communication service assurance; Stage 2 and stage 3".

[39]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[40]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[41]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM); Stage 1".

[42]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[43]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[44]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management ".

[45]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace;Trace data definition and management".

[46]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[47]	3GPP TS 28.557: "Management and orchestration; Management of Non-Public Networks (NPN); Stage 1 and stage 2".

[48]	3GPP TS 28.404: "Telecommunication management; Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements".

[49]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[50]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[51]	3GPP TS 28.631: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[52]	3GPP TS 28.632: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[53]	3GPP TS 28.633: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[54]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[55]	3GPP TS 32.130: "Telecommunication management; Network sharing; Concepts and requirements".

[56]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[57]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[58]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[59]	3GPP TS 28.314: "Management and orchestration; Plug and Connect; Concepts and requirements".

[60]	3GPP TS 28.315: "Management and orchestration; Plug and Connect; Procedure flows".

[61]	3GPP TS 28.316: "Management and orchestration; Plug and Connect; Data formats".

[62]	3GPP TS 28.555: "Management and orchestration; Network policy management for 5G mobile networks; Stage 1".

[63]	3GPP TS 28.556: "Management and orchestration; Network policy management for 5G mobile networks; Stage 2 and stage 3".

[64]	ETSI GS NFV-IFA008 (V4.3.1): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[65]	ETSI GS NFV-IFA013 (V4.3.1): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[66]	3GPP TS 28.105: " Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management "

[67]	3GPP TS 28.317: "Management and orchestration;Self-configuration of Radio Access Network Entities (RAN NEs) "

[68]	3GPP TS 28.111:  "Management and orchestration;Fault management"

[69]	3GPP TS 28.318 "Management and Orchestration; Network and services operations for energy utilities"

[70]	3GPP TS 28.319 "Management and orchestration; Access Control for Management services"

[x]	3GPP 29.501: 5G System; Principles and Guidelines for Services Definition

2nd Change

## 4.X 	MnS Versioning

### 4.X.1	Introduction

A version number is generally defined as a unique identifier to differentiate specific product releases.

In the context of SBMA, the MnS version is used to differentiate capabilities (i.e. component A) to assist an MnS Consumer in understanding the standardized operations of the MnS.

For OpenAPI/yaml solution set this pertains to the MnS definitions, i.e. yaml with “MnS” definitions, and filename includes “MnS”, as defined by SA5.

For NETONF/yang this does not apply as the NETCONF protocol and associated capabilities which are considered to be the Component A equivalents are defined outside SA5 in relevant RFCs.

The MnS version does not imply other capabilities (i.e. component B and component C) which are provided during detailed discovery.

For SBMA, each MnS definition (e.g. yaml with “MnS” definitions) also contains information to identify the 3GPP specification(s) and version which were used to define it.  For OpenAPI this 3GPP version is provided in “info:version:”, distinct from the MnS version found in “servers:MnSVersion”.

### 4.X.2	MnS version definition

Per definition of “API version numbers” in [x], clause 4.3.1.1 an MnS version number consists of at least 3 fields, following a MAJOR.MINOR.PATCH pattern plus operator specific version information.

The 1st Field (MAJOR), the 2nd Field (MINOR), and the 3rd Field (PATCH) shall contain unsigned integer numbers, and they shall not contain leading zeroes.

The 4th element (vendor specific version) shall be appended after the 3 first version fields using the plus sign "+" character and shall consist of a list of dot-separated identifiers, where each identifier may contain only alphanumeric characters and hyphens ([0-9A-Za-z-]).

### 4.X.3	MnS version value handling

For SBMA, each MnS definition (e.g. yaml with “MnS” definitions) contains information to identify the MnS version and is provided in “servers:MnSVersion”.

A placeholder MnS value (e.g. ‘XXX’) is provided in the solution set definitions provided by 3GPP.

Each vendor provides appropriate value for each MnS based on their MnS implementation and product release strategy.

Recommended guidelines for version incrementing are provided in [x], clause 4.3.1.2.

### 4.X.4	MnS version usage

The MnS version is available in the MnS Registry NRM fragment [32] in support of MnS Discovery as defined in [39], clause 5.

3rd Change

# A.x	Examples of MnS version usage

## A.x.1	Version information in OpenAPI definitions

The OpenAPI solution set follows a file naming convention that indicates the TS and MnS, e.g. TS28532_ProvMnS.yaml.  This can be helpful in identifying which yaml files define an MnS.

Each contains information which can be used to associate the yaml definition to the specific 3GPP TS version on which it is based.  Excerpt from TS28532_ProvMnS.yaml:

openapi: 3.0.1

info:

title: Provisioning MnS

version: 18.5.0

description: >-

OAS 3.0.1 definition of the Provisioning MnS

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

Such files also contain default MnSVersion information:

servers:

- url: '{MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

URI-LDN-first-part:

description: See clause 4.4.2 of TS 32.158

default: ''

The default value is not used to publish the MnS version in the MnS Registry.  An appropriate MnS version value is provided by the implementation and used in the MnS Registry.

### A.x.1.1	Example, MnS version follows 3GPP release

In case of an “as-is” MnS definition (i.e. no modifications from 3GPP specification) the MnS version would be the same as that of the 3GPP release containing the 3GPP definition.

For example [A.x.1] above the value for 3GPP Release 18 would be:

MnSVersion:

description: Vendor X, Version number of the OpenAPI definition

default: 18.5.0

In case of a modified MnS definition (i.e. modifications from 3GPP specification) the MnS version should be updated to reflect the modifications.

For example [A.x.1] above, the value for 3GPP Release 18 (v18.5.0) with a backward compatible correction could be:

MnSVersion:

description: Vendor X, Version number of the OpenAPI definition

default: 18.5.1.vendorX.2025-01

For example [A.x.1] above, the value for 3GPP Release 18 with a backward compatible feature addition could be:

MnSVersion:

description: Vendor X, Version number of the OpenAPI definition

default: 18.6.0.vendorX.2025-02

For example [A.x.1] above, the value for 3GPP Release 18 with a non-backward compatible feature addition should result in a new 3GPP release:

MnSVersion:

description: Vendor X, Version number of the OpenAPI definition

default: 19.0.0.vendorX.2025-01

### A.x.1.2	Example, MnS version follows vendor release

In cases where a vendor provides their MnS implementation in a manner consistent with their product release strategy, which may be decoupled from a specific 3GPP release, the 3GPP release number would likely not be the basis for the MnS version values.

For example [A.x.1] the value for 3GPP Release 18 (v18.5.0) could be:

MnSVersion:

description: Vendor X, Version number of the OpenAPI definition

default: 1.0.0.vendorX.2025-01

For example [A.x.1] the value for 3GPP Release 18 (v18.5.0) and 3GPP Release 19 (v19.2.1) could be:

MnSVersion:

description: Vendor X, Version number of the OpenAPI definition

default: 2.0.0.vendorX.2026-01