3GPP TSG-SA5 Meeting #159	S5-250557

Sophia-Antipolis, FR, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0069 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.310 Add use case and requirements for collecting gNB and UPF Carbon Emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Use case and requirements to collect the Carbon Emission of gNBs and UPFs are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add use case and requirements to collect the Carbon Emission of gNBs and UPFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use case and requirements to collect the Carbon Emission of gNBs and UPFs would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.x (new), 5.2.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.1.x	Carbon Emission (CE) collection

#### 5.1.x.1	Applicability

The use cases for Carbon Emission measurement control, data file reporting and streaming in the following sub-clauses of clause 5.1.x are valid for gNBs and UPFs.

#### 5.1.x.2	CE measurement control

Use cases specified in TS 28.550 [5] – clause 5.1.1.1 ("NF measurement job control service") – apply for measurement job control of Carbon Emission.

Depending on scenarios, NF measurement job control services may not exist. In such a case, the NF measurement control of CE may be achieved as specified in TS 28.531 [6] – clause 5.1.18 ("Configuration of a 3GPP NF instance").

Traceability: REQ-CEMCS-FUN-001, REQ-CEMCS-FUN-002, REQ-CEMCS-FUN-003, REQ-CEMCS-FUN-004, REQ-CEMCS-FUN-005.

#### 5.1.x.3	CE measurement data file reporting

Use cases specified in TS 28.550 [5] – clause 5.1.1.2 – apply for Carbon Emission measurement data file reporting, in compliance with TS 32.432 [8], TS 32.435 [9], TS 32.436 [10].

Traceability: REQ-CEFRS-FUN-010, REQ-CEFRS-FUN-011.

#### 5.1.x.4	CE measurement data streaming

Use cases specified in TS 28.550 [5] – clause 5.1.1.3 – apply for Carbon Emission measurement data streaming.

Traceability: REQ-CEDS-FUN-020.

Next change

### 5.2.y	Requirements for Carbon Emission (CE) measurement

#### 5.2.y.1	Applicability

The requirements for Carbon Emission measurement control, data file reporting and streaming in the following sub-clauses of clause 5.2.y are valid for gNBs and UPFs.

#### 5.2.y.2	Requirements for CE measurement control

REQ-CEMCS-FUN-001: The management service producer responsible for CE measurement control shall have the capability allowing its authorized consumer to request starting the collection of CE measurement data of gNB(s) and/or UPF(s).

REQ-CEMCS-FUN-002: The management service producer responsible for CE measurement control shall have the capability allowing its authorized consumer to indicate the reporting method, granularity period, reporting period, etc. for CE measurement data of gNB(s) and/or UPF(s).

REQ-CEMCS-FUN-003: The management service producer responsible for CE measurement control shall have the capability to generate the CE measurement data of gNB(s) and/or UPF(s) according to the request of the consumer.

REQ-CEMCS-FUN-004: The management service producer responsible for CE measurement control shall have the capability allowing its authorized consumer to request stopping the collection of CE measurement data of gNB(s) and/or UPF(s).

REQ-CEMCS-FUN-005: The management service producer responsible for CE measurement control shall have the capability allowing its authorized consumer to query the information about the ongoing collection of CE measurement data of gNB(s) and/or UPF(s).

#### 5.2.y.3	Requirements for CE measurement data file reporting

REQ-CEFRS-FUN-010:	The management service producer responsible for CE performance data file reporting shall have the capability to send the notification about CE performance data (of NF(s)) file ready to its authorized consumer.

REQ-CEFRS-FUN-011:	The management service producer responsible for CE performance data file reporting shall have the capability to allow its authorized consumer to fetch the CE performance data (of NF(s)) file.

#### 5.2.y.4	Requirements for CE measurement data streaming service

REQ-CEDS-FUN-020:	The management service producer responsible for CE performance data streaming shall have the capability to send the stream containing CE performance data (of NF(s)) to its authorized consumer.

End of change