3GPP TSG-SA5 Meeting #159	S5-250584

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | 0107 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel 19 TS 28.538 Correct attributes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Attribute federationExpiry does not follow camelCase rules. Attribute allowedBundleRef is spelled incorrectly in the large attribute table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Attribute federationExpiry is changed to camelCase in attribute tables and forge. Attribute allowedBundleRef spelling is corrected in the large attribute table and IOC attribute table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May lead to incorrect implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.22, 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1580 at commit b3c41d2fda7d913fe88e2adae8fa57deb5341ed7 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 6.3.22	OperatorEdgeFederation

#### 6.3.22.1	Definition

This IOC contains attributes to support the edge federation. An instance of OperatorEdgeFederation IOC should be created and configured for each federation to be maintained provided by PO and LO. When configured the attributes override those in parent EdgeFederation instance. This IOC when instantiated represents a particular available federation.

#### 6.3.22.2	Attributes

The OperatorEdgeFederation IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622 [4]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| federationID | M | T | T | F | T |
| federationExpiry | M | T | T | F | T |
| leadingOPiD | M | T | T | F | T |
| avaibleEDNList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| acceptedEDNList | M | T | T | F | T |

#### 6.3.22.3	Attribute constraints

None.

#### 6.3.22.4	Notifications

The common notifications defined in clause 5.5 of TS 28.541 [3] are valid for this IOC, without exceptions or additions.

Next Change

### 6.3.27	EASBundleInfo <<dataType>>

#### 6.3.27.1	Definition

This datatype represent the consumer opinion on whether the EAS can be part of a bundle. ASP decides to use EAS bundle functionality due to internal policies. Post EAS deployment, ASP decideds whether the EAS should be part of any or some of the bundle. ASP may decide that a particular EAS shall never be part of any of the bundle or it can be part of some of the available bundle only. According it send modifyMOIAttribute request for EASFunction MOI to update the value of the attribute present in this <<datatype>>.

#### 6.3.27.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| isBundlingAllowed | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| allowedBundleRef | O | T | T | F | T |

#### 6.3.27.3	Attribute constraints

None.

#### 6.3.27.4	Notifications

The clause 5.5, in TS 28.541[3], of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

Next Change

### 6.4.1	Attribute Properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| eASIdentifier | It refers to EASID that identifies a particular application (e.g. SA6Video, SA6Game, … etc.) (see clause 7.2.4 in TS 23.558 [2]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASAddress | One or more URLs and/or IP Address(es) of EAS(s) (See EAS Endpoint defined in clause 8.2.4 in TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eASREquirementsRef | This is the DN of EASRequirements. allowedValues: Not applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eESFunctionRef | This is the DN of EESFunction. allowedValues: DN of the EESFunction MOI. | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| registrationInfo | This refers to the registration information (see clause see clause 8.4.3 and 8.4.4 in TS 23.558 [2]). allowedValues: N/A | type: RegistrationInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| registrationExpiry | This specifies the expiration time of the EAS and EES Registration (see clause 8.4.3 and 8.4.4 in TS 23.558 [2]). | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| registrationID | This identifies particular EAS and EES registration. (see clause 8.4.3 and 8.4.4 in TS 23.558 [2]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| secCredential | This specifies the security credentials of the EAS and EES Registration (see clause 8.4.3 and 8.4.4 in TS 23.558 [2]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| edgeDataNetworkRef | This holds a list of DN of EdgeDataNetwork. | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| requiredEASservingLocation | It defines the location where the EAS service should be available (see clause 7.3.3.6 in TS 23.558 [2]). | type: ServingLocation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| geographicalLocation | This refers to the Geographical Service Area, (see clause 7.3.3.3 in TS 23.558 [2] that is defined as a datatype (see clause 6.3.4). allowedValues: N/A | type: GeoLoc multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| latitude | This defines the single latitude coordinate. | type: Float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| longitude | This defines the single longitude coordinate. | type: Float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| civicLocation | This defines the civic locations, such as: a well-known buildings, parks, arenas, civic addresses, or ZIP code etc (see clause 7.3.3.3 in TS 23.558 [2]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| topologicalLocation | This refers to the Topological Service Area, (see clause 7.3.3.2 in TS 23.558 [2]) that is defined as a datatype (see clause 6.3.7). allowedValues: N/A | type: TopologicalServiceArea multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| geographicalCoordinates | This refers to the Topological Service Area, (see clause 7.3.3.2 in TS 23.558 [2]) that is defined as a datatype (see clause 6.3.8). allowedValues: N/A | type: GeographicalCoordinates multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| softwareImageInfo | This refers to the software image information (e.g. software image location, minimum RAM, disk requirements) (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). It is defined as a datatype (see clause 6.3.9). allowedValues: N/A | type: SoftwareImageInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| swImageRef | It indicates the reference to the actual software image that is represented by URL (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| minimumDisk | It indicates the minimum disk size requirement for the EAS software (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). The unit is Megabyte. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| minimumRAM | It indicates the minimum RAM size requirement for the EAS software (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). The unit is Megabyte. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| diskFormat | It indicates the disk format requirement for the EAS software (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| operatingSystem | It indicates the operating system requirement for the EAS software (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| cellIDList | It represents the list of NR cells. The cell ID, together with the gNB Identifier (using gNBId of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction), identifies a NR cell within a PLMN. This is the NR Cell Identity (NCI). See subclause 8.2 of TS 38.300 [13]. AllowedValues: Not applicable | type: Integer multiplicity: * isOrdered: False isUnique: Yes defaultValue: None isNullable: False |
| trackingAreaIdList | It represents the list of tracking areas within a PLMN. | type: TAI multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servingPLMN | It specifies the PLMN to be served. | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ecsAddress | One or more URLs and/or IP Address(es) of ECS(s) (See ECS address defined in clause 8.2.12 in TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| providerIdentifier | The identifier of the ECSP that provides the ECS (See TS 23.558 [2]). allowedValues: N/A | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eDNConnectionInfo | It defines the set of information needed to connect to an EDN. | type: EDNConnectionInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eDNServiceArea | This parameter defines the service location for the EDN (see clause 7.3.3.4 in TS 23.558 [2]). | type: ServingLocation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ednIdentifier | The identifier of the edge data network (See TS 23.558 [2]). allowedValues: N/A | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| affinityAntiAffinity | This parameter defines the affinity and anti-requirements of the EAS with other EAS on the same EDN. | type: AffinityAntiAffinity multiplicity: 1 isOrdered: N/A isUnique: defaultValue: None isNullable: False |
| affinityEAS | This parameter defines the EAS identifier with which the affinity is required. | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| antiAffinityEAS | This parameter defines the EAS identifier with which the anti-affinity is required. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceContinuity | This parameter defines if the service continuity is required by the EAS. If the value is TRUE, the EAS will be deployed with an EES supporting service continuity. | type: Boolean multiplicity: 1..* isOrdered: False isUnique: True defaultValue: False isNullable: False |
| virtualResource | This parameter defines the virtual resource requirements of an EAS. | type: VirtualResource multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| virtualMemory | It indicates the minimum virtual memory size requirements for EAS in megabytes. (see clause 7.1.9.3.2.2 in ETSI NFV IFA-011 [7]). | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| virtualDisk | It indicates the minimum virtual disk storage requirement for the EAS (see clause 7.1.9.4.3.2 in ETSI NFV IFA-011 [7]). | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| virtualCPU | It indicates the virtual CPU requirement for the EAS (see clause 7.1.9.2.3.2 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eESAddress | One or more URLs and/or IP Address(es) of EES(s) (See EES Endpoint defined in clause 8.2.6 in TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eESIdentifier | It identifies the EES, see EESID defined in clause 8.2.6 in 3GPP TS 23.558 [2]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASFunctionRef | This is the DN of EASFunction. allowedValues: DN of the EASFunction MOI. | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| serviceContinuitySupport | This parameter defines whether the EES supports service continuity, see 3GPP TS 23.558 | type: Boolen multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eESservingLocation | It defines the serving location for an EES. | type: ServingLocation multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eESFunctionRef | This is the DN of EESFunction. allowedValues: DN of the EESFunction MOI. | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aCID | Identifies the AC(s) that can be served by the EAS (See TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eASProvider | The identifier of the ASP that provides the EAS (See EAS Provider ID defined in clause 8.6.5.3.2 in TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASdescription | Human-readable description of the EAS (See EAS description defined in clause 8.2.4 in TS 23.558 [2]). allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASSchedule | The availability schedule of the EAS (e.g. time windows) (See EAS Schedule defined in clause 8.2.4 in TS 23.558 [2]). The detailed definition for TimeWindow can be found in TS 28.622 [4]. allowedValues: N/A | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASGeographicalServiceArea | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served (See EAS Geographical Service Area defined in clause 8.2.4 in TS 23.558 [2]). allowedValues: N/A | type: GeoLoc multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eASTopologicalServiceArea | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. (See EAS Topological Service Area defined in clause 8.2.4 in TS 23.558 [2]). allowedValues: N/A | type: TopologicalServiceArea multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eASServicePermissionLevel | Level of service permissions e.g. trial, gold-class supported by the EAS (See EAS service permission level defined in clasue 8.2.4 in TS 23.558 [2]). Allowed Values: TRIAL, SILVER, GOLD | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASFeature | Service features e.g. single vs. multi-player gaming service supported by the EAS (See EAS Feature(s) defined in clause 8.2.4 in TS 23.558 [2]). Allowed Value: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASServiceContinuitySupport | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS (See EAS Service continuity support defined in clause 8.2.4 in TS 23.558 [2]). | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| eASDNAI | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASAvailabilityReportingPeriod | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration (See EAS Availability Reporting Period defined in clause 8.2.4 in TS 23.558 [2]). allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eASStatus | The status of the EAS (e.g. enabled, disabled, overload warning, etc.) (See EAS Status defined in clause 8.2.4 in TS 23.558 [2]). Allowed values: ENABLED, DISABLED, OVERLOAD_WARNING | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reservationLocation | This parameter defines the location where the resource needs to be reserved | type: ServingLocation multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| resourceReservationRequirement | This parameter defines the resource requirements that needs to be reserved. | type: ResourceReservationRequirement multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| computeRequirement | This parameter defines the compute requirement for reservation (see VirtualComputeDesc in clause 7.1.9.2.2 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| storageRequirement | This parameter defines the storaget requirement for reservation (see VirtualStorageDesc in clause 7.1.9.4.2 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networtkingRequirement | This parameter defines the networking requirement for reservation. It is described as the connection bandwidth in Kbit/s reserved for EAS to use. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| requestedReservationExpiration | This parameter defines the MnS consumer's requirememts for the validity period of the resource reservation. allowedValues: N/A | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| resourceReservationStatus | This parameter defines the status for the reserved resources. | type: ResourceReservationStatus multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| resourceId | It identifies a reserved resource. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reservationStatus | This parameter defines the status for a reserved resource. This attribute is configured by MnS producer and can be read by MnS consumer. Allowed Value: RESERVED: which means the specified resources is reserved and available to be used by the ASP. USED: which means the reserved resource is used by ASP. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| relocationTriggerInfo | This attribute dictates the relocation trigger for the EAS. It is a complex type which include the following attributes. allowedValues: N/A | type: RelocationTriggerInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| relocationType | This attribute defines if the EAS is to be relocated immediately or at a future point of time. AllowedValue: “IMMEDIATE”, “FUTURE”, “NO-RELOCATION” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| futuristicTriggerTime | This attribute defines a time stamp in future at which the EAS relocation will be initiated. allowedValues: N/A | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| relocationRejectByASP | A Boolean attribute which can be updated by the ASP to indicate its disagreement with the relocation. The value TRUE indicate that the ASP do not agree with the relocation. Allowed Values: NA | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| relocationPolicy | This attribute described the EAS relocation policies from the ASP. YES: This dictates that an EAS can be relocated as and when required NO: This dictates an EAS cannot be relocated at all YESwNOTIFY: This indicates that an EAS can be relocated with a prior notification allowedValues: "YES", "NO", “YESwNOTIFY” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availableEdgeVirtualResources | This parameter defines the available edge virtual resources managed by an EDN (see NfviCapacityInfo in clause 10.5.2.3 of ETSI NFV SOL-005 [17]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| vnfdId | It indicates the identifier of the VNFD which contains the virtual resource requirements of an EAS. (see clause 7.1.2.2 in ETSI NFV IFA-011 [7]). | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| participatingOPiD | This identifies the PO. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| leadingOPiD | This identifies the LO. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| federationID | This identifies the particular federation created. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| federationExpiry | This defines the time post which the federation relationship shall expire. allowedValues: N/A | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| initiationTime | Date and time of the federation initiated by the Leading operator. allowedValues: N/A | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| acceptedEDNList | It provides the list of EDN that are accepted by the LO. allowedValues: N/A | type: DN multiplicity: 1…* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| resourceQuota | This defines the virtual resource quota assigned to the LO by the PO as per the federation relationship. This may be the subset of available virtual resource (indicate with attribute availableVirtualResource) in the EDN. The LO will only be authorized to reserve and use this amount of resources. allowedValues: N/A | type: VirtualResource multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availableEASResource | This defines the available EAS in the shared EDN. This will be the DN of EASProfile. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availableEDNList | This defines information related with offered EDN available with PO. allowedValues: N/A | type: AvailableEDN multiplicity: 1...* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| federationID | This defines the federation ID provided by the PO to LO at the time of federation establishment. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reservationJobRef | This identifies the reserved block of resources. This will be the DN of EASResourceReservationJob. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| federateECSIdentifier | This defines the ECS that is to be shared as part of edge federation. This will be a DN of the ECS deployed in the participating operator domain for edge services. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| federatedPOPIdentifier | The identifier of the participating operator | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| federatedECSProfile | The information related with ECS Profile. See clause 8.2.12 of [2]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| servedEASList | This defines the list of EAS(s) available with the partner ECS. This specifies the DN of EASFunction instance. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| servedEESList | This defines the list of EES(s) available with the partner ECS. This specifies the DN of EESFunction instance. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| sharedECSInfo | This defines the ECS(s) belonging to P-OP that can be used in case of roaming and federation | type: FederatedECSInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| federatedECSInfo | This defines the information related with shared ECS | type: FederatedECSInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| availableEDN | This defines the available EDN. | type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eASDeploymentMonitor | Provides monitoring for the process of deployment of EAS(s). The data type of this attribute is the "ProcessMonitor" as defined in TS 28.622[4]. allowedValues: N/A | type: ProcessMonitor multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bundleIdentifier | Identifier of the bundle. See clause 8.2.2 [2] | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bundledEASIdentifier | List of EAS identifier associated with the EAS bundle. See clause 8.2.2 [2]. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| bundleType | Type of the EAS bundle. See clause 8.2.2 [2]. Allowed Values: DIRECT, PROXY | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mainEASIdentifier | Indicate which EAS in a bundle takes the main EAS service role. See clause 8.2.2 [2]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coordinatedEASDiscovery | Indicates if coordinated EAS discovery is required i.e., if EAS discovery request for one of the bundled EAS is processed, then EAS discovery response should include information of all the EASs belonging to the bundle. See clause 8.2.10 [2]. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| coordinatedACR | Indicates if coordinated ACR is required i.e., if EAS ACR is initiated for one of the bundled EAS, then ACR should be initiated for all the EASs belonging to the bundle. The IE may further indicate what actions must be taken if ACR for one or more bundled EAS fails e.g. ACR for all other EAS that are part of the bundle must be cancelled or not. See clause 8.2.10 [2]. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eDNAffinity | Indicates the affinity requirement of the EAS bundle. The IE can be set to "strong" indicating that the EASs must be in the same EDN, "preferred" indicating that it is nice to have EASs in the same EDN but not essential or "weak" indicating that it’s not essential for EASs to be in the same EDN. See clause 8.2.10 [2]. Allowed Values: “STRONG”, “WEAK”, “PREFERRED” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EASBundle.eASFunctionRef | This indicates the constituent EAS of the EAS bundle. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| EASBundle.EESFunctionRef | This indicate the related EES with the EAS bundle | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| EASBundle. eASRequirementsRef | This indicates the requirements for each constituent EAS in the bundle. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| EASFunction. eASBundleRef | This indicates the EAS bundles in which the EAS is included. See clause 8.2.2 [2]. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| EESFunction. eASBundleRef | This indicates the related EAS bundles with the EES. See clause 8.2.2 [2]. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| eASBundleInfo | This defines the bundle relation information for the EAS | type: EASBundleInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue:None isNullable: False |
| isBundlingAllowed | This defines whether the EAS should be put in a bundle. The FALSE value indicates that this EAS shall not be part of any bundle. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue:TRUE isNullable: False |
| allowedBundleRef | This defines the EAS bundles where this EAS can be placed. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| administrativeState | It indicates the administrative state of the managed edge NF service. It describes the permission to use or prohibition against using the managed edge NF service, imposed through the OAM services. allowedValues: “LOCKED”, “UNLOCKED”, SHUTTINGDOWN” The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | It indicates the operational state of the managed edge NF service. It describes whether or not the resource is physically installed and working. allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| userLabel | A user-friendly (and user assignable) name of this services offered by the edge entities as defined in 3GPP TS 29.558 [20]. allowedValues: N/A | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sAP | This parameter specifies the service access point of the managed Edge NF service instance. allowedValues: N/A | type: SAP multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| operations | This parameter defines set of operations supported by the managed Edge NF service instance. allowedValues: See TS 29.558 [20] for supported operations | type: Operation multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| Operation.name | This parameter defines the name of the operation of the managed edge NF service instance. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| operationSemantics | This paramerter identifies the semantics type of the operation. See TS 29.558 [20] allowedValues: “REQUEST_RESPONSE”, “SUBSCRIBE_NOTIFY”. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| host | This parameter specifies the host address of the managed edge NF service instance. It can be FQDN (See TS 23.003 [13]) or an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). allowedValues: N/A | type: Host multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| port | This parameter specifies the transport port of the managed edge NF service instance. allowedValues: 1 - 65535 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

Next Change

*** START OF CHANGE 1 ***

*** OpenAPI/TS28538_EdgeNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: 3GPP Edge NRM

version: 19.2.0

description: >-

OAS 3.0.1 specification of the Edge NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.538; Edge NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.538/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

ServingLocation:

type: object

properties:

geographicalLocation:

$ref: '#/components/schemas/GeoLoc'

topologicalLocation:

$ref: '#/components/schemas/TopologicalServiceArea'

TopologicalServiceArea:

type: object

properties:

cellIdList:

type: array

items:

type: integer

trackingAreaIdList:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/TaiList'

servingPLMN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

GeoLoc:

type: object

properties:

geographicalCoordinates:

$ref: '#/components/schemas/GeographicalCoordinates'

civicLocation:

type: string

GeographicalCoordinates:

type: object

properties:

latitude:

type: integer

longitude:

type: integer

EDNConnectionInfo:

type: object

properties:

dNN:

type: string

eDNServiceArea:

$ref: '#/components/schemas/ServingLocation'

AffinityAntiAffinity:

type: object

properties:

affinityEAS:

type: array

items:

type: string

antiAffinityEAS:

type: array

items:

type: string

VirtualResource:

type: object

properties:

virtualMemory:

type: integer

virtualDisk:

type: integer

virtualCPU:

type: string

vnfdId:

type: string

SoftwareImageInfo:

type: object

properties:

minimumDisk:

type: integer

minimumRAM:

type: integer

discFormat:

type: string

operatingSystem:

type: string

swImageRef:

type: string

RegistrationInfo:

type: object

properties:

registrationExpiry:

type: string

readOnly: true

registrationID:

type: string

readOnly: true

secCredential:

type: string

EASServicePermission:

type: string

description: any of enumerated value

readOnly: true

enum:

- TRIAL

- SILVER

- GOLD

EASFeature:

type: string

EASStatus:

type: string

description: any of enumerated value

readOnly: true

enum:

- ENABLED

- DISABLED

- OVERLOAD_WARNING

ResourceReservationRequirement:

type: object

properties:

computeRequirement:

type: string

storageRequirement:

type: string

networkingRequirement:

type: integer

ResourceReservationStatus:

type: object

properties:

resourceId:

type: string

readOnly: true

reservationStatus:

type: string

description: any of enumrated value

readOnly: true

enum:

- RESERVED

- USEd

RelocationTriggerInfo:

type: object

properties:

triggerType:

type: string

description: any of enumrated value

enum:

- IMMEDIATE

- FUTURE

futuristicTriggerTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

AvailableEDN:

type: object

properties:

AvailableEDN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

resourceQuota:

$ref: '#/components/schemas/VirtualResource'

FederatedECSInfo:

type: object

properties:

federateECSIdentifier:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

federatedECSProfile:

type: integer

servedEASList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

servedEESList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

SAP:

type: object

properties:

host:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/HostAddr'

port:

type: integer

Operation:

type: object

properties:

name:

type: string

allowedConsumers:

type: string

operationSemantics:

type: string

description: any of enumrated value

enum:

- REQUEST_RESPONSE

- SUBSCRIBE_NOTIFY

#-------- Definition of types for name-containments ------

SubNetwork-ncO-EdgeNrm:

type: object

properties:

ECSFunction:

$ref: '#/components/schemas/ECSFunction-Multiple'

EdgeDataNetwork:

$ref: '#/components/schemas/EdgeDataNetwork-Multiple'

EASBundle:

$ref: '#/components/schemas/EASBundle-Multiple'

#-------- Definition of Edge common IOCs --------------------------------------------

ManagedEdgeNFService-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

userLabel:

type: string

sAP:

$ref: '#/components/schemas/SAP'

operations:

type: array

items:

$ref: '#/components/schemas/Operation'

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

#-------- Definition of Edge common IOCs --------------------------------------------

#-------- Definition of concrete IOCs --------------------------------------------

EdgeDataNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

ednIdentifier:

type: string

eDNConnectionInfo:

$ref: '#/components/schemas/EDNConnectionInfo'

- type: object

properties:

EASFunction:

$ref: '#/components/schemas/EASFunction-Multiple'

EESFunction:

$ref: '#/components/schemas/EESFunction-Multiple'

availableEdgeVirtualResources:

type: string

readOnly: true

EASFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eASIdentifier:

type: string

eESAddress:

type: array

items:

type: string

registrationInfo:

$ref: '#/components/schemas/RegistrationInfo'

eASRequirementsRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

eASAddress:

type: array

items:

type: string

relocationTriggerInfo:

$ref: '#/components/schemas/RelocationTriggerInfo'

relocationRejectByASP:

type: boolean

default: FALSE

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

EASProfile-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

aCID:

type: string

readOnly: true

eASProvider:

type: string

readOnly: true

eASdescription:

type: string

readOnly: true

eASSchedule:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindowRo'

eASGeographicalServiceArea:

$ref: '#/components/schemas/GeoLoc'

eASTopologicalServiceArea:

$ref: '#/components/schemas/TopologicalServiceArea'

eASServicePermissionLevel:

$ref: '#/components/schemas/EASServicePermission'

eASFeature:

$ref: '#/components/schemas/EASFeature'

eASServiceContinuitySupport:

type: boolean

readOnly: true

default: FALSE

eASDNAI:

type: string

readOnly: true

eASAvailabilityReportingPeriod:

type: integer

readOnly: true

eASStatus:

$ref: '#/components/schemas/EASStatus'

EESFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eESIdentifier:

type: string

eESServingLocation:

type: array

items:

$ref: '#/components/schemas/ServingLocation'

eESAddress:

type: array

items:

type: string

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

serviceContinuitySupport:

type: boolean

eASFunctionRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

registrationInfo:

$ref: '#/components/schemas/RegistrationInfo'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

ManagedEdgeNFService:

$ref: '#/components/schemas/ManagedEdgeNFService-Multiple'

ECSFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eCSAddress:

type: string

providerIdentifier:

type: string

edgeDataNetworkRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

eESFuncitonRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

sharedECSInfo:

$ref: '#/components/schemas/FederatedECSInfo'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

ManagedEdgeNFService:

$ref: '#/components/schemas/ManagedEdgeNFService-Multiple'

EASRequirements-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

requiredEASservingLocation:

$ref: '#/components/schemas/ServingLocation'

affinityAntiAffinity:

$ref: '#/components/schemas/AffinityAntiAffinity'

serviceContinuity:

type: boolean

virtualResource:

$ref: '#/components/schemas/VirtualResource'

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

eASSchedule:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

eASFeature:

$ref: '#/components/schemas/EASFeature'

relocationPolicy:

type: string

description: any of enumrated value

enum:

- YES

- NO

- YESwNOTIFY

federationID:

type: string

eASDeploymentMonitor:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ProcessMonitor'

reservationJobRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

EASResourceReservationJob-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

reservationLocation:

$ref: '#/components/schemas/ServingLocation'

resourceReservationRequirement:

type: array

items:

$ref: '#/components/schemas/ResourceReservationRequirement'

requestedReservationExpiration:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

resourceReservationStatus:

type: array

items:

$ref: '#/components/schemas/ResourceReservationStatus'

EdgeFederation-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

participatingOPiD:

type: string

leadingOPiD:

type: string

federatedECSInfo:

$ref: '#/components/schemas/FederatedECSInfo'

OperatorEdgeFederation-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

federationID:

type: string

federationExpiry:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

leadingOPiD:

type: string

availableEDNList:

type: array

items:

$ref: '#/components/schemas/AvailableEDN'

acceptedEDN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

OperatorEdgeDataNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

availableEASResource:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

edgeDataNetworkRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

EASBundle-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

bundleIdentifier:

type: string

readOnly: true

bundledEASIdentifier:

type: string

bundleType:

type: string

mainEASIdentifier:

type: string

coordinatedEASDiscovery:

type: boolean

default: FALSE

coordinatedACR:

type: boolean

eDNAffinity:

type: string

description: any of enumrated value

enum:

- STRONG

- WEAK

- PREFERRED

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

EASFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/EASFunction-Single'

ECSFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ECSFunction-Single'

EESFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/EESFunction-Single'

EdgeDataNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/EdgeDataNetwork-Single'

EASProfile-Multiple:

type: array

items:

$ref: '#/components/schemas/EASProfile-Single'

EdgeFederation-Multiple:

type: array

items:

$ref: '#/components/schemas/EASProfile-Single'

OperatorEdgeFederation-Multiple:

type: array

items:

$ref: '#/components/schemas/OperatorEdgeFederation-Single'

OperatorEdgeDataNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/OperatorEdgeDataNetwork-Single'

EASBundle-Multiple:

type: array

items:

$ref: '#/components/schemas/EASBundle-Single'

ManagedEdgeNFService-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedEdgeNFService-Single'

#--------------------------------- Definition ------------------------------------

resources-edgeNrm:

oneOf:

- $ref: '#/components/schemas/EASFunction-Single'

- $ref: '#/components/schemas/ECSFunction-Single'

- $ref: '#/components/schemas/EESFunction-Single'

- $ref: '#/components/schemas/EdgeDataNetwork-Single'

- $ref: '#/components/schemas/EASRequirements-Single'

- $ref: '#/components/schemas/EASProfile-Single'

- $ref: '#/components/schemas/EdgeFederation-Single'

- $ref: '#/components/schemas/OperatorEdgeFederation-Single'

- $ref: '#/components/schemas/OperatorEdgeDataNetwork-Single'

- $ref: '#/components/schemas/EASResourceReservationJob-Single'

- $ref: '#/components/schemas/EASBundle-Single'

- $ref: '#/components/schemas/ManagedEdgeNFService-Single'

<CODE ENDS>