3GPP TSG-SA5 Meeting #159	S5-250596

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0497 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.623 Add GenericCollection to support grouping of objects - Stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR is related to WT-2 (Energy supply, carbon emission and renewable energy information elements) of Energy_OAM_Ph3 WID. Requirements in TS 22.261 (clause 6.15a.5.2) specifies the requirements to expose the energy related information, including ratio of renewable energy and carbon emission information for the 5G system. The Energy Information Function (EIF) newly introduced into TS 23.501 is one of the consumer of gNB and UPF Energy Consumption and Data Volume measurements from OAM, as well as other energy related information such as carbon emission and renewable energy factor. The energy related information includes the energy usage information for group of one or more sub-network(s), one or more managed functions and/or managed elements that are deployed at an operator site or data center or grouped under a sub-network, that are are powered using the same energy sources. Hence to support this use case a mechanism to group such entities is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce GenericCollection IOCs in stage 3 YAML, and updates to support energy related IOCs defined in TS 28.310 CR 0067. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator would not be able to group the managed entities that are powered using the same energy supplies. Operator would not be able to configure the energy related information for the managed entities, and hence would not be able to satisfy SA1 requirements specified in TS 22.261. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (Normative stage 3 in forge) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 28.622 CR 0518, TS 28.310 CR 0067, TS 28.532 CR 0365 |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1599 at commit 0500b068848a8812e7fc37c707b0d600bdf2da17 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-250596 is a revision of S5-250203 |  |  |  |  |  |  |  |  |

1st Change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1599 at commit 0500b068848a8812e7fc37c707b0d600bdf2da17

*** START OF CHANGE 3 ***

*** OpenAPI/TS28623_GenericNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Generic NRM

version: 19.2.0

description: >-

OAS 3.0.1 definition of the Generic NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

VnfParameter:

type: object

properties:

vnfInstanceId:

type: string

vnfdId:

type: string

flavourId:

type: string

autoScalable:

type: boolean

PeeParameter:

type: object

properties:

siteIdentification:

type: string

siteDescription:

type: string

siteLatitude:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Latitude'

siteLongitude:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Longitude'

siteAltitude:

type: number

format: float

equipmentType:

type: string

environmentType:

type: string

powerInterface:

type: string

TransportProtocol:

anyOf:

- type: string

enum:

- TCP

- type: string

SupportedPerfMetricGroup:

type: object

properties:

performanceMetrics:

type: array

uniqueItems: true

items:

type: string

readOnly: true

minItems: 1

granularityPeriods:

type: array

uniqueItems: true

items:

type: integer

minimum: 1

readOnly: true

reportingMethods:

type: array

uniqueItems: true

items:

type: string

enum:

- FILE_BASED_LOC_SET_BY_PRODUCER

- FILE_BASED_LOC_SET_BY_CONSUMER

- STREAM_BASED

readOnly: true

reportingPeriods:

type: array

uniqueItems: true

items:

type: integer

minimum: 1

readOnly: true

ReportingCtrl:

oneOf:

- type: object

properties:

fileReportingPeriod:

type: integer

- type: object

properties:

fileReportingPeriod:

type: integer

notificationRecipientAddress:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

- type: object

properties:

fileReportingPeriod:

type: integer

fileLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

- type: object

properties:

streamTarget:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

ProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type without specialisations.

type: object

properties:

id:

type: string

readOnly: true

status:

type: string

enum:

- NOT_STARTED

- RUNNING

- FINSHED

- FAILED

- PARTIALLY_FAILED

- CANCELLING

- CANCELLED

readOnly: true

progressPercentage:

type: integer

minimum: 0

maximum: 100

readOnly: true

progressStateInfo:

type: string

readOnly: true

resultStateInfo:

type: string

readOnly: true

startTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

endTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

timer:

type: integer

AreaScope:

allOf:

- type: array

items:

$ref: '#/components/schemas/NpnId-Type'

minItems: 1

- oneOf:

- type: array

items:

$ref: '#/components/schemas/EutraCellId'

minItems: 1

maxItems: 32

- type: array

items:

$ref: '#/components/schemas/NrCellId'

minItems: 1

maxItems: 32

- type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

maxItems: 8

- type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 1

maxItems: 8

Tai:

type: object

properties:

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

mnc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mnc'

tac:

$ref: '#/components/schemas/Tac'

Tac:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

EutraCellId:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

NrCellId:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

IpAddr:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Ipv4Addr'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Ipv6Addr'

SchedulingTime:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

- type: object

properties:

timeIntervals:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

- type: object

properties:

timeIntervals:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

daysOfWeek:

type: array

uniqueItems: true

items:

type: string

enum:

- MONDAY

- TUESDAY

- WEDNESDAY

- THURSDAY

- FRIDAY

- SATURDAY

- SUNDAY

minItems: 1

maxItems: 7

- type: object

properties:

timeIntervals:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

daysOfMonth:

type: array

uniqueItems: true

items:

type: integer

minimum: 0

maximum: 31

NpnId-Type:

type: object

properties:

plmnId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

cAGIdList:

type: array

uniqueItems: true

items:

type: string

maxItems: 256

nIDList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Nid'

maxItems: 16

oneOf:

- required: [plmnId, cAGIdList]

- required: [plmnId, nIDList]

#-------- Definition of abstract IOC Top -----------------------------------------

Top-Attr:

#  This definition will be deprecated, when all occurances of Top-Attr

#  are replaced by Top.

type: object

properties:

id:

type: string

nullable: true

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

required:

- id

Top:

type: object

properties:

id:

type: string

nullable: true

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

required:

- id

#-------- Definition of IOCs with new name-containments defined in other TS ------

SubNetwork-Attr:

type: object

properties:

dnPrefix:

type: string

userLabel:

type: string

userDefinedNetworkType:

type: string

setOfMcc:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

readOnly: true

minItems: 1

priorityLabel:

type: integer

readOnly: true

supportedPerfMetricGroups:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

uniqueItems: true

items:

type: string

ManagedElement-Attr:

type: object

properties:

dnPrefix:

type: string

managedElementTypeList:

type: array

uniqueItems: true

items:

type: string

userLabel:

type: string

locationName:

type: string

managedBy:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

vendorName:

type: string

readOnly: true

userDefinedState:

type: string

swVersion:

type: string

readOnly: true

priorityLabel:

type: integer

supportedPerfMetricGroups:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

uniqueItems: true

items:

type: string

readOnly: true

SubNetwork-ncO:

type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

ManagementNode:

$ref: '#/components/schemas/ManagementNode-Multiple'

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

MeContext:

$ref: '#/components/schemas/MeContext-Multiple'

AlarmList:

$ref: 'TS28111_FaultNrm.yaml#/components/schemas/AlarmList-Single'

Scheduler:

$ref: '#/components/schemas/Scheduler-Multiple'

ConditionMonitor:

$ref: '#/components/schemas/ConditionMonitor-Multiple'

SupportedNotifications:

$ref: '#/components/schemas/SupportedNotifications-Single'

GenericCollection:

$ref: '#/components/schemas/GenericCollection-Multiple'

ManagedElement-ncO:

type: object

properties:

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

AlarmList:

$ref: 'TS28111_FaultNrm.yaml#/components/schemas/AlarmList-Single'

Scheduler:

$ref: '#/components/schemas/Scheduler-Multiple'

ConditionMonitor:

$ref: '#/components/schemas/ConditionMonitor-Multiple'

SupportedNotifications:

$ref: '#/components/schemas/SupportedNotifications-Single'

#-------- Definition of abstract IOCs --------------------------------------------

ManagedFunction-Attr:

type: object

properties:

userLabel:

type: string

vnfParametersList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/VnfParameter'

peeParametersList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/PeeParameter'

priorityLabel:

type: integer

supportedPerfMetricGroups:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

uniqueItems: true

items:

type: string

EP_RP-Attr:

type: object

properties:

userLabel:

type: string

farEndEntity:

type: string

readOnly: true

supportedPerfMetricGroups:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

ManagedFunction-ncO:

type: object

properties:

PerfMetricJob:

$ref: 'TS28623_PmControlNrm.yaml#/components/schemas/PerfMetricJob-Multiple'

ThresholdMonitor:

$ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ThresholdMonitor-Multiple'

TraceJob:

$ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/TraceJob-Multiple'

#-------- Definition of concrete IOCs --------------------------------------------

VsDataContainer-Single:

type: object

properties:

id:

type: string

attributes:

type: object

properties:

vsDataType:

type: string

readOnly: true

vsDataFormatVersion:

type: string

readOnly: true

vsData:

nullable: true

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

ManagementNode-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

userLabel:

type: string

managedElements:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

vendorName:

type: string

readOnly: true

userDefinedState:

type: string

locationName:

type: string

readOnly: true

swVersion:

type: string

readOnly: true

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

MnsAgent-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

systemDN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

MeContext-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

dnPrefix:

type: string

readOnly: true

Scheduler-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

schedulingTimes:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/SchedulingTime'

minItems: 1

schedulerStatus:

type: boolean

readOnly: true

ConditionMonitor-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

condition:

type: string

conditionStatus:

type: boolean

readOnly: true

SupportedNotifications-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

notificationTypes:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationType'

notificationProtocols:

type: array

uniqueItems: true

items:

type: string

enum:

- HTTP

- HTTP_VES_ENCAPS

minItems: 1

readOnly: true

SubNetwork-Single:

allOf:

- $ref: '#/components/schemas/Top'

- $ref: '#/components/schemas/SubNetwork-Attr'

- $ref: '#/components/schemas/SubNetwork-ncO'

- $ref: '#/components/schemas/SubNetwork-Multiple'

- $ref: '#/components/schemas/ManagedElement-Multiple'

- $ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/SubNetwork-ncO-FileManagementNrm'

- $ref: 'TS28623_PmControlNrm.yaml#/components/schemas/SubNetwork-ncO-PmControlNrm'

- $ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/SubNetwork-ncO-SubscriptionControlNrm'

- $ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/SubNetwork-ncO-ThresholdMonitorNrm'

- $ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/SubNetwork-ncO-TraceControlNrm'

- $ref: 'TS28623_QoEMeasurementCollectionNrm.yaml#/components/schemas/SubNetwork-ncO-QoEMeasurementCollectionNrm'

- $ref: 'TS28623_ManagementDataCollectionNrm.yaml#/components/schemas/SubNetwork-ncO-ManagementDataCollectionNrm'

- $ref: 'TS28623_MnSRegistryNrm.yaml#/components/schemas/SubNetwork-ncO-MnSRegistryNrm'

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/SubNetwork-ncO-MdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/SubNetwork-ncO-AiMlNrm'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/SubNetwork-ncO-IntentNrm'

- $ref: 'TS28317_RanScNrm.yaml#/components/schemas/SubNetwork-ncO-RanScNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/SubNetwork-ncO-CoslaNrm'

- $ref: 'TS28538_EdgeNrm.yaml#/components/schemas/SubNetwork-ncO-EdgeNrm'

- $ref: 'TS28541_SliceNrm.yaml#/components/schemas/SubNetwork-ncO-SliceNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/SubNetwork-ncO-NrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/SubNetwork-ncO-5GcNrm'

- $ref: 'TS28318_DsoNrm.yaml#/components/schemas/SubNetwork-ncO-DsoNrm'

- $ref: 'TS28310_EnergyEfficiency.yaml#/components/schemas/SubNetwork-ncO-EENrm'

ManagedElement-Single:

allOf:

- $ref: '#/components/schemas/Top'

- $ref: '#/components/schemas/ManagedElement-Attr'

- $ref: '#/components/schemas/ManagedElement-ncO'

- $ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/ManagedElement-ncO-FileManagementNrm'

- $ref: 'TS28623_PmControlNrm.yaml#/components/schemas/ManagedElement-PmControlNrm'

- $ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/ManagedElement-ncO-SubscriptionControlNrm'

- $ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ManagedElement-ncO-ThresholdMonitorNrm'

- $ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/SubNetwork-ncO-TraceControlNrm'

- $ref: 'TS28623_QoEMeasurementCollectionNrm.yaml#/components/schemas/ManagedElement-ncO-QoEMeasurementCollectionNrm'

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/ManagedElement-ncO-MdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/ManagedElement-ncO-AiMlNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/ManagedElement-ncO-CoslaNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/ManagedElement-ncO-NrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/ManagedElement-ncO-5GcNrm'

NrmRoot:

oneOf:

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

- type: object

properties:

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

GenericCollection-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

userLabel:

type: string

memberDNs:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

#-------- Definition of YAML arrays for name-contained IOCs ----------------------

VsDataContainer-Multiple:

type: array

items:

$ref: '#/components/schemas/VsDataContainer-Single'

ManagementNode-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagementNode-Single'

MnsAgent-Multiple:

type: array

items:

$ref: '#/components/schemas/MnsAgent-Single'

MeContext-Multiple:

type: array

items:

$ref: '#/components/schemas/MeContext-Single'

Scheduler-Multiple:

type: array

items:

$ref: '#/components/schemas/Scheduler-Single'

ConditionMonitor-Multiple:

type: array

items:

$ref: '#/components/schemas/ConditionMonitor-Single'

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

GenericCollection-Multiple:

type: array

items:

$ref: '#/components/schemas/GenericCollection-Single'

#-------- Definitions in TS 28.623 for TS 28.532 ---------------------------------

resources-genericNrm:

oneOf:

- $ref: '#/components/schemas/NrmRoot'

- $ref: '#/components/schemas/VsDataContainer-Single'

- $ref: '#/components/schemas/ManagementNode-Single'

- $ref: '#/components/schemas/MnsAgent-Single'

- $ref: '#/components/schemas/MeContext-Single'

- $ref: '#/components/schemas/Scheduler-Single'

- $ref: '#/components/schemas/ConditionMonitor-Single'

- $ref: '#/components/schemas/SupportedNotifications-Single'

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/GenericCollection-Single'

<CODE ENDS>

*** END OF CHANGE 3 ***

End of Change