3GPP TSG-SA5 Meeting #159	S5-250605

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0067 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.310 Introduce energy supply, carbon emission and renewable energy information elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR is related to WT-2 (Energy supply, carbon emission and renewable energy information elements) of Energy_OAM_Ph3 WID. Requirements in TS 22.261 (clause 6.15a.5.2) specifies the requirements to expose the energy related information, including ratio of renewable energy and carbon emission information for the 5G system. The Energy Information Function (EIF) newly introduced into TS 23.501 is one of the consumer of gNB and UPF Energy Consumption and Data Volume measurements from OAM, as well as other energy related information such as carbon emission and renewable energy factor. The energy related information includes the energy usage information for group of one or more sub-network(s), one or more managed functions and/or managed elements that are deployed at an operator site or data center or grouped under a sub-network, that are are powered using the same energy sources. Hence to support this use case a the information element where the operator can configure such energy information is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NRM to support this use case is proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator would not be able to configure the energy related information for the managed entities, and hence would not be able to satisfy SA1 requirements specified in TS 22.261. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, X (new), Y (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 28.623 CR 0497, TS 28.622 CR 0518, TS 28.532 CR 0365 |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1599 at commit 0500b068848a8812e7fc37c707b0d600bdf2da17 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-250605 is a revision of S5-250436 |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI ES 203 228: "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[3]	ETSI ES 202 336-1 V1.2.1: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 1: Generic Interface".

[4]	ETSI ES 202 336-12 V1.1.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[5]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[6]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[7]	Void.

[8]	3GPP TS 32.432: "Telecommunication management; Performance measurement: File format definition".

[9]	3GPP TS 32.435: "Telecommunication management; Performance measurement; eXtensible Markup Language (XML) file format definition".

[10]	3GPP TS 32.436: "Telecommunication management; Performance measurement: Abstract Syntax Notation 1 (ASN.1) file format definition".

[11]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[12]	3GPP TS 38.401: "NG-RAN; Architecture description".

[13]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[14]	Void.

[15]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[16]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[17]	3GPP TS 32.551: "Energy Saving Management (ESM); Concepts and requirements".

[18]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[19]	ETSI GR NFV-IFA 015 V3.4.1 (2020-06): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Report on NFV Information Model".

[20]	ETSI GR NFV 003 V1.6.1 (2021-03): "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[21]	3GPP TS 28.530: " Management and orchestration; Concepts, use cases and requirements".

[22]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[23]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

[24]	Void.

[25]	ETSI GS NFV-IFA 027 (V4.3.1) (2022-06): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[26]	ETSI ES 202 336-12 (V1.2.1) (2019-02): "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[27]	ETSI GS NFV-IFA 008 V3.5.1 (2021-11): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[28]	ETSI GS NFV-TST 008 V3.3.1 (2020-12): "Network Functions Virtualisation (NFV) Release 3; Testing; NFVI Compute and Network Metrics Specification".

[29]	ETSI GS NFV-IFA 006 (V3.7.1) (2022-11): " Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Vi-Vnfm reference point – Interface and Information Model Specification".

[30]	3GPP TS 28.111: "Fault management".

[A]	ETSI GS OEU 020: "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[B]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[C]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[D]	ITU-T Recommendation L.1333: "Carbon data intensity for network energy performance monitoring".

[E]	Guidebook "How to Develop a Sustainable Energy Action Plan (SEAP)" [i.6] © European Union, 2010.

Next change

# X	Information model definitions for NR NRM

## X.1	Imported and associated information

### X.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [B], IOC, SubNetwork | SubNetwork |
| TS 28.622 [B], IOC, ManagedElement | ManagedElement |
| TS 28.622 [B], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [B], IOC, Top | Top |
| TS 28.622 [B], IOC, GenericCollection | GenericCollection |
| TS 28.622 [B], ProxyClass, ManagedEntity | ManagedEntity |
| TS 28.622 [B], choice, TimeWindow | TimeWindow |
| TS 28.622 [B], choice, SchedulingTime | SchedulingTime |

### X.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |

## X.2	Class diagram

### X.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for energy efficiency. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

Figure X.2.1-1: Energy infomation group NRM fragment

### X.2.2	Inheritance

This clause depicts the inheritance relationships.

Figure X.2.2-1: Energy supply information NRM fragment

Figure X.2.2-2: Energy information group NRM fragment

Figure X.2.2-3: Energy usage information NRM fragment

## X.3	Class definitions

### X.3.3	EnergySupplyInfo

#### X.3.3.1	Definition

This IOC defines the information related to power supply and its energy mix. This information is configured by the operator with the information obtained from external sources.

The attribute energySupplyMode is used to indicate the source of power supply (see clause 6 of ITU-T Recommendation L.1333[D]).

The attribute energySourceList represents the energy mix information of the energy source. This includes energy related information including carbon and renewable energy related information, and the percentage of the energy produced using the source. The total sum of ratio of energy produced using the sources represented by attribute energyCompositionRatio of EnergySourceInfo) in energySourceList should be equal to 100.

The attribute plannedOutgates is used to indicate the planned outages from the energy supplier.

#### X.3.3.2	Attributes

The EnergyInfoGroup IOC includes the attributes inherited from Top IOC (defined in in TS 28.622 [B]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| energySupplyMode | M | T | T | F | T |
| energySourceList | M | T | T | F | T |
| plannedOutgates | O | T | T | F | T |

#### X.3.3.3	Attribute constraints

None.

#### X.3.3.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.4	EnergySourceInfo <<dataType>>

#### X.3.4.1	Definition

This data type defines the information related to a type of energy source. This information is configured by the operator with the information obtained from external sources.

The attribute energySourceType is used to indicate the type of fuel used by an energy supply to generate the energy. The attribute energySourceCef is used to indicate the carbon emission factor of the energy source. The attribute renewableEnergy is used to indicate if the energy source is renewable or non-renewable. The attribute energyCompositionRatio is used to indicate the percentage of energy source used in the energy supply.

#### X.3.4.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| energySourceType | M | T | T | F | T |
| energySourceCef | M | T | T | F | T |
| renewableEnergy | M | T | T | F | T |
| energyCompositionRatio | M | T | T | F | T |

#### X.3.4.3	Attribute constraints

None.

#### X.3.4.4	Notifications

The clause X.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### X.3.1	EnergyInfoGroup

#### X.3.1.1	Definition

This IOC represents the energy information for the group of one or more sub-network(s), one or more managed functions and/or managed elements that are deployed at an operator site or data center or grouped under a sub-network, that are are powered using the same energy sources.

Attribute memberDNList is used to group the SubNetwork, and/or ManagedElements and/or ManagedFunction MOIs. Attribute energyUsageInfoRefList is used to specify the information related to energy usage by the members of the group.

The energy usage information includes the energy supplies used that represented by attribute energySupplyRef of EnergyUsageInfo, and the ratio of energy consumption from each energy supply, represented by attribute energyConsumptionRatio of EnergyUsageInfo, during the configured time window. The total sum of energy consumption ratio (represented by attribute energyConsumptionRatio of EnergyUsageInfo) from all the energy supplies from the list of energyUsageInfoRefList should be equal to 100.

#### X.3.1.2	Attributes

The EnergyInfoGroup IOC includes the attributes inherited from GenericCollection IOC (defined in in TS 28.622 [B]) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |  |
| --- | --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |  |
| energyUsageInfoRefList | M | T | T | F | T |  |

#### X.3.1.3	Attribute constraints

None

#### X.3.1.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.2	EnergyUsageInfo

#### X.3.2.1	Definition

This IOC defines the energy usage information. This information is configured by the operator with the information obtained from external sources.

The attribute energySupplyRef is used to indicate the source of energy supply and attribute consumptionRatio is used to indicate the percentage of energy consumption using this source of energy supply. The attribute validityPeriod is used to indicate the validity of the information.

#### X.3.2.2	Attributes

The EnergyInfoGroup IOC includes the attributes inherited from Top IOC (defined in in TS 28.622 [B]) and the following attributes:

| Attribute name | S | isReadable |  | isWritable |  |  | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| energyConsumptionRatio | O | T |  | T |  |  | F | T |
| validityPeriod | O | T |  | T |  |  | F | T |
| Attributes related to role |  |  |  |  |  |  |  |  |
| energySupplyRef | M |  | T |  | T | F |  | T |

#### X.3.3.3	Attribute constraints

None.

#### X.3.3.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

## X.4	Attribute definitions

### X.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| energySourceType | The type of energy source. allowedValues: N/A See NOTE. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySourceCef | The Carbon Emission Factor of a type of energy source. Its unit is kg CO2eq / kWh. allowedValues: non-negative integers See NOTE. | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energyConsumptionRatio | The proportion of the energy provided by the type of energy supply over the total energy consumed by the members of the group during the configured validity time period. This information is configured by the operator with the information obtained from external sources. allowedValues: 1, …,100 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 100 isNullable: False |
| renewableEnergy | This attribute represents if the energy source is renewable or non-renewable. TRUE: the energy souce is renewable. FALSE: the energy souce is non-renewable. This information is configured by the operator with the information obtained from external sources. Its is the responsibility of the Operator to decide which type(s) of energy source can be considered as renewable. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| validityPeriod | Date and time of the start and end of the period of validity of the attribute values of the EnergyUsageInfo. This information is configured by the operator with the information obtained from external sources. | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySupplyMode | The mode by which energy is supplied to a managed entity. This information is configured by the operator with the information obtained from external sources. The mode can be one of the following: GRID_ELECTRICITY: Most of the energy consumed comes from local grid electricity (See Grid electricity in clause 6 of ITU-T Recommendation L.1333 [D]) BACKUP_ENERGY: A small proportion of energy comes from backup diesel or other types, or from generators such as solar panels. (See Backup energy in clause 6 of ITU-T Recommendation L.1333[D]) LOCALLY_GENERATED_ENERGY: energy provided by local generators not used for backup functionality, e.g., solar, wind generator. (See Locally generated energy in clause 6 of ITU-T Recommendation L.1333[D]) allowedValues: - “GRID_ELECTRICITY” - “BACKUP_ENERGY” - “LOCALLY_GENERATED_ENERGY” | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energyCompositionRatio | The ratio of energy source type used by the energy supply. This information is configured by the operator with the information obtained from external sources. allowedValues: 1, …,100 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 100 isNullable: False |
| energyUsageInfoRefList | The energy usage information for the members of the group from each energy supply source. This information is configured by the operator with the information obtained from external sources. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: N/A isNullable: False |
| plannedOutgates | This indicates the planned outages of an energy supply. | type: SchedulingTime multiplicity: * isOrdered: False isUnique: True defaultValue: N/A isNullable: False |
| energySupplyRef | The energy usage information for the members of the group from each energy supply source. This information is configured by the operator with the information obtained from external sources. | type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: N/A isNullable: False |
| NOTE: The example of energy source and their standard emission factor values can be found in the tables A.2 and A.3 of informative Annex A, in ETSI GS OEU 020 [A], where the Tables A.1 to A.3 correspond tables 5, 6 and 7 extracted from Guidebook "How to Develop a Sustainable Energy Action Plan (SEAP)" [E] © European Union, 2010. Reproduction is authorized provided the source is acknowledged. |  |  |

## X.5	Common notifications

## X.5	Common notifications

### X.5.1	Alarm notifications

This clause presents a list of notifications, defined in TS 28.111 [30], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | O | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

NOTE:	notifyChangedAlarm is deprecated in TS 28.111 [30].

### X.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [16], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyMOIChanges | O | -- |
| notifyEvent | O | -- |

### X.5.3	Threshold Crossing notifications

This clause presents a list of notifications, defined in TS 28.532 [16], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyThresholdCrossing | M |  |

Next change

# Y	Stage 3 definitions

## Y.1	OpenAPI document for Energy Efficiency NRM

The OpenAPI/YAML definitions for Energy Efficiency NRM are specified in 3GPP Forge, refer to clause 4.3 of TS 28.623 [C] for the Forge location. An example of Forge location is: "https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel19_SA107/".

Directory: OpenAPI

File: TS28310_EnergyEfficiency.yaml

Next change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1599 at commit 0500b068848a8812e7fc37c707b0d600bdf2da17

*** START OF CHANGE 1 ***

*** OpenAPI/TS28310_EnergyEfficiency.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: EE NRM

version: 19.0.0

description: >-

OAS 3.0.1 specification of the Energy Efficiency NRM

Â© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.310; Energy Efficiency NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.310/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

EnergySourceInfo:

type: object

properties:

energySourceType:

type: string

energySourceCef:

type: integer

renewableEnergy:

type: boolean

energyCompositionRatio:

type: integer

default: 100

minimum: 0

maximum: 100

EnergySupplyMode:

type: string

enum:

- GRID_ELECTRICITY

- BACKUP_ENERGY

- LOCALLY_GENERATED_ENERGY

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-ncO-EENrm:

type: object

properties:

EnergySupplyInfo:

$ref: '#/components/schemas/EnergySupplyInfo-Multiple'

EnergyInfoGroup:

$ref: '#/components/schemas/EnergyInfoGroup-Multiple'

EnergyUsageInfo:

$ref: '#/components/schemas/EnergyUsageInfo-Multiple'

EnergySupplyInfo-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

energySupplyMode:

$ref: '#/components/schemas/EnergySupplyMode'

energySourceList:

$ref: '#/components/schemas/EnergySourceInfo'

plannedOutgates:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SchedulingTime'

EnergyInfoGroup-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/GenericCollection-Single'

- type: object

properties:

attributes:

type: object

properties:

energyUsageInfoRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

EnergyUsageInfo-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

energyConsumptionRatio:

type: integer

default: 100

minimum: 0

maximum: 100

validityPeriod:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

energySupplyRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

EnergySupplyInfo-Multiple:

type: array

items:

$ref: '#/components/schemas/EnergySupplyInfo-Single'

EnergyInfoGroup-Multiple:

type: array

items:

$ref: '#/components/schemas/EnergyInfoGroup-Single'

EnergyUsageInfo-Multiple:

type: array

items:

$ref: '#/components/schemas/EnergyUsageInfo-Single'

#--------------------------------- Definition ------------------------------------

resources-EnergyEfficiency:

oneOf:

- $ref: '#/components/schemas/EnergySupplyInfo-Single'

- $ref: '#/components/schemas/EnergyInfoGroup-Single'

- $ref: '#/components/schemas/EnergyUsageInfo-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

End of change