3GPP TSG-SA5 Meeting #159	S5-250622

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.536 | CR | 0120 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.536 Correct association between various classes and their datatypes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The guideline for drawing UML diagram TS 32.156 was updated for release 18. Upon inspection of the ACCL UMl diagram the following observatnios where made: - The UML diagram for the ACCL NRM shows the relationship between various classes (respectively NetworkSlice, NetworkSliceSubnet and AssuranceGoal) and various attributes (respectively serviceProfile, sliceProfile and assuranceTarget). - The various attributes are defined as dataTypes. The relationship in the diagram is shown as an arrow pointing to the dataType (as if it is a class). - The use of an arrow between a class and a dataType is not common practice in UML and does not provide any additional information over and above an ordinary line (without arrow). It is therefore proposed to remove the arrows pointing to dataTypes from the UML diagrams from Rel-18 onwards. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UML diagram for various classes is updated by replacing the line with arrow between class and dataType with line without arrow. The following are updated: - NetworkSlice and ServiceProfile - NetworkSliceSubnet and SliceProfile - AssuranceGoal and AssuranceTarget - AssuranceReport and AssurancGoalStatus - AssuranceGoalStatus and AssuranceTargetStatus |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous relationship diagram may lead to interoperability problems |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.2.1, Figure 4.1.2.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No stage 3 impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 4.1.2.2	Class diagram

#### 4.1.2.2.1	Relationships

This clause depicts the set of classes that encapsulates the information relevant for this MnS. This clause provides an overview of the relationships between relevant classes in UML. This clause provides an overview of the relationships between relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

Figure 4.1.2.2.1.1: Assurance management NRM fragment

#### 4.1.2.2.2	Inheritance

Figure 4.1.2.2.2.1: Assurance management inheritance relationships

Second change

Annex D (informative):
Appendix with UML code for model diagrams

# D.1	UML code for Figure 4.1.2.2.1.1

@startuml

skinparam backgroundColor white

skinparam classBackgroundColor white

skinparam classBorderColor black

skinparam Shadowing false

skinparam noteBackgroundColor white

skinparam noteBorderColor white

skinparam arrowColor black

hide circle

hide members

class SubNetwork <<InformationObjectClass>>

class ManagedElement <<InformationObjectClass>>

class AssuranceClosedControlLoop <<InformationObjectClass>>

class AssuranceReport <<InformationObjectClass>>

class AssuranceGoal <<InformationObjectClass>>

class NetworkSlice <<InformationObjectClass>>

class NetworkSliceSubnet <<InformationObjectClass>>

class ServiceProfile <<dataType>>

class SliceProfile <<dataType>>

class AssuranceTarget <<dataType>>

class AssuranceGoalStatus <<dataType>>

class AssuranceTargetStatus <<dataType>>

SubNetwork "1" *-- "*" AssuranceClosedControlLoop: <<names>>

ManagedElement "1" *-- "*" AssuranceClosedControlLoop: <<names>>

AssuranceClosedControlLoop "1" *-- "*" AssuranceGoal: <<names>>

AssuranceReport "1" -left-* "1" AssuranceClosedControlLoop: <<names>>

AssuranceReport "1" -- "*" AssuranceGoalStatus

AssuranceGoalStatus "1" -- "*" AssuranceTargetStatus

AssuranceGoal "1" -- "*" AssuranceTarget

AssuranceClosedControlLoop "*" --> "1" NetworkSlice

NetworkSlice "1" -- "*" ServiceProfile

AssuranceClosedControlLoop "*" --> "1" NetworkSliceSubnet

NetworkSliceSubnet "1" -- "*" SliceProfile

note "{xor}" as Note1

Note1 .. (SubNetwork, AssuranceClosedControlLoop)

Note1 .. (ManagedElement, AssuranceClosedControlLoop)

@enduml

End of changes