3GPP TSG-SA5 Meeting #159	S5-250655

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0263 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.105 Add use case for Management of Reinforcement Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson AB |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update TS 28.105 to include use REL-19 use case for Management of Reinforcement Learning studied in TR 28.858. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a usecase for reinforcement learning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Management of Reinforcement Learning will not be supported in TS 28.105. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3, 6.Y (new), 6.Y.1 (new), 6.Y.2 (new), 6.Y.2.1 (new), 6.Y.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ML model: a manageable representation of an ML model algorithm.

NOTE 1:	an ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output.

NOTE 2:	ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification.

NOTE 3:	ML model may include metadata. Metadata may include e.g. information related to the trained model, and applicable runtime context.

ML model training: a process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model.

ML model initial training: a process of training an initial version of an ML model.

ML model re-training: a process of training a previously trained version of an ML model and generate a new version.

NOTE 4:	a new version of a trained ML model supports the same type of inference as the previous version of the ML model, i.e., the data type of inference input and data type of inference output remain unchanged between the two versions of the ML model, but parameter values might be different for the re-trained model.

ML model joint training: a process of training a group of ML models.

ML training function: a logical function with ML model training capabilities.

ML model testing: a process of evaluating the performance of an ML model using testing data different from data used for model training and validation.

ML model joint testing: a process of evaluating the performance of a group of ML models using testing data different from data used for model training and validation.

ML testing function: a logical function with ML model testing capabilities.

AI/ML inference: a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

NOTE 5:	the inference represents the process to realize the AI capabilities by utilizing a trained ML model and other AI enablers if needed, hence the AI/ML prefix is used when referring to inference as compared to training and testing.

AI/ML inference function: a logical function that employs trained ML model(s) to conduct inference.

AI/ML inference emulation: running the inference process to evaluate the performance of an ML model in an emulation environment before deploying it into the target environment.

ML model deployment: a process of making a trained ML model available for use in the target environment.

ML model loading: a process of making a trained ML model available to an inference function.

AI/ML activation: a process of enabling the inference capability of an AI/ML inference function.

AI/ML deactivation: a process of disabling the inference capability of an AI/ML inference function.

Reinforcement Learning: a machine learning approach where an RL agent learns to make decisions by interacting with the environment and taking actions to maximize cumulative rewards.

NOTE:	The reward is a score calculated based on the state of the environment, neither reward nor state are subject for standardization.

Next change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and TS 28.533 [15]. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 28.533 [15].

AI	Artificial Intelligence

ML	Machine Learning

RL	Reinforcement Learning

Next change

### 6.Y	Management of Reinforcement Learning

#### 6.Y.1	Description

Reinforcement Learning (RL) is a type of machine learning where an agent learns to make decisions by taking actions in an environment to achieve a goal. The agent learns from the consequences of its actions, rather than from being explicitly taught and it selects its actions based on its past experiences (exploitation) and also by new choices (exploration), which is essentially a trial-and-error approach. In RL process, an agent interacts with the environment in discrete time steps. At each time step, the RL agent receives the current state of the environment and selects actions to be taken. The environment provides the new state in turn and the RL agent will receive a reward based on the state of the environment. For example, RL is applied for coverage problem analysis use case, the RL agent actions are used to optimize the coverage problem, and the RL environment can be the simulation environment. The actions can be changes to the values of network adjustable parameters (e.g. change the transmission power of the NR sector carrier, see TS 28.104 [2]). The states can be the network PMs/KPIs like RSRP distribution etc. The reward can be the score of an RL performance metric to evaluate the PMs/KPIs. The goal of the agent is to learn a policy, which tells it what action to take under what circumstances, that maximizes the sum of rewards. The main advantage of this approach is in the ability to automatically adapt to the characteristics of the environment, making it suitable approach for handling dynamic environments such as mobile networks.

Figure 6.Y.1-1: Example diagram of Reinforcement Learning

#### 6.Y.2	Use cases

##### 6.Y.2.1	Exploration in Reinforcement Learning

Reinforcement Learning (RL) has the ability to learn and adapt itself to dynamic environments and thus finds the near optimal solution to the problem. This makes the RL-based approaches very interesting for applications in the mobile networks. However, the potential negative impact to the mobile network caused by RL is still the main drawback. In particular, during the exploration step performing trials and learning from errors may have an impact on the operational network and may result in unsafe operations causing network performance degradations. Therefore, the exploration step in RL needs to be under a controlled environment or a stable training configuration that is not supposed to violate system performance requirements. If the RL agent behaves in an unexpected manner, there needs to be a set of fall-back actions in place, a set of fall-back actions might be needed, e.g. to switch from RL-based solution to non-RL-based solution, to fall back to last discrete time step, to roll back to the previous configuration and to terminate the RL process. These actions are implementation specific and not a subject for standardization.

For RL management, the MnS consumer can query the ML Training MnS producer to discover if RL is supported. In the event RL is supported, the consumer may also want to provide a scope (e.g. geographical area, time window) that can aid the producer to select/create the environment when performing RL.

Next change

### 6.Y.3	Requirements for Management of Reinforcement Learning

Table 5.Y.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-RL_MGMT-01 | The ML training MnS producer should have a capability allowing an authenticated and authorized MnS consumer to query if RL training is supported. | Management of Reinforcement Learning (clause 6.Y.1) |
| REQ-RL_MGMT-02 | The ML training MnS producer should have a capability to allow an authenticated and authorized MnS consumer to specify the preferred RL environment scope. | Management of Reinforcement Learning (clause 6.Y.2.1) |
| REQ- RL_MGMT-03 | The ML training MnS producer should have a capability to allow an authenticated and authorized MnS consumer to specify RL configuration scope for exploration in reinforcement learning. | Management of Reinforcement Learning (clause 6.Y.2.1) |

End of changes