3GPP TSG-SA5 Meeting #159	S5-250679

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	Discussion paper on deployment scenarios relevant for modelling of energy-related data

# Document for:	Discussion

Agenda Item:	6.19.20.1

# 1	Decision/action requested

Discussion and recommendation for modelling.

# 2	References

[1]	ITU-T L series: “Environment and ICTs, climate change, e-waste, energy efficiency; construction, installation and protection of cables and other elements of outside plant”

[2]	ITU-T L.1200-L.1299: “Power feeding and energy storage”

[3]	ITU-T L.1205: “Direct current power feeding interface up to 400 V at the input to telecommunication and ICT equipment”

[4]	ETSI EN 300 132: multi-part deliverable (1 – 3) covering “Environmental Engineering (EE); Power supply interface at the input to telecommunication and datacom (ICT) equipment”

[5]	ETSI ES 202 336: Multi-part deliverable 1 – 12 covering “Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks)”

[6]	ETSI ES 202 336-12: “Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model”

# 3	Rationale

The 3GPP management system shall provide a management interface that enables MnS consumers to obtain a comprehensive view on the managed entities and services of the 3GPP network and the related consumption of energy, e.g. as input for the Energy Information Function (EIF) as defined by 3GPP SA2.

Such a management interface for energy-related information has to cover the many possible deployment variants that are in use to power equipment of information and communication technologies (ICT) in field.

This discussion paper presents some aspects of the power supply system and its management that have been standardized by ITU-T [1 - 3] and ETSI [4 - 6] (disclaimer: by no means exhaustive), which are defining the concrete infrastructure of telecommunication equipment like the -48V DC or 400V DC power supplies, concrete cabling, concrete placement of meter devices, and concrete measurements.

These concrete metering devices and measurements are used by a dedicated management architecture that is unaware of network elements and network functions. In contrast, the 3GPP management system is concerned with telecommunication aspects of network functions, while basically unaware of the corresponding power supplies.

This paper will discuss in how far the above mentioned standards from ITU-T and ETSI might help to integrate energy-related data from power supplies in an abstract way into the network resource model in order to offer MnS consumers a harmonized, abstracted, and common view on energy-related aspects and network-related aspects.

# 4	Detailed proposal

## 4.1	Background on energy supplies for telecommunication equipment

A management interface to handle energy information has to cover all the many possible variants that are used to power information and communication technology (ICT) equipment in field. The power supply of ICT equipment in telecommunication or data centers in relation to safety, functional performance and EMC is normatively defined e.g. by ITU-T K series, ITU-T L.12xx series [1-3] and ETSI EN 300 132 series [4].

Measuring and controlling the power supply system is by standards according to the ETSI ES 202 336 series [5], including corresponding management interfaces [6]. These standards also describe how the management system responsible for the overall power supply system may interact with the network management system or operations support system responsible for the telecommunication equipment and provided telecommunication services (Figure 2, from [6]).

Operators experience the demand for more detailed reports on the consumed energy and need more detailed information in order to use the energy more efficiently. At the same time, the formerly simple task to report power consumption is getting very complex due to the adoption of self-generated power (e.g. solar cells) on site and the multiple primary energy source that a grid operator is able to use, as well as due to the possibilities of storing and releasing energy by batteries either towards managed entities or back towards the grid.

The power supply system, especially in complex base station sites, might consist of several power sources (grid, renewables), several storage devices, power distribution units and internal micro-grids, and a multitude of power consumers. Some of the consumers are managed by a 3GPP management system; probably many of the 3GPP-related consumers are powered according to ETSI EN 300 132-2 interface "A" (i.e. -48V DC) or ETSI EN 300 132-3 interface "A3" (i.e. 400V DC) [4]. Some example figures copied from Annex A of ITU-T L.1205 [3] to show a variety of set ups; for detailed descriptions of the scenarios refer to ITU-T L.1205 [3]:

Fig A.1 (numbers maintained from [3] to ease reference) shows how the electricity supplied by the high-voltage AC grid is transformed to the regular low-voltage AC grid, that is use by the site. A main distribution board collects and distributes the electricity from/to the different sources and consumers. In example ITU-T Fig A.1 all sources of renewable energy are coupled into the system via AC, such that e.g. the electricity from the photo voltaic (PV) solar panels might be sold back to the grid. The DC outputs from the distribution frame on the right-hand side basically represent the feed for the telecommunication devices shelfed in the racks, either -48V DC or 400V DC. The schematic also shows AC feeds for other ICT equipment or the equipment of the building (cooling, heating, light, etc).

The example shown by [3] fig A.2 is similar as the above, yet all sources of renewable energy are coupled into the system via DC into 400V DC system. In this case the renewable energy cannot (easily) exported back to the grid and it cannot be used to power other devices on site.

[3] also describes more complex hybrid cases like shown in fig A.7.

Already today basically all flows of electricity are measured and managed in order to keep the system stable, e.g. to maintain the frequency of the grid, to control charging and discharging of batteries, and to optimize the usage of renewable energy. However, the corresponding management systems are not aware of the managed elements and managed functions that are using the energy. On the other hand, the 3GPP management systems have very limited awareness of the electricity used to operate the network.

In order to provide a comprehensive, common picture about the managed elements, managed functions, the telecommunication services and the related consumption of energy, sources of primary energy (e.g. coal, nuclear, solar), and on the environmental impacts (e.g. carbon emission factors) the 3GPP management system has to cope with several aspects as follows:

Variety of energy and energy supplies:

- The entities managed by a 3GPP management system are not able to measure all required information. Some can be measured but needs to be imported from other sources, while others can be regarded semi-static meta data that needs to be provided by other sources:

  - Measurements of in-built PEE sensors: Most probably all physical managed elements of a 3GPP network (i.e. devices shelfed in the rack, shown at the right hand side of the figures) are able to meter and to report their energy usage by built-in PEE sensors.

  - Measurements to be integrated from other management systems: Even a simple base station site consists of routers, power amplifiers, heating, cooling, and other devices that all are not managed by 3GPP but nevertheless are consuming a considerable amount of energy. As mentioned above, any energy generated, imported, or exported locally is metered and managed by dedicated systems. The energy usage by these systems need to be consumed by the 3GPP management system.

    - Although all the different flows of energy are measured, it is not trivial to attribute the used energy to the different consumers, and even more complicated to attribute them to the different telecommunication services.

    - Depending on the source of energy that was used to charge a battery it requires thorough book keeping to assign a primary energy to the electricity from a battery.

  - Semi-static meta data: Energy provided by the grid might have been generated by a mix of primary energy sources (coal, gas, nuclear, wind, solar, etc.). Neither the meters on site nor the in-built PEE sensors of managed elements are able to infer the information about this mix. Such information must be provided to the 3GPP management system via management interfaces. Depending on the use case and the grid operator these data are provided as statistics over longer periods, e.g. every six months with the invoice.

  - Unclear availability of data: VNF are not aware of the energy that their hosting cloud is using at all, neither the amount nor the source. It depends on the operator of the cloud in how far he is able and willing to expose such information.

Variety of deployments in field:

- A "telecommunication site" or "operator site" is not well-defined, and probably never will be due to the many possibilities to deploy equipment and functions, to run them virtually in a cloud, to share them, or to keep them separated for redundancy or privacy reasons.

## 4.2	Requirements for the model

### 4.2.1 Introduction

The model representing the system on the management interfaces needs to cope with the flexibility provided by the many deployment options of the energy supply system in combination with the many deployments options of telecommunication equipment as such.

### 4.2.2	Role of ManagedElement and ManagedFunctions

Telecommunication equipment (e.g. a “box” in a rack) might be represented in an abstract way by the IOC ManagedElement. On one hand, ManagedElement is hosting one or many ManagedFunctions (i.e. network functions like gNBCU-CP, gNBCU-UP, gNB-DU, AMF, UPF, etc). On the other hand, ManagedElement abstracts from the detailed hardware -this link is important since energy is consumed by HW. Therefore, a ManagedElement is a good candidate to measure the energy consumption of all its HW units.

A fair partitioning and attribution of the overall energy consumed by HW that is shared by serving multiple ManagedFunctions is not trivial. This is a highly political discussion which is out of scope of SA5, not discussed further here.

An gNB might be implemented as an “all-in-one” gNB that runs in one set-top box gNB-CU-UP, gNB-CU-CP, gNB-DU, and the active RF parts antennas. In contrast, a big macro gNB might be distributed across many sites: gNB-CU-UP and gNB-CU-CP are running on one common HW, many gNB-DU are distributed to several sites in a large area, and each gNB-DU serves many radio units.

From modeling point of view, the “all-in-one” gNB might be represented by one ManagedElement that name-contains GNBCUUPFunction, GNBCUCPFunction, GNBDUFunction, and NRSectorCarrier. This ManagedElement is able to meter its electricity usage by its inbuilt PEE sensor as sum of all its HW parts. The ManagedElement is not able to infer which other consumers or generators of electricity are co-located in the household. It’s not able to infer the mix of primary energy, nor is this information inherent to a ManagedElement, except somebody associates this as additional semi-static meta data, which probably is valid for the household or even the whole neighborhood.

The big macro gNB will be represented by many ManagedElements. E.g. one ManagedElement that name-contains GNBCUUPFunction and GNBCUCPFunction. Each of the many gNB-DU might be represented by one ManagedElement that name-contains one GNBDUFunction, and the many, many radio units each are represented by one ManagedElement that name-contains one NRSectorCarrier each. Each of these ManagedElements is able to meter its own energy usage by inbuilt PEE sensors, potentially even for mutiple (redundant) supplies. By their PEE sensors the ManagedElements are not able to differentiate between different sources of primary energy. None of the ManagedElements is aware of other consumers or producers of energy on the same site. They are not able to infer the mix of primary energy, except somebody associates this as semi-static meta data.

### 4.2.3	Site

Most network deployments co-locate cells of multiple RAT at the same site. The site offers electricity from the grid, might be equipped with diesel generators, generators of renewable energy, backup batteries etc. Each of these sources is metered on site. The provider of each source might inform about the mix of primary energy the source is using.

The current flowing into the site or getting exported from the site is the sum of all equipment on site, be it generators or consumers of energy. To enable the MnS consumer to identify equipment that is metered by the same meter (i.e. using the same power supply), the management interface needs a means to group the corresponding equipment. This grouping –probably- differs from partitioning of the network into SubNetworks. Since name-containment allows for one containing entity, such grouping of equipment per site cannot use name-containment.

## 4.3	Concept for a model

The model needs to allow the MnS consumer to associate the energy usage to corresponding groups of ManagedElements (Site) or individual ManagedElements.

The corresponding PEE sensors are able to measure the usage of electricity, but are not able to infer which kind of primary energy was used to supply the electricity. This is semi-static meta-data that needs to be configured manually or might be retrieved from the grid operator with time intervals as provided by the grid operator, e.g. as part of the monthly or yearly invoice.

To allow a MnS consumer to read and to set the mix of primary energy that was used to supply the electricity and related Carbon Emission Factors, the model needs a possibility to associate corresponding information about the energy supplier as meta-data to the energy usage. Since depending on the situation in a certain deployment this information might be the same for a SubNetwork, a group of ManagedElements (i.e. a "site"), or an individual ManagedElement, the model should allow for the different name-containment depending on the situation.