3GPP TSG-SA5 Meeting #159	S5-250703

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Ericsson

# Title:	Business scenario updates

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TR 28.846 Study on charging aspects of satellite access Phase 3

[2]	3GPP TR 23.700-29 Study on integration of satellite components in the 5G architecture; Phase 3

# 3	Rationale

The architecture in the TR 23.700-29 [2] are only informative, this means that other architectures are possible and the business scenarios should try to be architecture independent.

# 4	Detailed proposal

First change

## 5.2	Business scenarios

### 5.2.1	S&F operation

Business scenario#1a: S&F operation

A UE with the store and forward capability attaches to the satellite in the store and forward mode, the data could be stored on the satellite when the feeder link is not available. When the feeder link is available, the UE data could be forwarded to the ground DN. Several satellites may be involved to finish the attach procedure or data transmission procedure.

An MNO rents satellites to deploy a full EPC or part of (e.g.  eNB and part of MME functions) from an SSP, to be able to provide service to its customer. This means that there would be a wholesale agreement between MNO and SSP, where the MNO is the charged party and the SSP is the charging party. The MNO could be charged by SSP based on the usage of involved satellites.

An end user has a subscription with the MNO which allows it to use its services including the satellite store and forward service. The subscriber is charged party and the MNO is the charging party.

### 5.2.2	Roaming between MNOs with and without satellite networks

Business scenario#2a: Roaming between MNOs with and without satellite networks.

Subscriber of an MNO without satellite network roams into an MNO network with satellite network. The MNO without satellite network has a wholesale agreement with the MNO with satellite network where the MNO without satellite network would be the charged party and the MNO with satellite network would be the charging party for this scenario. The SCC can have subscription with the MNO that allows roaming into the MNO with satellite network. The SCC subscriber would be charged by the MNO, while the MNO with satellite network would do interconnect charging towards the MNO without satellite network. Likewise, an SCC can have a subscription with an MNO with satellite network allowing it to roam into an MNO without satellite network, where the MNO without satellite network would do interconnect charging towards the MNO with satellite network and the MNO with satellite network would charge the SCC.

### 5.2.3	5G satellite backhaul without UPF on-board

Business scenario#3a: 5G Satellite Backhaul.

SSP can lease satellites to MNO that can be used as part of the backhaul between (R)AN and 5GC. An MNO use satellites from SSP to achieve satellite backhaul. MNO could be charged by SSP based on the total data volume transferred via the satellite. In this scenario, UPF is not deployed on the satellite.

### 5.2.4	5G satellite backhaul with UPF on-board

Business scenario#3b: Edge Computing.

SSP can lease satellites to MNO that can be used to deploy, UPF on satellite. An MNO can use satellites from SSP to achieve edge computing via UPF deployed on the satellite. MNO could be charged by SSP based on usage of satellite per EAS related to EAS deployment (EAS deployment, EAS modification, EAS termination) and infrastructure resource (virtual CPU usage, virtual memory usage, virtual disk usage, data volumes).

### 5.2.5	MVNO which provide satellite communication services

Business scenario#4a: MVNO providing satellite communication services

MVNO rents satellite network resource from MNOs and then provide satellite communication services to its subscribers, i.e. it allows the subscribers usage of 5G data connectivity while in the host MNO.

The host MNO will collect charging information related to 5G data connectivity usage for charging the MVNO, and may collect charging information related to the 5G data connectivity usage for MVNO subscribes (per UE) and convey this to the MVNO. MVNO could be charged by host MNO based on the total data volume or other types of resource usage.

### 5.2.6	Business scenarios for UE-satellite-UE communication

Business scenario#5a: UE-satellite-UE communication

An end user has a subscription with the MNO which allows it to use its services including the UE-satellite-UE communication. The subscriber is charged party and the MNO is the charging party. An MNO rents satellites from an SSP, to be able to provide service to its customer. This means that there would be a wholesale agreement between MNO and SSP, where the MNO is the charged party and the SSP is the charging party. The MNO could be charged by SSP based on the traffic and the usage of involved satellites.

End of changes