3GPP TSG-SA5 Meeting #159	S5-250705

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Ericsson

# Title:	New solution for topic 11

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree a way forward.

# 2	References

[1]	3GPP TR 28.846 Study on charging aspects of satellite access Phase 3

[2]	3GPP TR 23.700-29 Study on integration of satellite components in the 5G architecture; Phase 3

# 3	Rationale

New solution for

# 4	Detailed proposal

First change

#### 6.1.4.3	Solution #1.3: MME CDF/CGF based solution for S&F operation charging with User Plane CIoT

This solution which relying on EPC CDF/CGF for store and forward satellite operation charging, addresses the Key Issue#1.1 and Key Issue#1.2.

As specified in the TS 23.401 [5], only the whole EPC on-board architecture support using the User Plane CIoT EPS optimisation to transfer data.

UE using satellite access with S&F operation indication in the initial attach, or the service request procedure can only be known by MME, CDR generation in MME can be used to charge the S&F operation of the User Plane CIoT service. For the whole EPC on-board architecture, the CDF/CGF may be deployed on the satellite. The high level of the whole EPC on-board architecture is the same as the Figure 6.1.4.1-2 of the solution#1.2.

The MME on-board reports charging information to CGF or Billing Domain about satellite access running in S&F mode with the following trigger events:

-	Attach Complete via the MME onboard for the CIoT UP Optimizations;

-	Service Request via the MME onboard for the CIoT UP Optimizations.

The charging solution is similar with the solution#1.2 except for all the devices are on the satellite.

Since the MME cannot obtain the store duration and store data volume, the charging information would be:

-	RAT type extends to include "WB-E-UTRAN(LEO)", "WB-E-UTRAN(MEO)", " WB-E-UTRAN(GEO)", " WB-E-UTRAN(OTHERSAT)", "NB-IoT(LEO)", "NB-IoT(MEO)", "NB-IoT(GEO)", "NB-IoT(OTHERSAT)", "LTE-M(LEO)", "LTE-M(MEO)", "LTE-M(GEO)" and "LTE-M(OTHERSAT)"

-	Satellite information

-	Store and Forward indicator-	S&F Monitoring list (i.e. Satellite IDs)

-	Satellite Access Indicator

End of changes