3GPP TSG SA WG5 Meeting #159 	S5-250711

Sophia-Antipolis, FRANCE 17 - 21 February 2025

# Source:	CATT

# Title:	Add charging solutions for UE-SAT-UE communication

# Document for:	Approval

# Agenda Item:	7.5.1

1	Decision

The group is asked to discuss and agree on the proposal.

2	References

[1]		3GPP TR 28.846: " Study on charging aspects of satellite access Phase 3 ".

3	Rationale

According to the IMS converged charging architecture specified in the TS 32.260, the IMS Nodes that can embed the CTF are the MRFC, IMS-GWF (connected to S-CSCF using ISC) and SIP AS. According to the procedure specified in the TS 23.228, the IMS-AGW can obtain the satellite IDs from the P-CSCF for the UE-SAT-UE communication. The  P-CSCF sends the message to the IMS system including S-CSCF and IMS-GWF. Therefore, this solution proposes to use IMS-GWF to trigger to charge UE with the UE-SAT-UE communication service.

4	Detailed proposal

The following changes are proposed to be incorporated into the new TR.

1st Modified Section

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TR 23.700-29: "Study on integration of satellite components in the 5G architecture; Phase 3".

[4]	3GPP TR 28.844: "Study on charging aspects of satellite in the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TR 22.822: "Study on using Satellite Access in 5G ".

[7]	3GPP TS 32.251: " Packet Switched (PS) domain charging".

[8]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

[9]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.228: " IP Multimedia Subsystem (IMS); Stage 2".

[12]	3GPP TS 32.253: "Control Plane (CP) data transfer domain charging".

[13]	3GPP TS 32.299: "Diameter charging applications".

[14]	3GPP TS 32.298: "Charging Data Record (CDR) parameter description".

[x]	3GPP TS 32.260: "IP Multimedia Subsystem (IMS) charging".

[y]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP)

2nd Modified Section

6.4.4	Possible solutions

6.4.4.x	Solution #4.x: IMS-GWF Charging Trigger Function (CTF) based solution for UE-SAT-UE communication

This solution which relying on IMS coverged Charging System for UE-SAT-UE communication, addresses the Key Issue#1.1 and Key Issue#1.2.

As specified in the TS 23.228 [11], if UE-Satellite-UE communication in IMS is supported, the P-CSCF receives from the PCF the identifier of the satellite serving the UE and receives from the P-CSCF serving the remote UE the identifier of the satellite serving the remote UE via the IMS AGW on ground. The P-CSCF uses this information to determine whether or not activate optimized media routing. If the P-CSCF determines the activation of optimized media routing is possible, the P-CSCF releases the IMS AGW on ground when needed and select instead an IMS AGW on satellite.

For the session procedure, the Figure 6.4.4.x-1 describes the high level charging procedure for IMS-GWF (CTF) charging for UE-SAT-UE communication.

Figure 6.4.4.x-1: Charging message flow for UE-SAT-UE session establishment procedure

1.	UE A sends a SIP INVITE request, containing an initial SDP offer, to P CSCF A. The access information in the SIP INVITE request indicates that the UE is accessing from NR satellites access

2.	After receiving the SIP INVITE request indicating NR satellite access, if UE A is in the non-roaming scenario and the P-CSCF A supports UE-Satellite-UE communication, it may perform following actions to determine whether UE-satellite-UE communication is possible:

-	P-CSCF A retrieves Access Network Information from PCF based on the mechanism defined in TS 23.503 [95] and determines whether the UE A is using regenerative satellite access or not based on whether a serving satellite identification is obtained as part of the Access Network Information or not.

3.	The P-CSCF A allocates an IMS-AGW A (e.g. on the ground). P-CSCF A interacts with IMS-AGW A to allocate transport resources. The P-CSCF A sends the support of UE-Satellite-UE communication to the whole IMS functions including S-CSCF and IMS-GWF.

4.	The IMS-GWF A sends a Charging Data Request[Initial] to the CHF A to record start of a user session. And start of a media component in the CHF CDR including AGW on the ground indication, AGW IP address information, AGW on the ground start time stamp for UE-SAT-UE communication.

5.	The CHF A acknowledges the reception of the data and opens a CDR. P-CSCF sends a SIP MESSAGE to IMS AS to request it to send SIP re-INVITE to the terminating network and then towards the UE in the originating network after receiving the SDP answer from the terminating network. This SIP message contains the IP address allocated in IMS AGW on the target satellite to be used by the terminating network as the destination of media traffic. This SIP message also contains the satellite ID of the target satellite. IMS AS sends SIP re-INVITE to the terminating network. This SIP re-INVITE contains an SDP offer that has the IP address allocated in IMS AGW on the target satellite to be used by the terminating network as the destination of media traffic. This SIP re-INVITE also contains a SIP header for conveying the satellite ID of the target satellite.

6.  Same procedure as the steps 5-10 in Figure AE.5.1-1 of TS 23.228[11]. The P-CSCF A sends the satellite ID to the whole IMS functions including S-CSCF and IMS-GWF.

7. The IMS-GWF B sends a Charging Data Request[Initial] to the CHF B to record start of a user session for UE B. And start of a media component in the CHF CDR including AGW on the ground indication, AGW IP address information, AGW on the ground start time stamp for UE-SAT-UE communication.

8.	The CHF B acknowledges the reception of the data and opens a CDR.

9.	More SIP signalling as the step 11-15 in the TS23.228 [11]. P-CSCF A determines to perform UE-satellite-UE communication and determines to change IMS-AGW from A (e.g. on the ground) to A’ (e.g. on the satellite).

10. P-CSCF A interacts with IMS-AGW A’ on satellite to allocate transport resources for both IMS access and core network sides. P-CSCF A also releases the IMS-AGW A on ground.

11.	The IMS-GWF A sends a Charging Data Request[Update] to the CHF A to record update of a user session. And update of a media component in the CHF CDR including AGW on the ground indication, AGW IP address information, AGW on the ground end time stamp. The IMS-GWF A’ sends a Charging Data Request[Update] to the CHF A to record update of a user session. And update of a media component in the CHF CDR including Satellite ID, AGW on the satellite indication, AGW IP address information, AGW on the satellite start time stamp for UE-SAT-UE communication.

12.	The CHF A acknowledges the reception of the data and opens a CDR.

13-14. P-CSCF A forwards the Response Confirmation to P-CSCF B. The Response Confirmation includes SDP offer modified/generated by P-CSCF A. Upon receiving the SDP offer, P-CSCF B updates the allocated IMS-AGW on the satellite for UE B with the transport address in the SDP offer.

15. The IMS-GWF B sends a Charging Data Request[Update] to the CHF B to record update of a user session. And update of a media component in the CHF CDR including AGW on the ground indication, AGW IP address information, AGW on the ground end time stamp. The IMS-GWF B’ sends a Charging Data Request[Update] to the CHF B to record update of a user session. And update of a media component in the CHF CDR including Satellite ID, AGW on the satellite indication, AGW IP address information, AGW on the satellite start time stamp for UE-SAT-UE communication.

16.	The CHF B acknowledges the reception of the data and opens a CDR.

17. Procedure continues

18.	P-CSCF releases IMS AGW on the source satellite sometime after receiving SIP 200 OK.

19. The IMS-GWF sends a Charging Data Request[Termination] to the CHF to record end of a user session. And end of a media component in the CHF CDR including Satellite ID, AGW on the satellite indication, AGW IP address information, AGW on the satellite end time stamp for UE-SAT-UE communication.

20.	The CHF A acknowledges the reception of the data and opens a CDR.

As specified in the TS 23.228 [11], if the originating network cannot continue, P-CSCF determines IMS AGW relocation and media routing path change due to change of satellites. Moreover, if P-CSCF determines that optimized media routing cannot continue if there is no target satellite ID included, or the two satellites identified by the satellite ID for the originating network and the satellite ID stored for the terminating network has no ISLs. The P-CSCF determines to activate ground fallback routing.

For above cases, the main change is based on the SIP RE-INVITE message triggering the change of IMS GWF. So the default trigger conditions in IMS node in the Table 5.4.3.1 of TS 32.260[x] need to have the following enhancements:

-	SIP RE-INVITE or UPDATE due to IMS AGW IP address changes

The IMS charging information in the clause 6.4.2.2.1 of TS 32.260[x] need to have the following enhancements:

-	Satellite ID

Note: The satellite ID is planned to be specified in the SIP header of the TS 24.229[y].

End of Modified Sections