3GPP TSG SA WG5 Meeting #159	S5-250716

Sophia-Antipolis, FRANCE 17 - 21 February 2025

# Source:	Huawei

# Title:	New charging solution for DC application download by volume

# Document for:	Approval

Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to approve the follow update in TR 28.851.

2	References

[1]	3GPP TR 28.851: "Study on charging aspects of next generation real time communication services phase 2".

3	Rationale

This pCR proposes new solution for topic 2 on IMS DC download and usage charging in TR 28.851 [1].

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.851.

First change

5.2.4.1	Solution #1: DC application download charging by event

5.2.4.1.1	General

This solution #1 resolves the key issues #1for REQ_NG_RTC_CH-DCAPP-01. The solution is based on the data channel signalling control function (DCSF), as specified in TS 23.228 [5], to collect and report the charging information for DC application download event.

Next change

5.2.4.x	Solution #x: DC application download charging by reporting volume per bootstrap DC

5.2.4.x.1	General

This solution #x resolves the key issues #1 for REQ_NG_RTC_CH-DCAPP-01. The SMF charging specified in TS 32.255 [10] can support the volume based charging for IMS services, as specified in clause 5.1.18, which may be extended to support the reporting of volume for downloading DC application(s) during an IMS Bootstrap DC.

5.2.4.x.2	Description

The IMS DC application(s) are downloaded during the bootstrap DC procedure, according to bootstrap DC setup signalling procedure in the TS 23.228 [5] Figure AC.7.1-1. In this case, the volume for downloading applications can be counted by the volume consumed per bootstrap DC.

According to TS 23.228 [5] clause 4.6.1, the P-CSCF can derive the session information that is relevant for Policy and Charging Control from the SDP contained in the SIP signalling and forwards it to the PCF over Rx or N5 interface, which may identify the bootstrap DC information.

After receiving the IMS bootstrap DC setup session information from P-CSCF, the PCF may assign a service identifier for bootstrap DC. When SMF receives the PCC rule with service identifier for an IMS bootstrap DC, it can support the volume based charging for IMS DC applicaton download according to the application based charging principle specified in TS 32.255 [10] clause 5.1.17.

End of changes