3GPP TSG-SA5 Meeting #159	S5-250731

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	pCR TR 28.849 Solution for Topic#3

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

This is a pCR to TR 28.849 introducing a Solution for Topic #3.

# 2	References

[1]	3GPP TR 28.849 "Study on charging aspects for Common API Framework for Northbound APIs (CAPIF) phase2".

# 3	Rationale

This pCR proposes to introduce a new Solution for Topic #3 in TR 28.849 [1].

# 4	Detailed proposal

Propose to include the following changes into the TR 28.849 [1].

First change

#### 6.3.5.x	Solution #3.X: CAPIF APIs charging

##### 6.3.5.x.y	General description

This solution #3.x which relies on CHF/5G Converged Charging System for CAPIF Converged Charging, addresses the Key Issue #3.1.

##### 6.3.5.x.y	Architecture description

Figure 6.3.5.x.y-z shows the 5G System high level charging architecture for CAPIF Core Function (CCF) Charging, in the reference point representation for non-roaming:

Figure 6.3.5.x.y-z: High level functional architecture for CAPIF (TS 23.222 [2], clause 6.2.0)

##### 6.3.5.x.y	Procedures description

The Figure 6.3.5.x.y-z describes the high-level charging procedure for CCF Converged charging for API management via CAPIF-4, based on Figure 8.3.3-1, 8.4.3-1, 8.5.3-1 and 8.6.3-1 about the procedure for API Invocation in TS 23.222 [2]. The event-based charging, i.e. IEC, PEC and ECUR specified in 3GPP TS 32.290 [4] are supported.

Figure 6.3.5.x.y-z: Event Charging Procedure for API object to the CAPIF (IEC)

1.	The API Provider sends a service request that relates to an API object lifecycle change (creating/deleting) or API object access (reading/updating).

2.	The AEF checks whether the API invoker is authorized.

3.	Corresponding to the nature of the request in step 1, the AEF sends the Service API status trigger to CTF, via the CAPIF-3 interface.

3ch-a.	The CTF sends Charging Data Request to CHF for the received API Invocation.

3ch-b. The CHF opens CDR for this API Invocation.

3ch-c. The CHF acknowledges by sending Charging Data Response.

4.	The CCF notifies the AEF about the outcome of the charging procedure.

5.	The AEF executes the API logic, fulfilling the API request.

6.	The AEF returns the API response.

Figure 6.3.5.x.y-z: Event Charging Procedure for API object to the CAPIF (PEC)

1.	The API Provider sends a service request that relates to an API object lifecycle change (creating/deleting) or API object access (reading/updating).

2.	The API invoker is authorized.

3.	Corresponding to the nature of the request in step 1, the AEF sends the Service API status trigger to CTF, via the CAPIF-3 interface.

4.	The AEF returns the API response.

5.	Corresponding to the nature of the request in step 1, the AEF sends the matching API object lifecycle trigger or API object access trigger to CaTF, via the CAPIF-3 interface.

5ch-a.	The CTF sends Charging Data Request to CHF for the received API Invocation.

5ch-b. The CHF opens CDR for this API Invocation.

5ch-c. The CHF acknowledges by sending Charging Data Response.

6. The CCF notifies the AEF about the outcome of the charging procedure.

Figure 6.3.5.x.y-z: Event Charging Procedure for API object to the CAPIF (ECUR)

1.	The API invoker sends a service request that relates to an API object lifecycle change (creating/deleting) or API object access (reading/updating).

2.	The AEF checks whether the API invoker is authorized to perform this operation.

3. Corresponding to the nature of the request in step 1, the AEF sends an Service API status trigger to CTF, via the CAPIF-3 interface, indicating initialization of the unit reservation.

3ch-a.	The CTF sends Charging Data Request to CHF for the received API Invocation.

3ch-b. The CHF opens CDR for this API Invocation.

3ch-c. The CHF acknowledges by sending Charging Data Response whether the request is within the spending limit.

4. The CCF notifies the AEF about the outcome of the charging procedure, including an indication whether or not the processing can go ahead.

5.	The AEF executes the API logic, fulfilling the API request, in case it has received permission in step 3ch-c.

6.	Corresponding to the nature of the request in step 1, the AEF sends a Service API status trigger to CTF, via the CAPIF-3 interface, indicating termination of the unit reservation.

6ch-a.	The CTF sends Charging Data Request to CHF for the received API Invocation.

6ch-b. The CHF opens CDR for this API Invocation.

6ch-c. The CHF acknowledges by sending Charging Data Response, closes the reservation and responds to the CCF.

7.	The CCF notifies the AEF about the outcome of the charging procedure.

8.	The AEF returns the API response.

End of changes