3GPP TSG-SA5 Meeting #159	S5-250748

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	China Mobile

# Title:	Rel-19 pCR 28.851 Adding solution on Support of Avatar communication

# Document for:	Approval

Agenda Item:	7.5.3

1	Decision/action requested

This is a pCR to add solution on support of Avatar communication in TR 28.851.

2	References

[1]	3GPP TR 28.851: "Study on charging aspects of next generation real time communication services phase 2".

3	Rationale

This pCR proposes to add solution on support of Avatar communication in TR 28.851 [1].

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.851.

First change

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

API	Application Programming Interface

AS	Application Server

A2P	Application to Person

DC	Data Channel

DCSF	Data Channel Signalling Function

NEF	Network Exposure Function

P2A	Person to Application

P2P	Person to Person

Second change

5.5.4	Possible solutions

5.5.4.1	Solution #1: IMS AS based charging solution for Avatar communication

5.5.4.1.1	General

Solution #1 addresses Key Issue #1a and #1b for REQ-NG_RTC_CH_NCIAC-01. The solution is based on the IMS AS, which collects and reports specific charging information for Avatar communication.

5.5.4.1.2	Description

In TS 23.228 [5] Figure AC.11.3.3-1, UE1 requests the network to perform avatar animation. If the avatar animation negotiation between the DC AS and UE1 is successful, UE1 initiates new application data channels for avatar data transmission in Step 5. The IMS AS sends Charging Data Request[Start] to CHF for application data channel of Avatar communication service established successful, triggered after Step 5. The Avatar communication service release may be triggered by end of the Avatar communication service via SIP BYE message, or SDP negotiation via another SIP Re-INVITE message to release the application data channel. It will trigger IMS AS to send Charging Data Request[Stop] to CHF.

- The "Avatar Media Specification" attribute for Nimsas_MediaControl_MediaInstruction service operation which is invoked by the DCSF to the IMS AS specified in clause AA.2.4.3.2 of TS 23.228 [5] can support IMS AS based charging solution for Avatar communication. If there is a "Avatar Media Specification" attribute in Nimsas_MediaControl_MediaInstruction service operation, it indicates IMS AS that this is an Avatar communication media. The "Resource URL" attribute in "Avatar Media Specification" can be used to retrieve the avatar representation, and IMS AS can use it to identify Avatar Representation for charging purpose. In this case, the support of the IMS AS for the collection and reporting of charging information related to the Avatar communication media and  Avatar Representation Identifier in TS 32.260 [11] can be extended.

5.5.5	Evaluation

Solution #1 addresses Key Issue #1a and #1b proposing a converged charging method for network centric IMS Avatar communication. This requires IMS AS to collect and report specific charging information for Avatar communication to CHF.

5.5.6	Conclusion

Based on the evaluation in clause 5.5.5, Solution #1 is the only solution for network centric IMS Avatar communication. The Solution #1 is selected to the normative work.

End of changes