3GPP TSG-SA5 Meeting #159	S5-250829

Sophia-Antipolis, France, 17 - 21 February 2025

Title:	LS reply to LS on per-UE UE performance metrics

Response to:	S5-250286 LS on per-UE UE performance metrics

Release:	3GPP Rel-19

Work Item:	PM_KPI_5G_Ph3

Source:	3GPP SA5

To:	3GPP RAN3

Cc:	3GPP RAN2

Contact person:	Ashutosh Kaushik

Ashutosh19 <dot> k <at> samsung <dot> com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA5 thanks RAN3 for their LS on per-UE UE performance metrics. SA5 has discussed the LS and would like to make following clarification:

For the Average DL UE throughput in gNB and Average UL UE throughput in gNB, as defined in clauses 6.3.1.4.1 and 6.3.1.4.2 respectively in TS 28.558, the formulas in fields c) are producing the measurements with a “per-UE granularity”. The measurement name in field e) starting with “DRB” only indicates that they are part of the DRB family of measurements. The measurement is actually for “per-UE granularity”. “ThpVolDl/Ul” and "ThpTimeDl/Ul” are first obtained for each DRB (as defined in table) and are summed up to get a per UE value (for all the DRBs) which is used in formula of field c) to finally obtain a per UE average throughput value.

# 2	Actions

To RAN3

ACTION: 	SA5 kindly requests RAN3 to take the above into consideration.

# 3	Dates of next TSG SA WG5 meetings

SA5#160		07 April - 11 April 2025			Gothenburg, Sweden

SA5#161		19 May - 23 May 2025			Fukuoka, Japan