3GPP TSG-SA5 Meeting #159	S5-250838

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0291 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Enhancement of Intent handling state management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,Deutsche Telekom,Nokia, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Intent handling state diagram and corresponding state transition events were studied in TR 28.914 and recommended for normative work. So, it proposes to add intent handling state diagram and intent handling state transition table in 3GPP TS 28.312. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add intent handling state diagram and intent handling state transition table in 3GPP TS 28.312. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Intent handling state management |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x, 5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

4.x	Intent handling state management

This clause provides Intent handling state diagram and corresponding state transition events to show the Intent handling state management.

Figure 4.x-1 shows the intent handling state diagram, where the number in the figure identify the state changes. The explanations for the state changes are described in Table 4.x-1.

Figure 4.x-1: Intent handling state diagram

The transition numbers in the first column represent the state changes in Figure 4.x-1. The interactions specified under the column "The state transition events" of Table 4.x-1 should be present for the state transition.

Table 4.x-1: The intent handling state transition table

| Transition number | The state transition events | State |
| --- | --- | --- |
| 1 | The MnS producer creates the intent instances based on the received intent creation request. | ACKNOWLEDGED |
| 2 | The MnS Producer performs the feasibility check of the intent instance, and the feasibility check result is 'feasible'. | COMPLIANT |
| 3 | The MnS Producer performs the feasibility check of the intent instance, and the feasibility check result is 'infeasible' | FULFILLMENT_FAILED |
| 4 | The MnS producer modifies the intent instances based on the received intent modification request. Correspondingly, the state transitions from COMPLIANT, SUSPENDED, FULFILLED, FULFILLMENT_FAILED or DEGRADED to ACKNOWLEDGED. | ACKNOWLEDGED |
| 5 | The MnS producer considers that the intent has been fulfilled. | FULFILLED |
| 6 | MnS producer configures attribute intentAdminState with the value "DEACTIVATED" | SUSPENDED |
| 7 | MnS producer configures attribute intentAdminState with the value "ACTIVATED" | COMPLIANT |
| 8 | The MnS producer determines that he cannot do anything to fulfil the intent. | FULFILLMENT_FAILED |
| 9 | The MnS producer considers that the intent was fulfilled but after a period of observation it is found that the originally specified requirements are no longer met. | DEGRADED |
| 10 | The MnS producer deletes the intent instances based on the received intent deletion request. | TERMINATED |

2nd Change

### 5.3.2	Intent report

#### 5.3.2.1	Introduction

The intent fulfilment information is defined as one type of intent report information in which the intentFulfilmentInfo, expectationFulfilmentInfo and targetFulfilmentInfo are included for the MnS consumer to monitor the intent fulfilment information. The intent fulfilment information also can contain the current value for performance indicated by corresponding expectation targets (e.g. WeakRSRPRatio for the weakRSRPRatioTarget, Average UL RAN UE Throughput for aveULRANUEThptTarget), which can be used by MnS consumer to validate whether the intent is really fulfilled and to evaluate whether the intent (especially for expectation targets) needs to be updated if needed (improve the target value when corresponding target is fulfilled or reduce the target value when corresponding target is not fulfilled). Besides, intent conflict information and intent fulfilment feasibility check information sent by MnS producer to MnS consumer is another type of intent report information. So, following are the three types of information needs to be monitored by MnS consumer:

-	Intent fulfilment information, which represents the properties of a specific fulfilment information for an aspect of the intent (i.e. either an expectation, a target or the whole intent), including fulfilment status and achieved values for targets.

-	Intent conflict information, which represents intent with conflict. The information includes conflict type (i.e., intent conflict, expectation conflict and target conflict) and possible solutions (e.g. intent deletion, intent modification).

-	Intent fulfilment feasibility check information, which indicates that the intent is feasible or infeasible. Intent fulfilment feasibility check information is provided after MnS producer automatically performs feasibility check when receiving the intent creation and modification request from MnS consumer.

Different MnS consumer may have different requirements for intent report (e.g. Some MnS consumer may want to have corresponding performance value information while others do not want. Different MnS consumer may want to calculate or monitor the performance value in different period).

MnS consumers may subscribe to get notifications of changes in the intent report.

An intent under fulfilment may go through multiple states, i.e., the life cycle includes multiple alternative transitions. The transitions are triggered either by actions of the MnS consumer or observations at the MnS producer or its intent handling function. Generally, intents received at the MnS producer will have one of the states represented in Figure 4.x-1.

Figure 5.3.2.1-1: Void

Intent reports should be delivered for each of these different states and related state transitions, specifically for:

1.	ACKNOWLEDGED: When an intent instance is created, its default state is "ACKNOWLEDGED". An intent report should be delivered to indicate that the intent instance has been created. The transitions from "ACKNOWLEDGED" state can only be to "COMPLIANT", "FULFILLMENT_FAILED" or "TERMINATED " states as described hereafter.

2.	COMPLIANT: When the feasibility check for the intent is successful and the intent is accepted as being compliant, the intent state is changed to "COMPLIANT". An intent report should be delivered to indicate the results of the feasibility check. In the case of an intent delivered for fulfillment, a corresponding intent report should be delivered to indicate the status change. The payload for the intent feasibility report (outcome of the feasibility check) may be one of the following:

1.	an indication for feasible or infeasible;

2.	a detailed report indicating which intent expectations or ExpectationTargets are infeasible and the corresponding reasons;

The transitions from "COMPLIANT" state are either to "ACKNOWLEDGED" state when the intent attributes are modified by the MnS consumer, or to "SUSPENDED", " FULFILLED " or "FULFILLMENT_FAILED" states as described hereafter.

3.	SUSPENDED: If the MnS consumer decides to suspend the intent, the intent state changes to "SUSPENDED". Alternatively, an event may occur while the MnS producer attempts to fulfil the intent, in which case the MnS producer may suspend the intent pending further actions. Such events at the MnS producer may among others include conflicts from another intent and resource constraints. An intent report should be delivered to indicate that the intent has been suspended. The intent report may include a reason for the suspension when the intent was suspended by the MnS producer.

The transitions from "SUSPENDED" state are either to "ACKNOWLEDGED" state when the intent attributes are modified by the MnS consumer, or to "COMPLIANT" state if the suspension is lifted by the entity (MnS consumer or MnS producer) that suspended the intent. Otherwise, the state is "TERMINATED" if the intent is deleted.

4.	FULFILLED: If the MnS producer considers that the intent, expectation or target has been fulfilled as stated by the MnS consumer, the state changes to "FULFILLED". An intent report should be delivered to indicate that the intent has been fulfilled.

The transitions from "FULFILLED" state are either to "ACKNOWLEDGED" state when the intent attributes are modified by the MnS consumer or to "DEGRADED" as described for the "DEGRADED" state below.

5.	DEGRADED: If an intent that was previously fulfilled but after a period of observation it is found not be meeting the initially stated requirements, the state changes to "DEGRADED". An intent report should be delivered to indicate that the intent has degraded.

The transitions from "DEGRADED" state are either to "ACKNOWLEDGED" state when the intent attributes are modified by the MnS consumer, or to "FULFILLED" state if the MnS producer once again fulfills the intent.

6.	FULFILLMENT_FAILED: If the MnS producer determines that they cannot do anything to fulfil the intent, the state changes to "FULFILLMENT_FAILED". An intent report should be delivered to indicate that the fulfillment of the intent has ceased. The intent is not deleted unless the MnS consumer explicitly requests for it to be deleted.

The only transition from "FULFILLMENT_FAILED" state is to "ACKNOWLEDGED" state when the intent attributes are modified by the MnS consumer.

7.	TERMINATED: If the MnS consumer requests to delete the intent, the state changes to "TERMINATED". An intent report should be delivered to indicate that the intent has been terminated.

There is no possible transition from TERMINATED state.

8.	The intent may be modified when in any of the states "ACKNOWLEDGED", "COMPLIANT ", "SUSPENDED " "FULFILLED ", "DEGRADED" or "FULFILLMENT_FAILED". An intent report should be delivered to indicate that the intent has been modified.

9.	The intent may be deleted when in any of the states "ACKNOWLEDGED", "COMPLIANT ", "SUSPENDED " "FULFILLED ", "DEGRADED" or "FULFILLMENT_FAILED". An intent report should be delivered to indicate that the intent has been deleted.

#### 5.3.2.2	Requirements

REQ-IDMS_IntentReport-CON-1: The intent driven MnS producer shall have the capability to enable the MnS consumer to request intent report information.

REQ-IDMS_IntentReport-CON-2: The intent driven MnS producer shall have the capability to enable the MnS consumer to obtain intent report information with intent fulfilment information (including fulfilment status and achieved values for targets).

REQ-IDMS_IntentReport-CON-3: The intent driven MnS producer shall have the capability to enable the MnS consumer to obtain intent report information with intent conflict information.

REQ-IDMS_IntentReport-CON-4: The intent driven MnS producer shall have the capability to enable the MnS consumer to obtain intent report information with intent fulfilment feasibility check information.

REQ-IDMS_IntentReport-CON-5: The intent driven MnS producer shall have capability enabling MnS consumer to specify the content of the intent report.

REQ-IDMS_IntentReport-CON-6: The intent driven MnS producer shall have capability enabling MnS consumer to configure the frequency of the intent reporting.

REQ-IDMS_IntentReport-CON-7: The intent driven MnS producer shall have the capability enabling MnS consumer to receive reports, with different content and intervals according to its specified requirements.

REQ-IDMS_IntentReport-CON-8 The intent driven MnS producer shall have capability enabling the MnS consumer to receive an intent report when any of the following happens:

-	the intent state has been changed to either "ACKNOWLEDGED" (intent has been created or modified), "COMPLIANT ", "SUSPENDED " "FULFILLED ", "DEGRADED" or "TERMINATED" (intent has been deleted).

REQ-IDMS_IntentReport-CON-9 The intent driven MnS producer shall have capability enabling MnS consumer to receive an intent report indicating the reasons associated with any of the following events: “SUSPENDED”, “DEGRADED” or “TERMINATED”.

REQ-IDMS_IntentReport-CON-10 The intent driven MnS producer shall have capability enabling MnS consumer to receive an intent report on the outcomes of a feasibility check, the report indicating either of 1) an indication for feasible or infeasible; 2) a detailed report indicating which intent expectations or ExpectationTargets are infeasible and corresponding reasons.

End of Changes