3GPP TSG-SA5 Meeting #159	S5-250852

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	TS28.567 CCL Goal- and Goal-targets- conflicts handling

# Document for:	Approval

Agenda Item:	6.19.4.1

# 1	Decision/action requested

Group is requested to discuss and agree on the “Detailed Proposal”.

# 2	References

[1]	None

# 3	Rationale

Add use case, requirements and solution description for Conditional control of CCLs of CCLs

# 4	Detailed proposal

Start of First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[X]	3GPP TS 28.872: "Study on Management of planned configurations".

next change

# 6	Management Capabilities

## 5.X	 CCL Conflict management Capability - CONFMGT-

### 5.x.1			Description

These use cases deals with conflict management for CCL.

#### 5.x.2	Use Cases

#### 5.A.2.K	CCL Goal- and Goal-targets- conflicts handling - GTCONF-

##### 5.A.2.K.1	Introduction

CCL instances will be responsible for related, adjacent or in some cases overlapping scopes. In such cases it is good to ensure that the goals of two CCLs are not contradictory or conflicting or leading to contradictory or conflicting outcomes. The goal may be seen as a set of network measurements and KPIs (i.e. targets) to be concurrently achieved by the CCL. A CCL coordination capability, may have a goal management capability responsible for managing and optimizing the goals of the CCLs based on general objectives for the network scope, where the general objective describe the priorities among the different target values on the specific KPIs. For example, as illustrated by Figure 5.6.3.3.1-1, the input network scope objectives may simply require ensuring, for KPI K1, that with priorities, p1, p2, p3, the value of K1 should respectively be less than values V1, V2, V3. This may be provided for different KPIs on a network level (e.g. by the operator). Then, the goal management capability compares these objectives to choose the appropriate KPI targets for each CCL. In the example, the CCL goals are set as K1 < 0.1 and K2 > 85 %.

Figure 7.1.1.2-1: Illustration of example prioritized goal targets on a set of KPIs that need to be coordinated among a group of CCL instances.

##### 5.A.2.K.1	Detection and avoidance of potential goal targets conflicts

To support detection and avoidance of potential goal targets conflicts:

-	In the case where the CCL has been pre-configured e.g., by the operator with the expected targets, the CCL needs to register its goal targets with the CCL coordination capability goal management capability which triggers an evaluation of potential conflict, i.e. whether those targets are likely to conflict with the targets of another CCL.

-	In case of a potential conflict, the goal management capability needs to send the selected new or revised goal targets to each CCL ensuring to minimize contradictions or conflicts among the targets in the different goals of different CCLs, for example, that for a given scope a specific target is assigned to only one CCL.

-	Alternatively, using the define general objectives for the network scope, the CCL Coordination capability goal management capability may select the appropriate goal targets for the CCL

##### 5.A.2.K.2	Detection and avoidance of actual goal targets conflicts

To support detection and resolution of actual goal targets conflicts:

-	The CCLs attempt to fulfil its set targets, and where they ae unable to, the CCL sends feedback to the goal management capability in the coordination capability indicating which targets cannot be fulfilled. A CCL may for example indicate that there are ping-pong effects on a target, i.e. whenever the target is pushed in a given direction, it flips back to a previous state. The flipflop is an indication of a potential goal conflict which the CCL should notify to the goal management.

-	The CCL could detect potential constraints with the defined goals, e.g., that targets that cannot be reached due to conflicts among with other targets. The CCL should notify the coordination capability goal management capability, e.g., the response could be that “Target x cannot be achieved because it causes problems on higher priority target y.”.

-	Based on the feedback, the goal management capability acting as CCL coordination MnS producer can then confirm the existence of goal-target conflict and may revise the targets by setting new target or goal target values.

##### 5.A.2.K.3	CCL Goal-conflicts resolution

The targets in the goals of Closed Control Loop should not contradict one another within that goal or contradict with other targets in goals of related CCLs, otherwise a goal conflict is observed. For such a goal conflict, goal coordination interactions are needed to resolve the conflict, i.e. to align goals (and related targets) that should be achieved by the various deployed Closed Control Loops. Given the potentially high number and diversity of Closed Control Loops, the process of setting and coordination goals for the Closed Control Loops should be accomplished using another capability, the CCL coordination capability, that consumes the CCL-related monitoring and governance management services to coordinate the resolution of conflicts with the CCL.

The MnS producer for the CCL instance should inform the CCL coordination capability (acting as the MnS consumer) about a candidate goal conflict, e.g. about the values of the goal's targets that are in conflict with the targets of another goal. In response, the CCL coordination capability (acting as the MnS consumer) could revise the goals of that CCL instance, terminate the execution of the CCL instance, delete the CCL instance.

#### 5.A.2.L	CCL scope conflicts handling - SCPCONF

##### 5.A.2.L.1	Introduction

Each CCL should have specific scopes for which it is responsible.

The network may be assumed to be a p-dimensional space SP from which subregions dP Є D maybe created. Accordingly, SP is the full scope space whose dimension may include time, geography, etc. as showed in Table 5.5.A-1 while dp Є D can be CCL's scope. In that respect, scope assignment is the mapping of CCLs to regions dЄD that are part of the network's full scope S. There may be 2 types of scopes - the measurement scope where related measurements are collected and the impact or control scope which is the scope to which the CCL's actions may have impact. The scopes for the different CCLs can be managed by the MnS consumer.

Table 5.5.A-1: Example scope-space map from which the scope of CCL may be derived

| Scope dimension | Granularity | Example values to be assigned |
| --- | --- | --- |
| Time | Seconds, minutes, days | Every hour, Every Saturday at 2:00 hours |
| Network domains | Not applicable | Radio Core |
| Geography | Region/City | City x Street y in City x |
| Network Elements | gNB | gNB X |
|  | Cells | Cell A on gNB X |
|  | Terminals, e.g. types of users | users |
| Resources | Slices | IoT slice |
|  | Network Function | Virtual Network Function A Physical Network Function B |
|  | Transport containers (links, flows, etc.) | an identifiable link, a specific flow |
| Target Purpose | The purpose of the CCL target | Coverage Targets, Performance Targets, Energy Efficiency Targets, Fault Management Targets, UE specific Targets |

NOTE:	Table 5.5.A-1 is not complete and can be improved and/or extended as needed.

##### 5.A.2.L.2	Detection and avoidance of potential scope conflicts

To coordinate scope assignments, a CCL coordination capability, say in CCL Coordination capability, needs a capability to coordinate the scope assignment across multiple CCLs, say called the scope assignment coordination capability. The scope assignment coordination capability considers a defined full scope space Sp and a set of scope rules to define the best scope to be assigned to each CCL. An example rule may be that the defined CCL scope should not overlap. The rules may for example be defined by an operator or can be implementation specific depending on the types of CCLs that are to be configured.

To support detection and avoidance of potential scope conflicts:

-	Each CCL has a scope, e.g. the assuranceScope for the ACCL. The scope may be defined by an operator. Alternatively, the CCL Coordination capability may derive the required scope and configure it onto the CCL.

-	The CCL needs to register its scopes with the coordination capability (e.g., for the case where the scope is not defined by the CCL Coordination capability). The notification of the CCL scope to the CCL Coordination capability triggers an evaluation of potential conflict, i.e. whether those scopes are likely to conflict with the scopes of another CCL.

-	Applying the rules, the scope assignment coordination capability needs to divide the scope space into regions such that each region is matched to a CCL in a way that maximizes fulfilment of the assignment rules defined in the scope assignment coordination capability. The For example, if the benefit is to avoid overlaps, the subregions are assigned to the different CCLs in a way that ensures no overlaps and that all the scope space has been assigned.

-	A CCL may have four scopes - the measurement scope, target scope, control scope and impact scope with different rules applied for each scope. The selection of subregions of the scope space should consider the different rules for each type of scope.

-	In case of a potential conflict, the new optimized subregions need to be selected and assigned to the individual CCLs. The scope assignment coordination capability should be enabled to configure the scopes of the CCLs.

##### 5.A.2.L.3	Detection and resolution of actual scope conflicts

Scope conflicts are only considered actual if their use results in negative outcomes. To support detection and resolution of actual scope conflicts:

-	The CCLs needs to monitor changes in their scope and if the scope is changed, the CCL should be able to inform the scope assignment coordination capability of any observed changes in the scope.

-	The CCL needs to notify differences between what was configured and the actual scopes e.g. if the considered scope for taking measurement data are affected by the actions of another CCL.

-	The scope assignment coordination capability may subsequently need to trigger scope conflict evaluation based on the actual scope that is notified by the CCL.

#### 5.A.2.M	CCL Trigger-time and action-execution-time conflicts handling - TMCONF

##### 5.A.2.M.1	Introduction

Typically, a CCL needs to be triggered to run at a specific time and terminate when certain conditions are met, to run when a certain performance threshold is crossed. If triggered independently, there may be conflicts among the CCLs. The triggers for different CCLs to be executed need to be coordinated to avoid conflicts among the CCLs.

In a network with several CCLs, it can be that each CCL focuses on a smaller scope of the network problem space, and thus, propose changes that are suboptimal to the full problem space. To support the full problem space, several CCLs may need to be executed but it is important that their actions do not collide with each other in any network scope. The CCLs need to be explicitly scheduled to act by the CCL coordination capability which has a wider view of the network problems, the capabilities of the CCLs and their likely impacts.

##### 5.A.2.M.2	Avoidance of action-execution-time conflicts

Since each CCL focuses on a smaller scope of the network problem space, several CCLs may need to be executed.  To avoid collision of their actions in a given network scope, the CCLs can be explicitly scheduled by the CCL coordination capability. The CCL coordination capability obtains the capabilities of the available CCLs including their desired scopes and execution times. The CCL coordination capability can evaluate network data and analytics to identify the nature of the problem and best CCLs to be scheduled at specific times to address the problem but without their execution conflicting with one another.

NOTE:	The effectiveness of the avoidance relies on accuracy of network state analytics, inaccuracies in analytics can lead to suboptimal decisions and unidentified conflicts.

##### 5.A.2.M.3	Detection of potential action-execution-time conflicts

5.A.2.M.3.1	Information on action plans for alignment and selection

Each CCL deployed in the network has a set of scopes for which it takes responsibility and actions that it can execute within those scopes and from which the CCL can derive the decision and action plans that should be executed. An action plan is the combination of a set of actions that can be taken and the scopes under which those actions can be applied. To minimize conflicts (e.g. where the scopes overlap), the CCLs need to align the action plans, for example, through a coordination capability that selects which action plan to execute and when. Thereby:

-	The CCLs needs inform the coordination capability about their respective action plans. The action plans contain information of target resources, scheduled time for execution, and may include other additional information such as historical results of the proposed actions.

-	The coordination capability needs to assess each plan and choose the most appropriate combination of action plan(s) based on the selection policy. The appropriateness of action plan(s) or their combinations can be evaluated by multiple means and by using, for instance, historical data and/or operational data.

-	Notify the selected action plan(s) to the related CCLs and/or the CCL coordination capability.

5.A.2.M.3.2	Information on detected action-execution-time conflict

Coordinating actions among multiple CCLs requires that there is a supervisory action-critic functionality that oversees the actions of the different CCLs. The action-critic functionality which may be part of the CCL coordination capability that takes the responsibility for the end-to-end performance of the Network.

For a given CCL, the action-critic needs to receive the recommended changes from the CCLs, evaluates them to see:

1)	if they overlap with other proposed changes from other CCLs; and

2)	what their likely effects may be.

To determine the likely impacts, the action-critic may rely on network states analytics capabilities which discretize the state of the network into specific discrete scenarios and provide insights on performance characteristics in those scenarios. Such insights may for example characterize whether the network is in a scenario of low traffic and normal performance or scenario of normal traffic and anomalous performance.

Where there are likely conflicts and expected undesired impacts, the orchestration capability decides the changes that should be executed on the network to minimize concurrent changes on the same network resources. The selection of actions to be accepted may be based on the priorities of the CCLs or the priorities of their goals and targets.

The CCL coordination capability then need to provide feedback to the CCL instance (s) regarding their recommended actions. The feedback may include information on which actions can be executed or not as well as information on the expected effects of the CCLs actions. Feedback may also include redefining the allowed control parameter spaces and ranges of the individual CCLs (i.e. which parameters the CCL should not control any further or the range in which the CCL needs to set the value of a control parameter).

##### 5.A.2.M.4	Resolution of CCL Trigger-time conflicts

The triggers for execution of different CCLs need to be coordinated to avoid conflicts among the CCLs. And in some instances, the conditions in the network may be such that it is not clear which CCL should be triggered, requiring to trigger multiple CCL in sequence. The triggering may be done by a coordination capability that consumes the CCL-related monitoring and governance services to receive information with which to evaluate the conditions and determines which CCL to be triggered.

It may be the case that CCLs need to operate in a hierarchy with each CCL having an operational profile indicating the specific level of hierarchy. The operational profile describes characteristic sunder which the CCL operates, e.g. when or after which other CCLs, this CCL should be executed. For example, to ensure that handovers are always optimal, a CCL on handover optimization may need to be triggered every after a CCL on Energy saving has been executed to be sure that there are appropriate handover relations even when some cells may have been disabled. The MnS consumer that coordinates the execution times of the CCLs needs to configure the appropriate hierarchy for the CCLs. Using the operational profiles of the CCLs, , the MnS consumer can evaluate the description of the third CCL against at least one of the profiles P1 and P2 and accordingly determines and configures the operational profile of the third CCL.

NOTE:	A CCL may be involved in more than 1 hierarchies or within a single hierarchy, the CLL may relate to multiple other CCLS, which requires the hierarchies to be coordinated.

#### 5.A.2.N	CCL Direct-actions conflicts management handling - DACONF

##### 5.A.2.N.A	Detection and avoidance of potential direct actions conflicts

If two CCLs execute their actions within the same time period, the actions could cause undesirable effects, e.g. by conflicting for the same parameter on the managed entities. Before the interacting CCLs execute their actions, the Pre-execution coordination can be used to detect potential direct-actions conflicts.

To detect and avoid potential direct-actions conflicts:

-	A CCL intending to take an action needs to send its proposed configuration management changes to the CCL coordination capability prior to execution of those configuration changes. The configuration changes contain information of target resources and scheduled time for execution.

-	The CCL coordination capability can check the submitted configuration changes on a given scope against other previous configuration changes on the same or related scope from other CCLs (that have been executed) to see if there are any potential conflicting actions based on the provided information. This ensures to check planned configuration changes against actions that have already been executed.

To avoid potential direct-actions conflicts:

-	The CCL coordination capability needs to notify the detected conflict(s) to the related CCLs.

-	The CCL may need to adjust its planned configurations to a new set that could have less conflicts.

##### 5.A.2.N.B	Detecting actual conflicts via counter-productiveness

In certain cases, two CCLs may work together on the same managed , maybe at different times scales or involving different aspects/sub functionalities of the managed . However, there may be some known or unknown interdependence between actions taken by the two CCLs, e.g., in the cases where the scopes of the two CCLs cannot be separated. Furthermore, two CCLs may change the same parameter one after the other. For multi-aspect optimization, such interdependence is often expected, and it should be tolerated by the system as long as it is not harmful in terms of the overall performance of the managed .

A way to detect actual conflicts and minimize their impacts is detect counter-productiveness. CCLs that operate on the same managed object monitor any counter-productiveness and if observed, maintain it within some tolerance limits.

The CCLs need to subscribe to their scopes of interest and based on scopes they have subscribed to, the coordinator capability needs to inform each CCL instances of the other CCLs instances which have related scopes. A CCL instance A that is likely to be affected, needs to monitor a specific scope or context that could be affected by another CCL instance B. And CCL B may know that CCL may take actions that could affect the same scope as CCL B. CCL B can provide a conflict monitoring context/scope to CCL A informing CCL A about CCL B's latest actions on the managed entity and its tolerance w.r.t to its parameters and metrics in this managed . This may be either directly to B or via the coordination CCL. The tolerance indicates the performance limits which CCL B would like CCL A to respect. CCL A (the context recipient CCL) should work within these bounds, i.e. its actions should not violate the said tolerances to avoid counter-productiveness. CCL A observes the conflict monitoring context, so that if it observes the violations of the said tolerances, it reports the conflict to the CCL B– again either directly to B or via the coordination CCL.

NOTE:	If CCL A is able to predict violation prior to activating the actions, CCL A inform CCL B of the predicted impact at the time when the action is being activated. Otherwise, CCL a first observes the impacts and ten informs CCL B.

##### 5.A.2.N.4	Bargaining for resolution of potential direct actions conflicts

The simplest way to resolve direct parameter conflicts is to separate the control spaces of the CCLs, i.e. to allocate each parameter to a specific CCL. However, this is not guaranteed to always be possible, i.e. in some cases two or more CCLs may want to set different values for the same parameter and the parameter cannot be assigned to only one CCL. In these cases, a coordinator capability, e.g. a coordination capability should compute a compromise value for the parameter, a value which can be considered to be equally good for all the CCLs. However, since different CCLs have different goals, it is necessary for the coordination capability to understand the importance of the parameter to each CCL. For this purpose, the CCLs provide their usefulness for the parameter to the coordinator CCL. The usefulness provided by a CCL shows the relative goodness of different values of the parameter to the CCL in a pre-defined scale, e.g. [0:1]. Since all the CCLs used the same scale, when the Coordination capability selects a parameter value, it can clearly understand how important this value is for each CCL. The Coordination capability can then derive the compromise values which is then (provided to the CCLs to be) executed onto the managed object. An example way to compute the compromise is to use the Nash Social Welfare Function since it provides equal fairness to all competing entities.

A compromise based only on usefulness does not consider the relative (level of) interest of the CCLs in the parameter. To account for the interests, the CCLs needs to provide to the Coordination capability their relative interest in the parameter, so that the computed compromise value accounts for the combined interests of the CCLs. The relative interest may be computed based on a fixed scale. For example, for a CCL on cell interference management on a scale of [0-10], a cell's transmit power has a goodness of say 9 than the cells load which has a goodness of 3.

NOTE 1:	The CCL coordination capability does not have to calculate the compromise value all the time as this requires information exchange among the CCLs and computational energy. It should be possible to configure the CCL coordination capability such that it calculates the compromise values only when certain conditions are met. The CCL coordination capability should be able to expose required services to the MnS consumer to configure such conditions.

NOTE 2:	For a given CCL, the usefulness may be equivalent to the level of interest, but it is not always the case. It is possible that a CCL has high interest in a parameter that has low usefulness.

#### 5.A.2.P	CCL Indirect target conflict handling - INDCONF

##### 5.A.2.P.1	Introduction

Two (or more) CCLs configuring different control parameter may all influence the same metric. If the two CL desire different values for the metric, the CCLs are in conflict for the metric resulting into a metric-value conflict. In effect the actions of the two CCLs are in conflict but indirectly since they are conflicting for the same control parameter but their impacts are conflict on the desired value of the metric or target. Such conflicts are thus called Indirect target conflicts.

The Indirect target conflicts should be avoided and if they occur, they should be detected and resolved.

##### 5.6.3.5.2	Detecting potential and actual indirect targets conflicts

Two CCLs (CCL1 and CCL2) may optimize 2 target metrics Y1 and Y2, e.g. one intending to ensure "HO failure is < 2 %" while the other wants "SINR > 10dB". Due to coupling between Y1 and Y2, actions to optimize these by CCLs may lead to correlated oscillations/degradations in Y1 or Y2. The correlated oscillations indicate a potential conflict, but the CCLs may not see the oscillations in the metric that is not of their interest. The coordination capability may analyse the behavior of Y1 and Y2 to see if there are correlated oscillations as result of actions by CCL1 and CCL2 which then can indicate potential conflict between CCL1 and CCL2. When the oscillations are observed, the CCL coordination capability should be able to inform the related MnS Consumer(s) (i.e. CCL1 and CCL2) about the detected potential conflict represented by the correlated oscillations.

For detected potential conflict the CCL coordination service producer needs to confirm that it is an actual harmful conflict. This can be determined based on the severity of degradation in the performance metrics of the related CCLs. The threshold to determine the severity may be defined by the MnS consumer (e.g. the operator or coordinator CCL). If the degree of degradation is higher than the threshold then it is a confirmed conflict that requires resolution. Otherwise, no action is needed.

##### 5.A.2.P.3	Avoiding potential indirect target conflicts

For any CCL, large and frequent changes to network parameters may affect network stability since they increase the probability of occurrence of conflicts, i.e. avoiding making unnecessary configuration changes to the managed objects guarantees network stability and minimize the probability of conflicts between CCLs. This may then imply that executing large changes, e.g. to quickly improve the performance, in case of a poor decision, may also result in significant degradation. So, it is preferred to take small smooth changes in the case where the impact is not so clear, and only make the large changes when the CCL is sure that the impact is positive.

For any planned action, the CCL sends to the coordination capability the planned change, its claimed/predicted performance improvement and reliability/confidence in that action/decision. The coordination capability evaluates the claimed performance improvement and reliability/confidence to determine if the action should be allowed or not. This ensures to avoid counter-productive actions - if the CCL demands to make large changes, it proves high reliability/ confidence and significant improvement in performance. The criteria applied by the coordination capability to match acceptance/rejection of a planned action to the reliability and performance may be implementation-specific or defined by the operator.

The coordination capability then sends the decision and possibly the failed criteria to the CCL - to either be executed or to be used to compute better decisions. It is assumed that based on feedback on the quality of its decisions, the CCL updates its decision-making engine and repeats the decision evaluation process. Then if the CCL has consistently made good large action-decisions, the coordination capability can consider the CCL as trusted to make such large decisions. The coordination capability informs the CCL that the CCL has consistently made good decisions and achieved its ultimate trust.

##### 5.A.2.P.4	Resolving indirect targets conflicts

For a detected indirect targets conflict, the coordination capability can trigger one or more CCLs to respond to the detected potential conflict. If the CCLs that has been requested to resolve potential conflict is unable to resolve that conflict, the CCL should inform the CCL coordination capability about the failure to resolve the problem.

### 5.A.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-CONFMGT - DACONF-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL goal and goal targets conflicts. | CONFMGT-GTCONF |
| REQ-CONFMGT-GTCONF--02 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to avoid a CCL goal and goal targets conflict | CONFMGT-GTCONF |
| REQ-CONFMGT-GTCONF--03 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL goal and goal targets conflict after it is confirmed. | CONFMGT-GTCONF |
| REQ-CONFMGT-GTCONF--04 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to resolve a CCL goal and goal targets conflict | CONFMGT-GTCONF |
| REQ-CONFMGT-GTCONF--05 | The 3GPP Management System should support a capability to resolve a CCL goal and goal targets conflict that has been detected | CONFMGT-GTCONF |
| REQ-CONFMGT-GTCONF--06 | The 3GPP Management System should support a capability to coordinate the resolution of CCL goal and goal targets conflicts among multiple CCLs | CONFMGT-GTCONF |
| REQ-CONFMGT-SCPCONF-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL scope targets conflicts. | CONFMGT-SCPCONF |
| REQ-CONFMGT-SCPCONF-02 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to avoid a CCL scope targets conflict | CONFMGT-SCPCONF |
| REQ-CONFMGT-SCPCONF-03 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL scope targets conflict after it is confirmed. | CONFMGT-SCPCONF |
| REQ-CONFMGT-SCPCONF-04 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to resolve a CCL scope targets conflict | CONFMGT-SCPCONF |
| REQ-CONFMGT-SCPCONF-05 | The 3GPP Management System should support a capability to resolve a CCL scope targets conflict that has been detected | CONFMGT-SCPCONF |
| REQ-CONFMGT-SCPCONF-06 | The 3GPP Management System should support a capability to coordinate the resolution of CCL scope targets conflicts among multiple CCLs | CONFMGT-SCPCONF |
| REQ-CONFMGT-SCPCONF-07 | The 3GPP management system should support a capability enabling the MnS consumer to configure the scopes of a CCL, including the measurement scope, the control scope , and the impact scope. NOTE: Measurement scope is where the related measurements are collected, the control scope is where the CCL acts and the impact scope is where the CLL actions causes effects. | CONFMGT-SCPCONF |
| REQ-CONFMGT-SCPCONF-08 | The 3GPP management system should support a capability to coordinate assignment of the scopes among multiple CCLs,. including the measurement scope, the control scope and the impact scope | CONFMGT-SCPCONF |
| REQ- CONFMGT- DACONF-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL trigger-time and action-execution-time targets conflicts. | CONFMGT-TMCONF |
| REQ-CONFMGT-TMCONF-02 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to avoid a CCL trigger-time and action-execution-time targets conflict | CONFMGT-TMCONF |
| REQ-CONFMGT-TMCONF-03 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL trigger-time and action-execution-time targets conflict after it is confirmed. | CONFMGT-TMCONF |
| REQ-CONFMGT-TMCONF-04 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to resolve a CCL trigger-time and action-execution-time targets conflict | CONFMGT-TMCONF |
| REQ-CONFMGT-TMCONF-05 | The 3GPP Management System should support a capability enabling an MnS consumer to define and coordinate the hierarchies of the CCL. | CONFMGT-TMCONF |
| REQ-CONFMGT-TMCONF-06 | The 3GPP Management System should support a capability to resolve a CCL trigger-time and action-execution-time targets conflict that has been detected | CONFMGT-TMCONF |
| REQ-CONFMGT-TMCONF-07 | The 3GPP Management System should support a capability to coordinate the resolution of CCL trigger-time and action-execution-time targets conflicts among multiple CCLs | CONFMGT-TMCONF |
| REQ-CONFMGT-INDCONF-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL Indirect targets conflicts. | CONFMGT-INDCONF |
| REQ-CONFMGT-INDCONF-02 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to avoid a CCL Indirect targets conflict | CONFMGT-INDCONF |
| REQ-CONFMGT-INDCONF-03 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL Indirect targets conflict after it is confirmed. | CONFMGT-INDCONF |
| REQ-CONFMGT-INDCONF-04 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to resolve a CCL Indirect targets conflict | CONFMGT-INDCONF |
| REQ-CONFMGT-INDCONF-05 | The 3GPP Management System should support a capability to resolve a CCL Indirect targets conflict that has been detected | CONFMGT-INDCONF |
| REQ-CONFMGT-INDCONF-06 | The 3GPP Management System should support a capability to coordinate the resolution of CCL Indirect targets conflicts among multiple CCLs | CONFMGT-INDCONF |

End Change